3GPP TS 36.213 V9.2.0 (2010-06)

Technical Specification

3rd Generation Partnership Project;

Technical Specification Group Radio Access Network;

Evolved Universal Terrestrial Radio Access (E-UTRA);

Physical layer procedures

(Release 9)

The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP.	 
The present document has not been subject to any approval process by the 3GPP Organisational Partners and shall not be implemented.	 
This Specification is provided for future development work within 3GPP only. The Organisational Partners accept no liability for any use of this Specification.
Specifications and reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organisational Partners’ Publications Offices.

Keywords

UMTS, radio, layer 1

3GPP

Postal address

3GPP support office address

650 Route des Lucioles – Sophia Antipolis

Valbonne – France

Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16

Int

ernet

http://www.3gpp.org

Copyright Notification

No part may be reproduced except as authorized by written permission.
The copyright and the foregoing restriction extend to reproduction in all media.

© 2010, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TTA, TTC).

All rights reserved.

UMTS™ is a Trade Mark of ETSI registered for the benefit of its members

3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners
LTE™ is a Trade Mark of ETSI currently being registered for the benefit of its Members and of the 3GPP Organizational Partners

GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	5

1	Scope	6

2	References	6

3	Definitions, symbols, and abbreviations	7

3.1	Symbols	7

3.2	Abbreviations	7

4	Synchronisation procedures	8

4.1	Cell search	8

4.2	Timing synchronisation	8

4.2.1	Radio link monitoring	8

4.2.2	Inter-cell synchronisation	8

4.2.3	Transmission timing adjustments	8

5	Power control	9

5.1	Uplink power control	9

5.1.1	Physical uplink shared channel	9

5.1.1.1	UE behaviour	9

5.1.1.2	Power headroom	12

5.1.2	Physical uplink control channel	12

5.1.2.1	UE behaviour	12

5.1.3	Sounding Reference Symbol	14

5.1.3.1	UE behaviour	14

5.2	Downlink power allocation	15

5.2.1 	eNodeB Relative Narrowband TX Power restrictions	16

6	Random access procedure	16

6.1	Physical non-synchronized random access procedure	17

6.1.1	Timing	17

6.2	Random Access Response Grant	17

7 	Physical downlink shared channel related procedures	19

7.1	UE procedure for receiving the physical downlink shared channel	19

7.1.1  	Single-antenna port scheme	22

7.1.2	 Transmit diversity scheme	23

7.1.3	 Large delay CDD scheme	23

7.1.4	 Closed-loop spatial multiplexing scheme	23

7.1.5	 Multi-user MIMO scheme	23

7.1.5A	 Dual layer scheme	23

7.1.6	 Resource allocation	23

7.1.6.1	Resource allocation type 0	23

7.1.6.2	Resource allocation type 1	24

7.1.6.3	Resource allocation type 2	25

7.1.7	 Modulation order and transport block size determination	26

7.1.7.1	Modulation order determination	27

7.1.7.2	Transport block size determination	27

7.1.7.2.1	Transport blocks not mapped to two-layer spatial multiplexing	28

7.1.7.2.2	Transport blocks mapped to two-layer spatial multiplexing	34

7.1.7.2.3	Transport blocks mapped for DCI Format 1C	34

7.1.7.3	Redundancy Version determination for Format 1C	35

7.2	UE procedure for reporting channel quality indication (CQI), precoding matrix indicator (PMI) and rank indication (RI)	35

7.2.1	Aperiodic CQI/PMI/RI Reporting using PUSCH	38

7.2.2	Periodic CQI/PMI/RI Reporting using PUCCH	42

7.2.3	Channel quality indicator (CQI) definition	48

7.2.4	Precoding Matrix Indicator (PMI) definition	50

7.3	UE procedure for reporting ACK/NACK	51

8	Physical uplink shared channel related procedures	54

8.1	Resource Allocation for PDCCH DCI Format 0	57

8.2	UE sounding procedure	57

8.3	UE ACK/NACK procedure	60

8.4	UE PUSCH Hopping procedure	60

8.4.1  	Type 1 PUSCH Hopping	61

8.4.2  	Type 2 PUSCH Hopping	62

8.5	UE Reference Symbol procedure	62

8.6	Modulation order, redundancy version and transport block size determination	62

8.6.1	 Modulation order and redundancy version determination	62

8.6.2	 Transport block size determination	64

8.6.3	 Control information MCS offset determination	64

8.7	UE Transmit Antenna Selection	66

9	Physical downlink control channel procedures	66

9.1	UE procedure for determining physical downlink control channel assignment	66

9.1.1  	PDCCH Assignment Procedure	66

9.1.2  	PHICH Assignment Procedure	67

9.2	PDCCH validation for semi-persistent scheduling	68

10	Physical uplink control channel procedures	70

10.1	UE procedure for determining physical uplink control channel assignment	70

10.2	Uplink ACK/NACK timing	75

11 	Physical multicast channel related procedures	76

11.1	UE procedure for receiving the physical multicast channel	76

11.2	UE procedure for receiving MCCH change notification	76

Annex A (informative):	Change history	77

Foreword

This Technical Specification (TS) has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of this present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

1	Scope

The present document specifies and establishes the characteristics of the physicals layer procedures in the FDD and TDD modes of E-UTRA.

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

- References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

- For a specific reference, subsequent revisions do not apply.

- For a non-specific reference, the latest version applies.  In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: “Vocabulary for 3GPP Specifications”

[2]	3GPP TS 36.201: “Evolved Universal Terrestrial Radio Access (E-UTRA); Physical Layer – General Description”

[3]	3GPP TS 36.211: “Evolved Universal Terrestrial Radio Access (E-UTRA); Physical channels and modulation”

[4]	3GPP TS 36.212: “Evolved Universal Terrestrial Radio Access (E-UTRA); Multiplexing and channel coding”

[5]	3GPP TS 36.214: “Evolved Universal Terrestrial Radio Access (E-UTRA); Physical layer – Measurements”

[6]	3GPP TS 36.101: “Evolved Universal Terrestrial Radio Access (E-UTRA); User Equipment (UE) radio transmission and reception”

[7]	3GPP TS 36.104: “Evolved Universal Terrestrial Radio Access (E-UTRA); Base Station (BS) radio transmission and reception”

[8]	3GPP TS36.321, “Evolved Universal Terrestrial Radio Access (E-UTRA); Medium Access Control (MAC) protocol specification”

[9]	3GPP TS36.423, “Evolved Universal Terrestrial Radio Access (E-UTRA); X2 Application Protocol (X2AP)”

[10]	3GPP TS36.133, “Evolved Universal Terrestrial Radio Access (E-UTRA); Requirements for support of radio resource management”

[11]	3GPP TS36.331, “Evolved Universal Terrestrial Radio Access (E-UTRA); Radio Resource Control (RRC) protocol specification”

3	Definitions, symbols, and abbreviations

3.1	Symbols

For the purposes of the present document, the following symbols apply:

System frame number as defined in [3]

Slot number within a radio frame as defined in [3]

Downlink bandwidth configuration, expressed in units of as defined in [3]

Uplink bandwidth configuration, expressed in units of as defined in [3]

Number of SC-FDMA symbols in an uplink slot as defined in [3]

Resource block size in the frequency domain, expressed as a number of subcarriers as defined in [3]

Basic time unit as defined in [3]

3.2	Abbreviations

For the purposes of the present document, the following abbreviations apply.

ACK	Acknowledgement

BCH	Broadcast Channel

CCE	Control Channel Element

CQI	Channel Quality Indicator

CRC	Cyclic Redundancy Check

DAI	Downlink Assignment Index

DCI	Downlink Control Information

DL	Downlink

DL-SCH	Downlink Shared Channel

DTX	Discontinuous Transmission

EPRE	Energy Per Resource Element

MCS	Modulation and Coding Scheme

NACK	Negative Acknowledgement

PBCH	Physical Broadcast Channel

PCFICH	Physical Control Format Indicator Channel

PDCCH	Physical Downlink Control Channel

PDSCH	Physical Downlink Shared Channel

PHICH	Physical Hybrid ARQ Indicator Channel

PMCH	Physical Multicast Channel

PRACH	Physical Random Access Channel

PRB	Physical Resource Block

PUCCH	Physical Uplink Control Channel

PUSCH	Physical Uplink Shared Channel

QoS	Quality of Service

RBG	Resource Block Group

RE	Resource Element

RPF	Repetition Factor

RS	Reference Signal

SIR	Signal-to-Interference Ratio

SINR	Signal to Interference plus Noise Ratio

SPS C-RNTI	Semi-Persistent Scheduling C-RNTI

SR	Scheduling Request

SRS	Sounding Reference Symbol

TA	Time alignment

TTI	Transmission Time Interval

UE	User Equipment

UL	Uplink

UL-SCH	Uplink Shared Channel

VRB	Virtual Resource Block

4	Synchronisation procedures

4.1	Cell search

Cell search is the procedure by which a UE acquires time and frequency synchronization with a cell and detects the physical layer Cell ID of that cell. E-UTRA cell search supports a scalable overall transmission bandwidth corresponding to 6 resource blocks and upwards.

The following signals are transmitted in the downlink to facilitate cell search: the primary and secondary synchronization signals.

4.2	Timing synchronisation

4.2.1	Radio link monitoring

The downlink radio link quality of the serving cell shall be monitored by the UE for the purpose of indicating out-of-sync/in-sync status to higher layers.

In non-DRX mode operation, the physical layer in the UE shall every radio frame assess the radio link quality, evaluated over the previous time period defined in [10], against thresholds (Qout and Qin) defined by relevant tests in [10].

In DRX mode operation, the physical layer in the UE shall at least once every DRX period assess the radio link quality, evaluated over the previous time period defined in [10], against thresholds (Qout and Qin) defined by relevant tests in [10].

The physical layer in the UE shall in radio frames where the radio link quality is assessed indicate out-of-sync to higher layers when the radio link quality is worse than the threshold Qout. When the radio link quality is better than the threshold Qin, the physical layer in the UE shall in radio frames where the radio link quality is assessed indicate in-sync to higher layers.

理解：UE的物理层能够检测无线链路的质量，来指示高层是否处于失同步。就是说除了ENodeB通过给UE TA命令，如果UE没有回复ACK，说明UE失步。同时UE也可以通过检测来发现自己失步。

4.2.2	Inter-cell synchronisation

No functionality is specified in this section in this release.

4.2.3	Transmission timing adjustments

Upon reception of a timing advance command, the UE shall adjust its uplink transmission timing for PUCCH/PUSCH/SRS. The timing advance command indicates the change of the uplink timing relative to the current uplink timing as multiples of 16. The start timing of the random access preamble is specified in [3].

In case of random access response, 11-bit timing advance command [8], TA,  indicates NTA values by index values of  TA = 0, 1, 2, ..., 1282, where an amount of the time alignment is given by NTA = TA 16. NTA is defined in [3].

In other cases, 6-bit timing advance command [8], TA, indicates adjustment of the current NTA value, NTA,old, to the new NTA value, NTA,new, by index values of TA = 0, 1, 2,..., 63, where NTA,new = NTA,old + (TA 31)16. Here, adjustment of NTA value by a positive or a negative amount indicates advancing or delaying the uplink transmission timing by a given amount respectively.

理解：在非RAR的TA中，TA只有6Bit，NTA的变化是+-32*16Ts=+-512Ts，超过此值就调整不回来。

For a timing advance command received on subframe n, the corresponding adjustment of the timing shall apply from the beginning of subframe n+6. When the UE’s uplink PUCCH/PUSCH/SRS transmissions in subframe n and subframe n+1 are overlapped due to the timing adjustment, the UE shall transmit complete subframe n  and not transmit the overlapped part of subframe n+1.

If the received downlink timing changes and is not compensated or is only partly compensated by the uplink timing adjustment without timing advance command as specified in [10], the UE changes NTA accordingly.

5	Power control

Downlink power control determines the energy per resource element (EPRE). The term resource element energy denotes the energy prior to CP insertion.  The term resource element energy also denotes the average energy taken over all constellation points for the modulation scheme applied. Uplink power control determines the average power over a SC-FDMA symbol in which the physical channel is transmitted.（对于UE来说上行功率控制是以一个SC-FDMA符号上的物理信道的平均功率来说，下行功率是以所有的RE的平均功率来说）

理解：功率控制是没有插入CP的，功率是不包括CP的功率的。

5.1	Uplink power control

Uplink power control controls the transmit power of the different uplink physical channels.

A cell wide overload indicator (OI) and a High Interference Indicator (HII) to control UL interference are defined in [9].

5.1.1	Physical uplink shared channel

5.1.1.1	UE behaviour

The setting of the UE Transmit power for the physical uplink shared channel (PUSCH) transmission in subframe i is defined by

[dBm]

where,

- is the configured UE transmitted power defined in [6]

- is the bandwidth of the PUSCH resource assignment expressed in number of resource blocks valid for subframe i.（以单个RB的功率来衡量）

- is a parameter composed of the sum of a cell specific nominal component  provided from higher layers for j=0 and 1 and a UE specific component  provided by higher layers for j=0 and 1. For PUSCH (re)transmissions corresponding to a semi-persistent grant then j=0 , for PUSCH (re)transmissions corresponding to a dynamic scheduled grant then j=1 and for PUSCH (re)transmissions corresponding to the random access response grant then j=2.  and , where the parameter PREAMBLE_INITIAL_RECEIVED_TARGET_POWER [8] () and  are signalled from higher layers.

理解：相当于UplinkPowerControl 信息中的deltaPreambleMsg3，相当于MSG3的小区特定功率值比其他的要大。

- 半静态调度为0，动态调度为1，RAR中的授权为2

- 相当于SPS-Config消息中的p0-NominalPUSCH-Persistent，相当于p0-UE-PUSCH-Persistent

- 相当于UplinkPowerControl消息中的p0-NominalPUSCH，相当于p0-UE-PUSCH

- 相当于RACH-ConfigCommon中的preambleInitialReceivedTargetPower

- For j =0 or 1,  is a 3-bit cell specific parameter provided by higher layers. For j=2, （对应随机接入相应授权消息调度的PUSCH传输）

理解：相当于UplinkPowerControl消息中的alpha

- PL is the downlink pathloss estimate calculated in the UE in dB and PL = referenceSignalPower – higher layer filtered RSRP, where referenceSignalPower is provided by higher layers and RSRP is defined in [5] and the higher layer filter configuration is defined in [11]

理解：相当于FilterCoefficient消息

- for and 0 for where  is given by the UE specific parameter deltaMCS-Enabled provided by higher layers

理解：表示不同MCS对应不同的功率，MPR越大，需要发射的功率越大。

  - for control data sent via PUSCH without UL-SCH data and for other cases.

    - where  is the number of code blocks,  is the size for code block ,  is the number of CQI bits including CRC bits  and  is the number of resource elements determined as, where , ,  and  are defined in [4].

  - for control data sent via PUSCH without UL-SCH data and  for other cases.

理解：MPR的值和控制数据是否在没有UL-SCH上的PUSCH上传输相关，取值分为2种情况。

同样的值也和控制数据是否在没有UL-SCH上的PUSCH上传输相关

- is a UE specific correction value, also referred to as a TPC command and is included in PDCCH with DCI format 0 or jointly coded with other TPC commands in PDCCH with DCI format 3/3A whose CRC parity bits are scrambled with TPC-PUSCH-RNTI. The current PUSCH power control adjustment state is given by which is defined by:

理解：为UE的专属的修正值，叫做TPC命令。由DCI0、DCI3和DCI3A中的功率控制命令字来确定。由表5.1.1.1-2和表5.1.1.1-3的功率控制字来确定的值。

  - if accumulation is enabled based on the UE-specific parameter Accumulation-enabled provided by higher layers or if the TPC command  is included in a PDCCH with DCI format 0 where the CRC is scrambled by the Temporary C-RNTI

理解：Accumulation-enabled决定了UE的调整值的公式，如果为TRUE，为递归累加性，如果为FALSE，为直接计算的值。

    - where was signalled on PDCCH with DCI format 0 or 3/3A on subframe , and where  is the first value after reset of accumulation.

    - The value of  is

      - For FDD,= 4

      - For TDD UL/DL configurations 1-6,  is given in Table 5.1.1.1-1

      - For TDD UL/DL configuration 0

        - If the PUSCH transmission in subframe 2 or 7 is scheduled with a PDCCH of DCI format 0 in which the LSB of the UL index is set to 1, = 7

        - For all other PUSCH transmissions,  is given in Table 5.1.1.1-1.

    - The UE attempts to decode a PDCCH of DCI format 0 with the UE’s C-RNTI or SPS C-RNTI and a PDCCH of DCI format 3/3A with this UE’s TPC-PUSCH-RNTI in every subframe except when in DRX

    - If DCI format 0 and DCI format 3/3A are both detected in the same subframe, then the UE shall use the  provided in DCI format 0.

理解：DCI0的优先级高于DCI3/3A

    - dB for a subframe where no TPC command is decoded or where DRX occurs or i is not an uplink subframe in TDD.

    - The dB accumulated values signalled on PDCCH with DCI format 0 are given in Table 5.1.1.1-2. If the PDCCH with DCI format 0 is validated as a SPS activation or release PDCCH, then  is 0dB.

    - The  dB accumulated values signalled on PDCCH with DCI format 3/3A are one of SET1 given in Table 5.1.1.1-2 or SET2 given in Table 5.1.1.1-3 as determined by the parameter TPC-Index provided by higher layers.

    - If UE has reached maximum power, positive TPC commands shall not be accumulated

    - If UE has reached minimum power, negative TPC commands shall not be accumulated

    - UE shall reset accumulation

      - when  value is changed by higher layers

      - when the UE receives random access response message

  - if accumulation is not enabled based on the UE-specific parameter Accumulation-enabled provided by higher layers

    - where was signalled on PDCCH with DCI format 0 on subframe

    - The value of  is

      - For FDD,= 4

      - For TDD UL/DL configurations 1-6,  is given in Table 5.1.1.1-1

      - For TDD UL/DL configuration 0

        - If the PUSCH transmission in subframe 2 or 7 is scheduled with a PDCCHof DCI format 0 in which the LSB of the UL index is set to 1, = 7

        - For all other PUSCH transmissions,  is given in Table 5.1.1.1-1.

    - The  dB absolute values signalled on PDCCH with DCI format 0 are given in Table 5.1.1.1-2. If the PDCCH with DCI format 0 is validated as a SPS activation or release PDCCH, then  is 0dB.

    - for a subframe where no PDCCH with DCI format 0 is decoded or where DRX occurs or i is not an uplink subframe in TDD.

  - For both types of  (accumulation or current absolute) the first value is set as follows:

    - If  value is changed by higher layers,

    - Else

        - where  is the TPC command indicated in the random access response, see Section 6.2, and

        - is provided by higher layers and corresponds to the total power ramp-up from the first to the last preamble

Table 5.1.1.1-1  for TDD configuration 0-6

| TDD UL/DL Configuration | subframe number i |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 |
| 0 | - | - | 6 | 7 | 4 | - | - | 6 | 7 | 4 |
| 1 | - | - | 6 | 4 | - | - | - | 6 | 4 | - |
| 2 | - | - | 4 | - | - | - | - | 4 | - | - |
| 3 | - | - | 4 | 4 | 4 | - | - | - | - | - |
| 4 | - | - | 4 | 4 | - | - | - | - | - | - |
| 5 | - | - | 4 | - | - | - | - | - | - | - |
| 6 | - | - | 7 | 7 | 5 | - | - | 7 | 7 | - |

Table 5.1.1.1-2: Mapping of TPC Command Field in DCI format 0/3 to absolute and accumulated  values.

| TPC Command Field in DCI format 0/3 | Accumulated [dB] | Absolute [dB] only DCI format 0 |
| --- | --- | --- |
| 0 | -1 | -4 |
| 1 | 0 | -1 |
| 2 | 1 | 1 |
| 3 | 3 | 4 |

Table 5.1.1.1-3: Mapping of TPC Command Field in DCI format 3A to accumulated  values.

| TPC Command Field in DCI format 3A | Accumulated [dB] |
| --- | --- |
| 0 | -1 |
| 1 | 1 |

5.1.1.2	Power headroom

The UE power headroom valid for subframe i is defined by

[dB]

where, , , , , PL,  and  are defined in section 5.1.1.1.

The power headroom shall be rounded to the closest value in the range [40; -23] dB with steps of 1 dB and is delivered by the physical layer to higher layers.

5.1.2	Physical uplink control channel

5.1.2.1	UE behaviour

The setting of the UE Transmit powerfor the physical uplink control channel (PUCCH) transmission in subframe i is defined by

[dBm]

where

- is the configured UE transmitted power defined in [6]

- The parameter  is provided by higher layers. Each  value corresponds to a PUCCH format (F) relative to PUCCH format 1a, where each PUCCH format (F ) is defined in Table 5.4-1 [3].

理解：相当于RRC信令UplinkPowerControl中的DeltaFList-PUCCH元素，是相当于1a的功率值。

- is a PUCCH format dependent value, where  corresponds to the number of information bits for the channel quality information defined in section 5.2.3.3 in [4] and  is the number of HARQ bits.

  - For PUCCH format 1,1a and 1b

  - For PUCCH format 2, 2a, 2b and normal cyclic prefix

  - For PUCCH format 2 and extended cyclic prefix

理解：h(n)是与PUCCH格式、CQI的Bit数和HARQ的Bit数相关的调整量。Format1系列的此值为0.

- is a parameter composed of the sum of a cell specific parameter  provided by higher layers and a UE specific component  provided by higher layers.

- is a UE specific correction value, also referred to as a TPC command, included in a PDCCH with DCI format 1A/1B/1D/1/2A/2/2B or sent jointly coded with other UE specific PUCCH correction values on a PDCCH with DCI format 3/3A whose CRC parity bits are scrambled with TPC-PUCCH-RNTI.

理解：为UE专属的修正值，称为TPC命令，在DCI1A/1B/1D/1/2A/2/2B和DCI3/3A中。

  - The UE attempts to decode a PDCCH of DCI format 3/3A with the UE’s TPC-PUCCH-RNTI and one or several PDCCHs of DCI format 1A/1B/1D/1/2A/2/2B with the UE’s C-RNTI or SPS C-RNTI on every subframe except when in DRX.

  - If the UE decodes a PDCCH with DCI format 1A/1B/1D/1/2A/2/2B and the corresponding detected RNTI equals the C-RNTI or SPS C-RNTI of the UE, the UE shall use the  provided in that PDCCH.

else

    - if the UE decodes a PDCCH with DCI format 3/3A, the UE shall use the  provided in that PDCCH

else the UE shall set  = 0 dB.

  - where  is the current PUCCH power control adjustment state and where is the first value after reset.

    - For FDD,  and .

    - For TDD, values of  and  are given in Table 10.1-1.

    - The  dB values signalled on PDCCH with DCI format 1A/1B/1D/1/2A/2/2B are given in Table 5.1.2.1-1. If the PDCCH with DCI format 1/1A/2/2A/2B is validated as an SPS activation PDCCH, or the PDCCH with DCI format 1A is validated as an SPS release PDCCH, then  is 0dB.

    - The  dB values signalled on PDCCH with DCI format 3/3A are given in Table 5.1.2.1-1 or in Table 5.1.2.1-2 as semi-statically configured by higher layers.

    - If  value is changed by higher layers,

    - Else

        - where  is the TPC command indicated in the random access response, see Section 6.2 and

        - is the total power ramp-up from the first to the last preamble provided by higher layers

    - If UE has reached maximum power, positive TPC commands shall not be accumulated

    - If UE has reached minimum power, negative TPC commands shall not be accumulated

    - UE shall reset accumulation

      - when  value is changed by higher layers

      - when the UE receives a random access response message

    - if  is not an uplink subframe in TDD.

Table 5.1.2.1-1: Mapping of TPC Command Field in DCI format 1A/1B/1D/1/2A/2B/2/3 to  values.

| TPC Command Field in DCI format 1A/1B/1D/1/2A/2B/2/3 | [dB] |
| --- | --- |
| 0 | -1 |
| 1 | 0 |
| 2 | 1 |
| 3 | 3 |

Table 5.1.2.1-2: Mapping of TPC Command Field in DCI format 3A to  values.

| TPC Command Field in DCI format 3A | [dB] |
| --- | --- |
| 0 | -1 |
| 1 | 1 |

5.1.3	Sounding Reference Symbol

5.1.3.1	UE behaviour

The setting of the UE Transmit power for the Sounding Reference Symbol transmitted on subframe i is defined by

[dBm]

where

- is the configured UE transmitted power defined in [6]

- For , is a 4-bit UE specific parameter semi-statically configured by higher layers with 1dB step size in the range [-3, 12] dB.

- For , is a 4-bit UE specific parameter semi-statically configured by higher layers with 1.5 dB step size in the range [-10.5,12] dB

理解：的值由RRC信令UplinkPowerControl中的pSRS-Offset来确定，当时，实际参数的值是pSRS-Offset – 3；当时，实际参数的值是-10.5+1.5*pSRS-Offset；

- is the bandwidth of the SRS transmission in subframe i expressed in number of resource blocks.

- is the current power control adjustment state for the PUSCH, see Section 5.1.1.1.

- and  are parameters as defined in Section 5.1.1.1, where .

理解：其他参数的值和PUSCH上的参数基本相同。

5.2	Downlink power allocation

The eNodeB determines the downlink transmit energy per resource element.

A UE may assume downlink cell-specific RS EPRE is constant across the downlink system bandwidth and constant across all subframes until different cell-specific RS power information is received. The downlink reference-signal EPRE can be derived from the downlink reference-signal transmit power given by the parameter Reference-signal-power provided by higher layers. The downlink reference-signal transmit power is defined as the linear average over the power contributions (in [W]) of all resource elements that carry cell-specific reference signals within the operating system bandwidth.

理解：Reference-signal-power相当于RRC信令PDSCH-Config中的referenceSignalPower，以dbm为单位

The ratio of PDSCH EPRE to cell-specific RS EPRE among PDSCH REs (not applicable to PDSCH REs with zero EPRE) for each OFDM symbol is denoted by either  or according to the OFDM symbol index as given by Table 5.2-2. In addition, and are UE-specific.

For a UE in transmission mode 8 when UE-specific RSs are not present in the PRBs upon which the corresponding PDSCH is mapped or in transmission modes 1 – 7, the UE may assume that for 16 QAM, 64 QAM, spatial multiplexing with more than one layer or for PDSCH transmissions associated with the multi-user MIMO transmission scheme,

- is equal to [dB] when the UE receives a PDSCH data transmission using precoding for transmit diversity with 4 cell-specific antenna ports according to Section 6.3.4.3 of [3];

- is equal to [dB] otherwise

理解：PA相当于RRC信令PDSCH-Config中的p-a，PB相当于RRC信令PDSCH-Config中的p-b

whereis 0 dB for all PDSCH transmission schemes except multi-user MIMO and where is a UE specific parameter provided by higher layers.

For transmission mode 7, if UE-specific RSs are present in the PRBs upon which the corresponding PDSCH is mapped, the ratio of PDSCH EPRE to UE-specific RS EPRE within each OFDM symbol containing UE-specific RSs shall be a constant, and that constant shall be maintained over all the OFDM symbols containing the UE-specific RSs in the corresponding PRBs. In addition, the UE may assume that for 16QAM or 64QAM, this ratio is 0 dB.

For transmission mode 8, if UE-specific RSs are present in the PRBs upon which the corresponding PDSCH is mapped, the UE may assume the ratio of PDSCH EPRE to UE-specific RS EPRE within each OFDM symbol containing UE-specific RSs is 0 dB.

A UE may assume that downlink positioning reference signal EPRE is constant across the positioning reference signal bandwidth and across all OFDM symbols that contain positioning reference signals in a given positioning reference signal occasion [10].

The cell-specific ratio  is given by Table 5.2-1 according to cell-specific parameter  signalled by higher layers and the number of configured eNodeB cell specific antenna ports.

Table 5.2-1: The cell-specific ratio  for 1, 2, or 4 cell specific antenna ports

|  | One Antenna Port | Two and Four Antenna Ports |
| --- | --- | --- |
| 0 | 1 | 5/4 |
| 1 | 4/5 | 1 |
| 2 | 3/5 | 3/4 |
| 3 | 2/5 | 1/2 |

For PMCH with 16QAM or 64QAM, the UE may assume that the ratio of PMCH EPRE to MBSFN RS EPRE is equal to 0 dB.

Table 5.2-2: OFDM symbol indices within a slot where the ratio of the corresponding PDSCH EPRE to the cell-specific RS EPRE is denoted by  or

| Number of antenna ports | OFDM symbol indices within a slot where the ratio of the corresponding PDSCH EPRE to the cell-specific RS EPRE is denoted by |  | OFDM symbol indices within a slot where the ratio of the corresponding PDSCH EPRE to the cell-specific RS EPRE is denoted by |  |
| --- | --- | --- | --- | --- |
|  | Normal cyclic prefix | Extended cyclic prefix | Normal cyclic prefix | Extended cyclic prefix |
| One or two | 1, 2, 3, 5, 6 | 1, 2, 4, 5 | 0, 4 | 0, 3 |
| Four | 2, 3, 5, 6 | 2, 4, 5 | 0, 1, 4 | 0, 1, 3 |

5.2.1 	eNodeB Relative Narrowband TX Power restrictions

The determination of reported Relative Narrowband TX Power indication  is defined as follows:

where  is the maximum intended EPRE of UE-specific PDSCH REs in OFDM symbols not containing RS in this physical resource block on antenna port p in the considered future time interval; is the physical resource block number ;  takes on one of the following values[dB] and

理解：表示在未来的一段时间内不包含参考符号的OFDM符号中的数据子载波的发射功率

以比特图的形式指示每个PRB 将要使用的发射功率是否超过该门限

where  is the base station maximum output power described in [7], and , and are defined in [3].

理解：相对窄带发射功率限制（RNTP）是为了支持下行小区间的干扰协调而定义的物理层测量，在X2上进行交互。它表示了该基站在未来一段时间内下行各个PRB将使用的最大发射功率的情况，相邻小区利用该消息来协调用户，实现同频小区干扰协调。

6	Random access procedure

Prior to initiation of the non-synchronized physical random access procedure, Layer 1 shall receive the following information from the higher layers:

- Random access channel parameters (PRACH configuration and frequency position)

- Parameters for determining the root sequences and their cyclic shifts in the preamble sequence set for the cell (index to logical root sequence table, cyclic shift (), and set type (unrestricted or restricted set))

6.1	Physical non-synchronized random access procedure

From the physical layer perspective, the L1 random access procedure encompasses the transmission of random access preamble and random access response.  The remaining messages are scheduled for transmission by the higher layer on the shared data channel and are not considered part of the L1 random access procedure.  A random access channel occupies 6 resource blocks in a subframe or set of consecutive subframes reserved for random access preamble transmissions. The eNodeB is not prohibited from scheduling data in the resource blocks reserved for random access channel preamble transmission.（eNodeB没有禁止在分配给随机接入的资源上调度数据，就是说PRACH和PUSCH是可以重叠的，看算法是否能够区分。）

The following steps are required for the L1 random access procedure:

- Layer 1 procedure is triggered upon request of a preamble transmission by higher layers.

- A preamble index, a target preamble received power (PREAMBLE_RECEIVED_TARGET_POWER), a corresponding RA-RNTI and a PRACH resource are indicated by higher layers as part of the request.

- A preamble transmission power PPRACH is determined as 
PPRACH = min{,  PREAMBLE_RECEIVED_TARGET_POWER + PL}_[dBm], where  is the configured UE transmitted power defined in [6]  and PL is the downlink pathloss estimate calculated in the UE.

- A preamble sequence is selected from the preamble sequence set using the preamble index.

- A single preamble is transmitted using the selected preamble sequence with transmission power PPRACH on the indicated PRACH resource.

- Detection of a PDCCH with the indicated RA-RNTI is attempted during a window controlled by higher layers (see [8], clause 5.1.4). If detected, the corresponding DL-SCH transport block is passed to higher layers. The higher layers parse the transport block and indicate the 20-bit uplink grant to the physical layer, which is processed according to section 6.2.

6.1.1	Timing

For the L1 random access procedure, UE’s uplink transmission timing after a random access preamble transmission is as follows.

  - If a PDCCH with associated RA-RNTI is detected in subframe n, and the corresponding DL-SCH transport block contains a response to the transmitted preamble sequence, the UE shall, according to the information in the response, transmit an UL-SCH transport block in the first subframe , , if the UL delay field in section 6.2 is set to zero where  is the first available UL subframe for PUSCH transmission. The UE shall postpone the PUSCH transmission to the next available UL subframe after  if the field is set to 1.

  - If a random access response is received in subframe n, and the corresponding DL-SCH transport block does not contain a response to the transmitted preamble sequence, the UE shall, if requested by higher layers, be ready to transmit a new preamble sequence no later than in subframe .

  - If no random access response is received in subframe n, where subframe n is the last subframe of the random access response window, the UE shall, if requested by higher layers, be ready to transmit a new preamble sequence no later than in subframe .

In case a random access procedure is initiated by a PDCCH order in subframe n, the UE shall, if requested by higher layers, transmit random access preamble in the first subframe , , where a PRACH resource is available.

6.2	Random Access Response Grant

The higher layers indicate the 20-bit UL Grant to the physical layer, as defined in [8]. This is referred to the Random Access Response Grant in the physical layer. The content of these 20 bits starting with the MSB and ending with the LSB are as follows:

- Hopping flag – 1 bit

- Fixed size resource block assignment – 10 bits

- Truncated modulation and coding scheme – 4 bits

- TPC command for scheduled PUSCH – 3 bits

- UL delay – 1 bit

- CQI request – 1 bit

The UE shall perform PUSCH frequency hopping if the single bit frequency hopping (FH) field in a corresponding Random Access Response Grant is set as 1, otherwise no PUSCH frequency hopping is performed.  When the hopping flag is set, the UE shall perform PUSCH hopping as indicated via the fixed size resource block assignment detailed below,

The fixed size resource block assignment field is interpreted as follows:

if

Truncate the fixed size resource block assignment to its b least significant bits, where , and interpret the truncated resource block assignment according to the rules for a regular DCI format 0

理解：当带宽小于或等于44时，取10Bit中的低b比特当做DCI0中的RIV。

else

Insert b most significant bits with value set to ‘0’ after the NUL_hop hopping bits in the fixed size resource block assignment, where the number of hopping bits NUL_hop is zero when the hopping flag bit is not set to 1, and is defined in Table 8.4-1 when the hopping flag bit is set to 1, and , and interpret the expanded resource block assignment according to the rules for a regular DCI format 0

理解：当跳频比特位为1时， NUL_hop比特个数见表8.4-1。大于49个RB比特数为2，其它比特数为1。在10Bit中NUL_hop个Bit后（从高位开始数），加入b个0，组成新的数据当做DCI0的RIV。

end if

The truncated modulation and coding scheme field is interpreted such that the modulation and coding scheme corresponding to the Random Access Response grant is determined from MCS indices 0 through 15 in Table 8.6.1-1.

The TPC command  shall be used for setting the power of the PUSCH, and is interpreted according to Table 6.2-1.

Table 6.2-1: TPC Command  for Scheduled PUSCH

| TPC Command | Value (in dB) |
| --- | --- |
| 0 | -6 |
| 1 | -4 |
| 2 | -2 |
| 3 | 0 |
| 4 | 2 |
| 5 | 4 |
| 6 | 6 |
| 7 | 8 |

In non-contention based random access procedure, the CQI request field is interpreted to determine whether an aperiodic CQI, PMI, and RI report is included in the corresponding PUSCH transmission according to section 7.2.1. In contention based random access procedure, the CQI request field is reserved.

理解：在竞争随机接入过程中，CQI request没有意义。在非竞争随机接入过程中，此比特代表非周期性的CSI信息在PUSCH中传输。

The UL delay applies for both TDD and FDD and this field can be set to 0 or 1 to indicate whether the delay of PUSCH is introduced as shown in section 6.1.1.

7 	Physical downlink shared channel related procedures

For FDD, there shall be a maximum of 8 HARQ processes in the downlink.

For TDD, the maximum number of HARQ processes in the downlink shall be determined by the UL/DL configuration (Table 4.2-2 of [3]), as indicated in table 7-1.

The dedicated broadcast HARQ process defined in [8] is not counted as part of the maximum number of HARQ processes for both FDD and TDD.

Table 7-1: Maximum number of DL HARQ processes for TDD

| TDD UL/DL configuration | Maximum number of HARQ processes |
| --- | --- |
| 0 | 4 |
| 1 | 7 |
| 2 | 10 |
| 3 | 9 |
| 4 | 12 |
| 5 | 15 |
| 6 | 6 |

7.1	UE procedure for receiving the physical downlink shared channel

A UE shall upon detection of a PDCCH with DCI format 1, 1A, 1B, 1C, 1D, 2, 2A or 2B intended for the UE in a subframe, decode the corresponding PDSCH in the same subframe with the restriction of the number of transport blocks defined in the higher layers.

A UE may assume that positioning reference signals are not present in resource blocks in which it shall decode PDSCH according to a detected PDCCH with CRC scrambled by the SI-RNTI or P-RNTI with DCI format 1A or 1C intended for the UE.

If a UE is configured by higher layers to decode PDCCH with CRC scrambled by the SI-RNTI, the UE shall decode the PDCCH and the corresponding PDSCH according to any of the combinations defined in table 7.1-1. The scrambling initialization of PDSCH corresponding to these PDCCHs is by SI-RNTI.

# Table 7.1-1: PDCCH and PDSCH configured by SI-RNTI

| DCI format | Search Space | Transmission scheme of PDSCH corresponding to PDCCH |
| --- | --- | --- |
| DCI format 1C | Common | If the number of PBCH antenna ports is one, Single-antenna port, port 0 is used, otherwise Transmit diversity. |
| DCI format 1A | Common | If the number of PBCH antenna ports is one, Single-antenna port, port 0 is used, otherwise Transmit diversity |

If a UE is configured by higher layers to decode PDCCH with CRC scrambled by the P-RNTI, the UE shall decode the PDCCH and the corresponding PDSCH according to any of the combinations defined in table 7.1-2. The scrambling initialization of PDSCH corresponding to these PDCCHs is by P-RNTI.

# Table 7.1-2: PDCCH and PDSCH configured by P-RNTI

| DCI format | Search Space | Transmission scheme of PDSCH corresponding to PDCCH |
| --- | --- | --- |
| DCI format 1C | Common | If the number of PBCH antenna ports is one, Single-antenna port, port 0 is used (see subclause 7.1.1), otherwise Transmit diversity (see subclause 7.1.2) |
| DCI format 1A | Common | If the number of PBCH antenna ports is one, Single-antenna port, port 0 is used (see subclause 7.1.1), otherwise Transmit diversity (see subclause 7.1.2) |

If a UE is configured by higher layers to decode PDCCH with CRC scrambled by the RA-RNTI, the UE shall decode the PDCCH and the corresponding PDSCH according to any of the combinations defined in table 7.1-3. The scrambling initialization of PDSCH corresponding to these PDCCHs is by RA-RNTI.

When RA-RNTI and either C-RNTI or SPS C-RNTI are assigned in the same subframe, UE is not required to decode a PDSCH indicated by a PDCCH with a CRC scrambled by C-RNTI or SPS C-RNTI.（在同一帧中如果出现RA-RNTI和C-RNTI 或者 SPS C-RNTI，UE只解RA-RNTI对应的PDSCH）

# Table 7.1-3: PDCCH and PDSCH configured by RA-RNTI

| DCI format | Search Space | Transmission scheme of PDSCH corresponding to PDCCH |
| --- | --- | --- |
| DCI format 1C | Common | If the number of PBCH antenna ports is one, Single-antenna port, port 0 is used (see subclause 7.1.1), otherwise Transmit diversity (see subclause 7.1.2) |
| DCI format 1A | Common | If the number of PBCH antenna ports is one, Single-antenna port, port 0 is used (see subclause 7.1.1), otherwise Transmit diversity (see subclause 7.1.2) |

The UE is semi-statically configured via higher layer signalling to receive PDSCH data transmissions signalled via PDCCH according to one of eight transmission modes, denoted mode 1 to mode 8.

For frame structure type 1,

- the UE is not expected to receive PDSCH resource blocks transmitted on antenna port 5 in any subframe in which the number of OFDM symbols for PDCCH with normal CP is equal to four;

- the UE is not expected to receive PDSCH resource blocks transmitted on antenna port 5, 7, or 8 in the two PRBs to which a pair of VRBs is mapped if either one of the two PRBs overlaps in frequency with a transmission of either PBCH or primary or secondary synchronisation signals in the same subframe;

  - the UE is not expected to receive PDSCH resource blocks transmitted on antenna port 7 for which distributed VRB resource allocation is assigned.

  - The UE may skip decoding the transport block(s) if it does not receive all assigned PDSCH resource blocks.  If the UE skips decoding, the physical layer indicates to higher layer that the transport block(s) are not successfully decoded.

For frame structure type 2,

  - the UE is not expected to receive PDSCH resource blocks transmitted on antenna port 5 in any subframe in which the number of OFDM symbols for PDCCH with normal CP is equal to four;

  - the UE is not expected to receive PDSCH resource blocks transmitted on antenna port 5 in the two PRBs to which a pair of VRBs is mapped if either one of the two PRBs overlaps in frequency with a transmission of PBCH in the same subframe;

  - the UE is not expected to receive PDSCH resource blocks transmitted on antenna port 7 or 8 in the two PRBs to which a pair of VRBs is mapped if either one of the two PRBs overlaps in frequency with a transmission of primary or secondary synchronisation signals in the same subframe;

  - with normal CP configuration, the UE is not expected to receive PDSCH on antenna port 5 for which distributed VRB resource allocation is assigned in the special subframe with configuration #1 or #6;

  - the UE is not expected to receive PDSCH on antenna port 7  for which distributed VRB resource allocation is assigned.

  - The UE may skip decoding the transport block(s) if it does not receive all assigned PDSCH resource blocks.  If the UE skips decoding, the physical layer indicates to higher layer that the transport block(s) are not successfully decoded.

If a UE is configured by higher layers to decode PDCCH with CRC scrambled by the C-RNTI, the UE shall decode the PDCCH and any corresponding PDSCH according to the respective combinations defined in table 7.1-5. The scrambling initialization of PDSCH corresponding to these PDCCHs is by C-RNTI.

When a UE configured in transmission mode 3, 4 or 8 receives a DCI Format 1A assignment, it shall assume that the PDSCH transmission is associated with transport block 1 and that transport block 2 is disabled.

When a UE is configured in transmission mode 7, scrambling initialization of UE-specific reference signals corresponding to these PDCCHs is by C-RNTI.

Table 7.1-5: PDCCH and PDSCH configured by C-RNTI

| Transmission mode | DCI format | Search Space | Transmission scheme of PDSCH corresponding to PDCCH |
| --- | --- | --- | --- |
| Mode 1 | DCI format 1A | Common and UE specific by C-RNTI | Single-antenna port, port 0 (see subclause 7.1.1) |
|  | DCI format 1 | UE specific by C-RNTI | Single-antenna port, port 0 (see subclause 7.1.1) |
| Mode 2 | DCI format 1A | Common and UE specific by C-RNTI | Transmit diversity (see subclause 7.1.2) |
|  | DCI format 1 | UE specific by C-RNTI | Transmit diversity (see subclause 7.1.2) |
| Mode 3 | DCI format 1A | Common and UE specific by C-RNTI | Transmit diversity (see subclause 7.1.2) |
|  | DCI format 2A | UE specific by C-RNTI | Large delay CDD (see subclause 7.1.3) or Transmit diversity (see subclause 7.1.2) |
| Mode 4 | DCI format 1A | Common and UE specific by C-RNTI | Transmit diversity (see subclause 7.1.2) |
|  | DCI format 2 | UE specific by C-RNTI | Closed-loop spatial multiplexing (see subclause 7.1.4)or Transmit diversity (see subclause 7.1.2) |
| Mode 5 | DCI format 1A | Common and UE specific by C-RNTI | Transmit diversity (see subclause 7.1.2) |
|  | DCI format 1D | UE specific by C-RNTI | Multi-user MIMO (see subclause 7.1.5) |
| Mode 6 | DCI format 1A | Common and UE specific by C-RNTI | Transmit diversity (see subclause 7.1.2) |
|  | DCI format 1B | UE specific by C-RNTI | Closed-loop spatial multiplexing (see subclause 7.1.4) using a single transmission layer |
| Mode 7 | DCI format 1A | Common and UE specific by C-RNTI | If the number of PBCH antenna ports is one, Single-antenna port, port 0 is used (see subclause 7.1.1), otherwise Transmit diversity (see subclause 7.1.2) |
|  | DCI format 1 | UE specific by C-RNTI | Single-antenna port; port 5 (see subclause 7.1.1) |
| Mode 8 | DCI format 1A | Common and UE specific by C-RNTI | If the number of PBCH antenna ports is one, Single-antenna port, port 0 is used (see subclause 7.1.1), otherwise Transmit diversity (see subclause 7.1.2) |
|  | DCI format 2B | UE specific by C-RNTI | Dual layer transmission; port 7 and 8 (see subclause 7.1.5A) or single-antenna port; port 7 or 8 (see subclause 7.1.1) |

If a UE is configured by higher layers to decode PDCCH with CRC scrambled by the SPS C-RNTI, the UE shall decode the PDCCH and any corresponding PDSCH according to the respective combinations defined in table 7.1-6. The same PDSCH related configuration applies in the case that a PDSCH is transmitted without a corresponding PDCCH. The scrambling initialization of PDSCH corresponding to these PDCCHs and PDSCH without a corresponding PDCCH is by SPS C-RNTI.

When a UE is configured in transmission mode 7, scrambling initialization of UE-specific reference signals corresponding to these PDCCHs is by SPS C-RNTI.

Table 7.1-6: PDCCH and PDSCH configured by SPS C-RNTI

| Transmission mode | DCI format | Search Space | Transmission scheme of PDSCH corresponding to PDCCH |
| --- | --- | --- | --- |
| Mode 1 | DCI format 1A | Common and UE specific by C-RNTI | Single-antenna port, port 0 (see subclause 7.1.1) |
|  | DCI format 1 | UE specific by C-RNTI | Single-antenna port, port 0 (see subclause 7.1.1) |
| Mode 2 | DCI format 1A | Common and UE specific by C-RNTI | Transmit diversity (see subclause 7.1.2) |
|  | DCI format 1 | UE specific by C-RNTI | Transmit diversity (see subclause 7.1.2) |
| Mode 3 | DCI format 1A | Common and UE specific by C-RNTI | Transmit diversity (see subclause 7.1.2) |
|  | DCI format 2A | UE specific by C-RNTI | Transmit diversity (see subclause 7.1.2) |
| Mode 4 | DCI format 1A | Common and UE specific by C-RNTI | Transmit diversity (see subclause 7.1.2) |
|  | DCI format 2 | UE specific by C-RNTI | Transmit diversity (see subclause 7.1.2) |
| Mode 5 | DCI format 1A | Common and UE specific by C-RNTI | Transmit diversity (see subclause 7.1.2) |
| Mode 6 | DCI format 1A | Common and UE specific by C-RNTI | Transmit diversity (see subclause 7.1.2) |
| Mode 7 | DCI format 1A | Common and UE specific by C-RNTI | Single-antenna port; port 5 (see subclause 7.1.1) |
|  | DCI format 1 | UE specific by C-RNTI | Single-antenna port; port 5 (see subclause 7.1.1) |
| Mode 8 | DCI format 1A | Common and UE specific by C-RNTI | Single-antenna port; port 7(see subclause 7.1.1) |
|  | DCI format 2B | UE specific by C-RNTI | Single-antenna port; port 7 or 8 (see subclause 7.1.1) |

If a UE is configured by higher layers to decode PDCCH with CRC scrambled by the Temporary C-RNTI and is not configured to decode PDCCH with CRC scrambled by the C-RNTI, the UE shall decode the PDCCH and the corresponding PDSCH according to the combination defined in table 7.1-7. The scrambling initialization of PDSCH corresponding to these PDCCHs is by Temporary C-RNTI.

Table 7.1-7: PDCCH and PDSCH configured by Temporary C-RNTI

| DCI format | Search Space | Transmission scheme of PDSCH corresponding to PDCCH |
| --- | --- | --- |
| DCI format 1A | Common and UE specific by Temporary C-RNTI | If the number of PBCH antenna port is one, Single-antenna port, port 0 is used (see subclause 7.1.1), otherwise Transmit diversity (see subclause 7.1.2) |
| DCI format 1 | UE specific by Temporary C-RNTI | If the number of PBCH antenna port is one, Single-antenna port, port 0 is used (see subclause 7.1.1), otherwise Transmit diversity (see subclause 7.1.2) |

The transmission schemes of the PDSCH are described in the following sub-clauses.

7.1.1  	Single-antenna port scheme

For the single-antenna port transmission schemes (port 0, port 5, port 7 or port 8) of the PDSCH, the UE may assume that an eNB transmission on the PDSCH would be performed according to Section 6.3.4.1 of [3].

In case an antenna port  is used, the UE cannot assume that the other antenna port in the set is not associated with transmission of PDSCH to another UE.

7.1.2		Transmit diversity scheme

For the transmit diversity transmission scheme of the PDSCH, the UE may assume that an eNB transmission on the PDSCH would be performed according to Section 6.3.4.3 of [3]

7.1.3		Large delay CDD scheme

For the large delay CDD transmission scheme of the PDSCH, the UE may assume that an eNB transmission on the PDSCH would be performed according to large delay CDD as defined in Section 6.3.4.2.2 of [3].

7.1.4		Closed-loop spatial multiplexing scheme

For the closed-loop spatial multiplexing transmission scheme of the PDSCH, the UE may assume that an eNB transmission on the PDSCH would be performed according to the applicable number of transmission layers as defined in Section 6.3.4.2.1 of [3].

7.1.5		Multi-user MIMO scheme

For the multi-user MIMO transmission scheme of the PDSCH, the UE may assume that an eNB transmission on the PDSCH would be performed on one layer and according to Section 6.3.4.2.1 of [3]. The dB value signalled on PDCCH with DCI format 1D using the downlink power offset field is given in Table 7.1.5-1.

Table 7.1.5-1: Mapping of downlink power offset field in DCI format 1D to the value.

| Downlink power offset field | [dB] |
| --- | --- |
| 0 | -10log10(2) |
| 1 | 0 |

理解：1Bit用于指示当前PDSCH传输的相对于参考信号的功率分配。0表示PDSCH传输的功率是参考信号传输功率的1/2。1表示PDSCH传输的功率与参考信号传输功率相同。

7.1.5A		Dual layer scheme

For the dual layer transmission scheme of the PDSCH, the UE may assume that an eNB transmission on the PDSCH would be performed with two transmission layers on antenna ports 7 and 8 as defined in Section 6.3.4.4 of [3].

7.1.6		Resource allocation

The UE shall interpret the resource allocation field depending on the PDCCH DCI format detected.  A resource allocation field in each PDCCH includes two parts, a resource allocation header field and information consisting of the actual resource block assignment. PDCCH DCI formats 1, 2, 2A and 2B with type 0 and PDCCH DCI formats 1, 2, 2A and 2B with type 1 resource allocation have the same format and are distinguished from each other via the single bit resource allocation header field which exists depending on the downlink system bandwidth (section 5.3.3.1 of [4]), where type 0 is indicated by 0 value and type 1 is indicated otherwise. PDCCH with DCI format 1A, 1B, 1C and 1D have a type 2 resource allocation while PDCCH with DCI format 1, 2, 2A and 2B have type 0 or type 1 resource allocation. PDCCH DCI formats with a type 2 resource allocation do not have a resource allocation header field. A UE shall discard PDSCH resource allocation in the corresponding PDCCH if consistent control information is not detected.

7.1.6.1	Resource allocation type 0

In resource allocations of type 0, resource block assignment information includes a bitmap indicating the resource block groups (RBGs) that are allocated to the scheduled UE where a RBG is a set of consecutive virtual resource blocks (VRBs) of localized type as defined in section 6.2.3.1 of [3]. Resource block group size (P) is a function of the system bandwidth as shown in Table 7.1.6.1-1. The total number of RBGs () for downlink system bandwidth of is given by where of the RBGs are of size P and if  then one of the RBGs is of size.  The bitmap is of size bits with one bitmap bit per RBG such that each RBG is addressable. The RBGs shall be indexed in the order of increasing frequency and non-increasing RBG sizes starting at the lowest frequency. The order of RBG to bitmap bit mapping is in such way that RBG 0 to RBG are mapped to MSB to LSB of the bitmap. The RBG is allocated to the UE if the corresponding bit value in the bitmap is 1, the RBG is not allocated to the UE otherwise.

Table 7.1.6.1-1: Type 0 Resource Allocation RBG Size vs. Downlink System Bandwidth

| System Bandwidth | RBG Size |
| --- | --- |
|  | (P) |
| ≤10 | 1 |
| 11 – 26 | 2 |
| 27 – 63 | 3 |
| 64 – 110 | 4 |

7.1.6.2	Resource allocation type 1

In resource allocations of type 1, a resource block assignment information of size  indicates to a scheduled UE the VRBs from the set of VRBs from one of P RBG subsets. The virtual resource blocks used are of localized type as defined in section 6.2.3.1 of [3]. Also P is the RBG size associated with the system bandwidth as shown in Table 7.1.6.1-1. A RBG subset , where , consists of every th RBG starting from RBG . The resource block assignment information consists of three fields [4].

The first field with bits is used to indicate the selected RBG subset among  RBG subsets.

The second field with one bit is used to indicate a shift of the resource allocation span within a subset. A bit value of 1 indicates shift is triggered. Shift is not triggered otherwise.

The third field includes a bitmap, where each bit of the bitmap addresses a single VRB in the selected RBG subset in such a way that MSB to LSB of the bitmap are mapped to the VRBs in the increasing frequency order. The VRB is allocated to the UE if the corresponding bit value in the bit field is 1, the VRB is not allocated to the UE otherwise.  The portion of the bitmap used to address VRBs in a selected RBG subset has size  and is defined as

The addressable VRB numbers of a selected RBG subset start from an offset,  to the smallest VRB number within the selected RBG subset, which is mapped to the MSB of the bitmap. The offset is in terms of the number of VRBs and is done within the selected RBG subset. If the value of the bit in the second field for shift of the resource allocation span is set to 0, the offset for RBG subset  is given by . Otherwise, the offset for RBG subset  is given by , where the LSB of the bitmap is justified with the highest VRB number within the selected RBG subset. is the number of VRBs in RBG subset and can be calculated by the following equation,

Consequently, when RBG subset  is indicated, bit  for  in the bitmap field indicates VRB number,

.

7.1.6.3	Resource allocation type 2

In resource allocations of type 2, the resource block assignment information indicates to a scheduled UE a set of contiguously allocated localized virtual resource blocks or distributed virtual resource blocks. In case of resource allocation signalled with PDCCH DCI format 1A, 1B or 1D, one bit flag indicates whether localized virtual resource blocks or distributed virtual resource blocks are assigned (value 0 indicates Localized and value 1 indicates Distributed VRB assignment) while distributed virtual resource blocks are always assigned in case of resource allocation signalled with PDCCH DCI format 1C. Localized VRB allocations for a UE vary from a single VRB up to a maximum number of VRBs spanning the system bandwidth. For DCI format 1A the distributed VRB allocations for a UE vary from a single VRB up to  VRBs, where  is defined in [3], if the DCI CRC is scrambled by P-RNTI, RA-RNTI, or SI-RNTI. With PDCCH DCI format 1B, 1D, or 1A with a CRC scrambled with C-RNTI, distributed VRB allocations for a UE vary from a single VRB up to  VRBs if  is 6-49 and vary from a single VRB up to 16 if  is 50-110. With PDCCH DCI format 1C, distributed VRB allocations for a UE vary from  VRB(s) up to  VRBs with an increment step of , where  value is determined depending on the downlink system bandwidth as shown in Table 7.1.6.3-1.

Table 7.1.6.3-1:  values vs. Downlink System Bandwidth

| System BW () |  |
| --- | --- |
|  | DCI format 1C |
| 6-49 | 2 |
| 50-110 | 4 |

For PDCCH DCI format 1A, 1B or 1D, a type 2 resource allocation field consists of a resource indication value (RIV) corresponding to a starting resource block () and a length in terms of virtually contiguously allocated resource blocks. The resource indication value is defined by

if  then

else

where 1 and shall not exceed .

For PDCCH DCI format 1C, a type 2 resource block assignment field consists of a resource indication value (RIV) corresponding to a starting resource block (=, , ,…, ) and a length in terms of virtually contiguously allocated resource blocks (=, ,…, ).  The resource indication value is defined by

if  then

else

where ,  and . Here,

1 and shall not exceed .

7.1.7		Modulation order and transport block size determination

To determine the modulation order and transport block size(s) in the physical downlink shared channel, the UE shall first

- read the 5-bit “modulation and coding scheme” field () in the DCI

and second if the DCI CRC is scrambled by P-RNTI, RA-RNTI, or SI-RNTI then

- for DCI format 1A:

  - set the Table 7.1.7.2.1-1 column indicator to from Section 5.3.3.1.3 in [4]

理解：在DCI1A的情况下，根据和确定TB块大小。从TPC command for PUCCH域中的2个Bit获取，取值为2或者3

- for DCI format 1C:

  - use Table 7.1.7.2.3-1 for determining its transport block size.

理解：在DCI1C的情况下，根据表7.1.7.2.3-1和获取传输块大小。

else

- set  to the total number of allocated PRBs based on the procedure defined in Section 7.1.6.

if the transport block is transmitted in DwPTS of the special subframe in frame structure type 2,  then

set the Table 7.1.7.2.1-1 column indicator ,

理解：TDD模式下在特殊子帧DwPTS情况下，RB个数需要根据指示的RB进行折算。

else, set the Table 7.1.7.2.1-1 column indicator .

The UE may skip decoding a transport block in an initial transmission if the effective channel code rate is higher than 0.930, where the effective channel code rate is defined as the number of downlink information bits (including CRC bits) divided by the number of physical channel bits on PDSCH. If the UE skips decoding, the physical layer indicates to higher layer that the transport block is not successfully decoded. For the special subframe configurations 0 and 5 with normal downlink CP or configurations 0 and 4 with extended downlink CP, shown in table 4.2-1 [3], there shall be no PDSCH transmission in  DwPTS of the special subframe.理解：在特殊子帧配比0和5的情况下，DwPTS只有3个下行符号，前2个符号用于PDCCH，第3个符号用于PSS信号，没有符号用于PDSCH，所以在此种情况下不传输PDSCH。

理解：信道码率为（TB块大小+CRC大小）/（物理信道Bit数，既根据RE个数和调制方式确定的Bit数）

7.1.7.1	Modulation order determination

The UE shall use = 2 if the DCI CRC is scrambled by P-RNTI, RA-RNTI, or SI-RNTI, otherwise, the UE shall useand Table 7.1.7.1-1 to determine the modulation order () used in the physical downlink shared channel.

理解：如果DCI是被P-RNTI、RA-RNTI和SI-RNTI来加扰的。UE确定使用QPSK的调制方式，不需要根据来查表获取调制方式。被其他的RNTI加扰的情况，需要根据查表获取调制方式。

Table 7.1.7.1-1: Modulation and TBS index table for PDSCH

| MCS Index | Modulation Order | TBS Index |
| --- | --- | --- |
| 0 | 2 | 0 |
| 1 | 2 | 1 |
| 2 | 2 | 2 |
| 3 | 2 | 3 |
| 4 | 2 | 4 |
| 5 | 2 | 5 |
| 6 | 2 | 6 |
| 7 | 2 | 7 |
| 8 | 2 | 8 |
| 9 | 2 | 9 |
| 10 | 4 | 9 |
| 11 | 4 | 10 |
| 12 | 4 | 11 |
| 13 | 4 | 12 |
| 14 | 4 | 13 |
| 15 | 4 | 14 |
| 16 | 4 | 15 |
| 17 | 6 | 15 |
| 18 | 6 | 16 |
| 19 | 6 | 17 |
| 20 | 6 | 18 |
| 21 | 6 | 19 |
| 22 | 6 | 20 |
| 23 | 6 | 21 |
| 24 | 6 | 22 |
| 25 | 6 | 23 |
| 26 | 6 | 24 |
| 27 | 6 | 25 |
| 28 | 6 | 26 |
| 29 | 2 | reserved |
| 30 | 4 |  |
| 31 | 6 |  |

7.1.7.2	Transport block size determination

If the DCI CRC is scrambled by P-RNTI, RA-RNTI, or SI-RNTI then

- for DCI format 1A:

  - the UE shall set the TBS index () equal to  and determine its TBS by the procedure in Section 7.1.7.2.1.

- for DCI format 1C:

  - the UE shall set the TBS index () equal to  and determine its TBS from Table 7.1.7.2.3-1.

理解：DCI 1A和DCI1C的情况下，和相等，不需要查表。

else

- for, the UE shall first determine the TBS index () usingand Table 7.1.7.1-1 except if the transport block is disabled in DCI formats 2, 2A and 2B as specified below. For a transport block that is not mapped to two-layer spatial multiplexing, the TBS is determined by the procedure in Section 7.1.7.2.1. For a transport block that is mapped to two-layer spatial multiplexing, the TBS is determined by the procedure in Section 7.1.7.2.2.

- for, the TBS is assumed to be as determined from DCI transported in the latest PDCCH for the same transport block using . If there is no PDCCH for the same transport block using, and if the initial PDSCH for the same transport block is semi-persistently scheduled, the TBS shall be determined from the most recent semi-persistent scheduling assignment PDCCH.

- In DCI formats 2, 2A and 2B a transport block is disabled if  and if rvidx = 1 otherwise the transport block is enabled.

理解：传输块是否使能是通过MCS和冗余版本号来判决的，当并且rvidx = 1时表示传输块不使能。

The NDI and HARQ process ID, as signalled on PDCCH, and the TBS, as determined above, shall be delivered to higher layers.

7.1.7.2.1	Transport blocks not mapped to two-layer spatial multiplexing

For, the TBS is given by the (,) entry of Table 7.1.7.2.1-1.

Table 7.1.7.2.1-1: Transport block size table (dimension 27×110)

|  | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 16 | 32 | 56 | 88 | 120 | 152 | 176 | 208 | 224 | 256 |
| 1 | 24 | 56 | 88 | 144 | 176 | 208 | 224 | 256 | 328 | 344 |
| 2 | 32 | 72 | 144 | 176 | 208 | 256 | 296 | 328 | 376 | 424 |
| 3 | 40 | 104 | 176 | 208 | 256 | 328 | 392 | 440 | 504 | 568 |
| 4 | 56 | 120 | 208 | 256 | 328 | 408 | 488 | 552 | 632 | 696 |
| 5 | 72 | 144 | 224 | 328 | 424 | 504 | 600 | 680 | 776 | 872 |
| 6 | 328 | 176 | 256 | 392 | 504 | 600 | 712 | 808 | 936 | 1032 |
| 7 | 104 | 224 | 328 | 472 | 584 | 712 | 840 | 968 | 1096 | 1224 |
| 8 | 120 | 256 | 392 | 536 | 680 | 808 | 968 | 1096 | 1256 | 1384 |
| 9 | 136 | 296 | 456 | 616 | 776 | 936 | 1096 | 1256 | 1416 | 1544 |
| 10 | 144 | 328 | 504 | 680 | 872 | 1032 | 1224 | 1384 | 1544 | 1736 |
| 11 | 176 | 376 | 584 | 776 | 1000 | 1192 | 1384 | 1608 | 1800 | 2024 |
| 12 | 208 | 440 | 680 | 904 | 1128 | 1352 | 1608 | 1800 | 2024 | 2280 |
| 13 | 224 | 488 | 744 | 1000 | 1256 | 1544 | 1800 | 2024 | 2280 | 2536 |
| 14 | 256 | 552 | 840 | 1128 | 1416 | 1736 | 1992 | 2280 | 2600 | 2856 |
| 15 | 280 | 600 | 904 | 1224 | 1544 | 1800 | 2152 | 2472 | 2728 | 3112 |
| 16 | 328 | 632 | 968 | 1288 | 1608 | 1928 | 2280 | 2600 | 2984 | 3240 |
| 17 | 336 | 696 | 1064 | 1416 | 1800 | 2152 | 2536 | 2856 | 3240 | 3624 |
| 18 | 376 | 776 | 1160 | 1544 | 1992 | 2344 | 2792 | 3112 | 3624 | 4008 |
| 19 | 408 | 840 | 1288 | 1736 | 2152 | 2600 | 2984 | 3496 | 3880 | 4264 |
| 20 | 440 | 904 | 1384 | 1864 | 2344 | 2792 | 3240 | 3752 | 4136 | 4584 |
| 21 | 488 | 1000 | 1480 | 1992 | 2472 | 2984 | 3496 | 4008 | 4584 | 4968 |
| 22 | 520 | 1064 | 1608 | 2152 | 2664 | 3240 | 3752 | 4264 | 4776 | 5352 |
| 23 | 552 | 1128 | 1736 | 2280 | 2856 | 3496 | 4008 | 4584 | 5160 | 5736 |
| 24 | 584 | 1192 | 1800 | 2408 | 2984 | 3624 | 4264 | 4968 | 5544 | 5992 |
| 25 | 616 | 1256 | 1864 | 2536 | 3112 | 3752 | 4392 | 5160 | 5736 | 6200 |
| 26 | 712 | 1480 | 2216 | 2984 | 3752 | 4392 | 5160 | 5992 | 6712 | 7480 |
|  | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 | 20 |
| 0 | 288 | 328 | 344 | 376 | 392 | 424 | 456 | 488 | 504 | 536 |
| 1 | 376 | 424 | 456 | 488 | 520 | 568 | 600 | 632 | 680 | 712 |
| 2 | 472 | 520 | 568 | 616 | 648 | 696 | 744 | 776 | 840 | 872 |
| 3 | 616 | 680 | 744 | 808 | 872 | 904 | 968 | 1032 | 1096 | 1160 |
| 4 | 776 | 840 | 904 | 1000 | 1064 | 1128 | 1192 | 1288 | 1352 | 1416 |
| 5 | 968 | 1032 | 1128 | 1224 | 1320 | 1384 | 1480 | 1544 | 1672 | 1736 |
| 6 | 1128 | 1224 | 1352 | 1480 | 1544 | 1672 | 1736 | 1864 | 1992 | 2088 |
| 7 | 1320 | 1480 | 1608 | 1672 | 1800 | 1928 | 2088 | 2216 | 2344 | 2472 |
| 8 | 1544 | 1672 | 1800 | 1928 | 2088 | 2216 | 2344 | 2536 | 2664 | 2792 |
| 9 | 1736 | 1864 | 2024 | 2216 | 2344 | 2536 | 2664 | 2856 | 2984 | 3112 |
| 10 | 1928 | 2088 | 2280 | 2472 | 2664 | 2792 | 2984 | 3112 | 3368 | 3496 |
| 11 | 2216 | 2408 | 2600 | 2792 | 2984 | 3240 | 3496 | 3624 | 3880 | 4008 |
| 12 | 2472 | 2728 | 2984 | 3240 | 3368 | 3624 | 3880 | 4136 | 4392 | 4584 |
| 13 | 2856 | 3112 | 3368 | 3624 | 3880 | 4136 | 4392 | 4584 | 4968 | 5160 |
| 14 | 3112 | 3496 | 3752 | 4008 | 4264 | 4584 | 4968 | 5160 | 5544 | 5736 |
| 15 | 3368 | 3624 | 4008 | 4264 | 4584 | 4968 | 5160 | 5544 | 5736 | 6200 |
| 16 | 3624 | 3880 | 4264 | 4584 | 4968 | 5160 | 5544 | 5992 | 6200 | 6456 |
| 17 | 4008 | 4392 | 4776 | 5160 | 5352 | 5736 | 6200 | 6456 | 6712 | 7224 |
| 18 | 4392 | 4776 | 5160 | 5544 | 5992 | 6200 | 6712 | 7224 | 7480 | 7992 |
| 19 | 4776 | 5160 | 5544 | 5992 | 6456 | 6968 | 7224 | 7736 | 8248 | 8504 |
| 20 | 5160 | 5544 | 5992 | 6456 | 6968 | 7480 | 7992 | 8248 | 8760 | 9144 |
| 21 | 5544 | 5992 | 6456 | 6968 | 7480 | 7992 | 8504 | 9144 | 9528 | 9912 |
| 22 | 5992 | 6456 | 6968 | 7480 | 7992 | 8504 | 9144 | 9528 | 10296 | 10680 |
| 23 | 6200 | 6968 | 7480 | 7992 | 8504 | 9144 | 9912 | 10296 | 11064 | 11448 |
| 24 | 6712 | 7224 | 7992 | 8504 | 9144 | 9912 | 10296 | 11064 | 11448 | 12216 |
| 25 | 6968 | 7480 | 8248 | 8760 | 9528 | 10296 | 10680 | 11448 | 12216 | 12576 |
| 26 | 8248 | 8760 | 9528 | 10296 | 11064 | 11832 | 12576 | 13536 | 14112 | 14688 |
|  | 21 | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 | 30 |
| 0 | 568 | 600 | 616 | 648 | 680 | 712 | 744 | 776 | 776 | 808 |
| 1 | 744 | 776 | 808 | 872 | 904 | 936 | 968 | 1000 | 1032 | 1064 |
| 2 | 936 | 968 | 1000 | 1064 | 1096 | 1160 | 1192 | 1256 | 1288 | 1320 |
| 3 | 1224 | 1256 | 1320 | 1384 | 1416 | 1480 | 1544 | 1608 | 1672 | 1736 |
| 4 | 1480 | 1544 | 1608 | 1736 | 1800 | 1864 | 1928 | 1992 | 2088 | 2152 |
| 5 | 1864 | 1928 | 2024 | 2088 | 2216 | 2280 | 2344 | 2472 | 2536 | 2664 |
| 6 | 2216 | 2280 | 2408 | 2472 | 2600 | 2728 | 2792 | 2984 | 2984 | 3112 |
| 7 | 2536 | 2664 | 2792 | 2984 | 3112 | 3240 | 3368 | 3368 | 3496 | 3624 |
| 8 | 2984 | 3112 | 3240 | 3368 | 3496 | 3624 | 3752 | 3880 | 4008 | 4264 |
| 9 | 3368 | 3496 | 3624 | 3752 | 4008 | 4136 | 4264 | 4392 | 4584 | 4776 |
| 10 | 3752 | 3880 | 4008 | 4264 | 4392 | 4584 | 4776 | 4968 | 5160 | 5352 |
| 11 | 4264 | 4392 | 4584 | 4776 | 4968 | 5352 | 5544 | 5736 | 5992 | 5992 |
| 12 | 4776 | 4968 | 5352 | 5544 | 5736 | 5992 | 6200 | 6456 | 6712 | 6712 |
| 13 | 5352 | 5736 | 5992 | 6200 | 6456 | 6712 | 6968 | 7224 | 7480 | 7736 |
| 14 | 5992 | 6200 | 6456 | 6968 | 7224 | 7480 | 7736 | 7992 | 8248 | 8504 |
| 15 | 6456 | 6712 | 6968 | 7224 | 7736 | 7992 | 8248 | 8504 | 8760 | 9144 |
| 16 | 6712 | 7224 | 7480 | 7736 | 7992 | 8504 | 8760 | 9144 | 9528 | 9912 |
| 17 | 7480 | 7992 | 8248 | 8760 | 9144 | 9528 | 9912 | 10296 | 10296 | 10680 |
| 18 | 8248 | 8760 | 9144 | 9528 | 9912 | 10296 | 10680 | 11064 | 11448 | 11832 |
| 19 | 9144 | 9528 | 9912 | 10296 | 10680 | 11064 | 11448 | 12216 | 12576 | 12960 |
| 20 | 9912 | 10296 | 10680 | 11064 | 11448 | 12216 | 12576 | 12960 | 13536 | 14112 |
| 21 | 10680 | 11064 | 11448 | 12216 | 12576 | 12960 | 13536 | 14112 | 14688 | 15264 |
| 22 | 11448 | 11832 | 12576 | 12960 | 13536 | 14112 | 14688 | 15264 | 15840 | 16416 |
| 23 | 12216 | 12576 | 12960 | 13536 | 14112 | 14688 | 15264 | 15840 | 16416 | 16992 |
| 24 | 12960 | 13536 | 14112 | 14688 | 15264 | 15840 | 16416 | 16992 | 17568 | 18336 |
| 25 | 13536 | 14112 | 14688 | 15264 | 15840 | 16416 | 16992 | 17568 | 18336 | 19080 |
| 26 | 15264 | 16416 | 16992 | 17568 | 18336 | 19080 | 19848 | 20616 | 21384 | 22152 |
|  | 31 | 32 | 33 | 34 | 35 | 36 | 37 | 38 | 39 | 40 |
| 0 | 840 | 872 | 904 | 936 | 968 | 1000 | 1032 | 1032 | 1064 | 1096 |
| 1 | 1128 | 1160 | 1192 | 1224 | 1256 | 1288 | 1352 | 1384 | 1416 | 1416 |
| 2 | 1384 | 1416 | 1480 | 1544 | 1544 | 1608 | 1672 | 1672 | 1736 | 1800 |
| 3 | 1800 | 1864 | 1928 | 1992 | 2024 | 2088 | 2152 | 2216 | 2280 | 2344 |
| 4 | 2216 | 2280 | 2344 | 2408 | 2472 | 2600 | 2664 | 2728 | 2792 | 2856 |
| 5 | 2728 | 2792 | 2856 | 2984 | 3112 | 3112 | 3240 | 3368 | 3496 | 3496 |
| 6 | 3240 | 3368 | 3496 | 3496 | 3624 | 3752 | 3880 | 4008 | 4136 | 4136 |
| 7 | 3752 | 3880 | 4008 | 4136 | 4264 | 4392 | 4584 | 4584 | 4776 | 4968 |
| 8 | 4392 | 4584 | 4584 | 4776 | 4968 | 4968 | 5160 | 5352 | 5544 | 5544 |
| 9 | 4968 | 5160 | 5160 | 5352 | 5544 | 5736 | 5736 | 5992 | 6200 | 6200 |
| 10 | 5544 | 5736 | 5736 | 5992 | 6200 | 6200 | 6456 | 6712 | 6712 | 6968 |
| 11 | 6200 | 6456 | 6712 | 6968 | 6968 | 7224 | 7480 | 7736 | 7736 | 7992 |
| 12 | 6968 | 7224 | 7480 | 7736 | 7992 | 8248 | 8504 | 8760 | 8760 | 9144 |
| 13 | 7992 | 8248 | 8504 | 8760 | 9144 | 9144 | 9528 | 9912 | 9912 | 10296 |
| 14 | 8760 | 9144 | 9528 | 9912 | 9912 | 10296 | 10680 | 11064 | 11064 | 11448 |
| 15 | 9528 | 9912 | 10296 | 10296 | 10680 | 11064 | 11448 | 11832 | 11832 | 12216 |
| 16 | 9912 | 10296 | 10680 | 11064 | 11448 | 11832 | 12216 | 12216 | 12576 | 12960 |
| 17 | 11064 | 11448 | 11832 | 12216 | 12576 | 12960 | 13536 | 13536 | 14112 | 14688 |
| 18 | 12216 | 12576 | 12960 | 13536 | 14112 | 14112 | 14688 | 15264 | 15264 | 15840 |
| 19 | 13536 | 13536 | 14112 | 14688 | 15264 | 15264 | 15840 | 16416 | 16992 | 16992 |
| 20 | 14688 | 14688 | 15264 | 15840 | 16416 | 16992 | 16992 | 17568 | 18336 | 18336 |
| 21 | 15840 | 15840 | 16416 | 16992 | 17568 | 18336 | 18336 | 19080 | 19848 | 19848 |
| 22 | 16992 | 16992 | 17568 | 18336 | 19080 | 19080 | 19848 | 20616 | 21384 | 21384 |
| 23 | 17568 | 18336 | 19080 | 19848 | 19848 | 20616 | 21384 | 22152 | 22152 | 22920 |
| 24 | 19080 | 19848 | 19848 | 20616 | 21384 | 22152 | 22920 | 22920 | 23688 | 24496 |
| 25 | 19848 | 20616 | 20616 | 21384 | 22152 | 22920 | 23688 | 24496 | 24496 | 25456 |
| 26 | 22920 | 23688 | 24496 | 25456 | 25456 | 26416 | 27376 | 28336 | 29296 | 29296 |
|  | 41 | 42 | 43 | 44 | 45 | 46 | 47 | 48 | 49 | 50 |
| 0 | 1128 | 1160 | 1192 | 1224 | 1256 | 1256 | 1288 | 1320 | 1352 | 1384 |
| 1 | 1480 | 1544 | 1544 | 1608 | 1608 | 1672 | 1736 | 1736 | 1800 | 1800 |
| 2 | 1800 | 1864 | 1928 | 1992 | 2024 | 2088 | 2088 | 2152 | 2216 | 2216 |
| 3 | 2408 | 2472 | 2536 | 2536 | 2600 | 2664 | 2728 | 2792 | 2856 | 2856 |
| 4 | 2984 | 2984 | 3112 | 3112 | 3240 | 3240 | 3368 | 3496 | 3496 | 3624 |
| 5 | 3624 | 3752 | 3752 | 3880 | 4008 | 4008 | 4136 | 4264 | 4392 | 4392 |
| 6 | 4264 | 4392 | 4584 | 4584 | 4776 | 4776 | 4968 | 4968 | 5160 | 5160 |
| 7 | 4968 | 5160 | 5352 | 5352 | 5544 | 5736 | 5736 | 5992 | 5992 | 6200 |
| 8 | 5736 | 5992 | 5992 | 6200 | 6200 | 6456 | 6456 | 6712 | 6968 | 6968 |
| 9 | 6456 | 6712 | 6712 | 6968 | 6968 | 7224 | 7480 | 7480 | 7736 | 7992 |
| 10 | 7224 | 7480 | 7480 | 7736 | 7992 | 7992 | 8248 | 8504 | 8504 | 8760 |
| 11 | 8248 | 8504 | 8760 | 8760 | 9144 | 9144 | 9528 | 9528 | 9912 | 9912 |
| 12 | 9528 | 9528 | 9912 | 9912 | 10296 | 10680 | 10680 | 11064 | 11064 | 11448 |
| 13 | 10680 | 10680 | 11064 | 11448 | 11448 | 11832 | 12216 | 12216 | 12576 | 12960 |
| 14 | 11832 | 12216 | 12216 | 12576 | 12960 | 12960 | 13536 | 13536 | 14112 | 14112 |
| 15 | 12576 | 12960 | 12960 | 13536 | 13536 | 14112 | 14688 | 14688 | 15264 | 15264 |
| 16 | 13536 | 13536 | 14112 | 14112 | 14688 | 14688 | 15264 | 15840 | 15840 | 16416 |
| 17 | 14688 | 15264 | 15264 | 15840 | 16416 | 16416 | 16992 | 17568 | 17568 | 18336 |
| 18 | 16416 | 16416 | 16992 | 17568 | 17568 | 18336 | 18336 | 19080 | 19080 | 19848 |
| 19 | 17568 | 18336 | 18336 | 19080 | 19080 | 19848 | 20616 | 20616 | 21384 | 21384 |
| 20 | 19080 | 19848 | 19848 | 20616 | 20616 | 21384 | 22152 | 22152 | 22920 | 22920 |
| 21 | 20616 | 21384 | 21384 | 22152 | 22920 | 22920 | 23688 | 24496 | 24496 | 25456 |
| 22 | 22152 | 22920 | 22920 | 23688 | 24496 | 24496 | 25456 | 25456 | 26416 | 27376 |
| 23 | 23688 | 24496 | 24496 | 25456 | 25456 | 26416 | 27376 | 27376 | 28336 | 28336 |
| 24 | 25456 | 25456 | 26416 | 26416 | 27376 | 28336 | 28336 | 29296 | 29296 | 30576 |
| 25 | 26416 | 26416 | 27376 | 28336 | 28336 | 29296 | 29296 | 30576 | 31704 | 31704 |
| 26 | 30576 | 30576 | 31704 | 32856 | 32856 | 34008 | 35160 | 35160 | 36696 | 36696 |
|  | 51 | 52 | 53 | 54 | 55 | 56 | 57 | 58 | 59 | 60 |
| 0 | 1416 | 1416 | 1480 | 1480 | 1544 | 1544 | 1608 | 1608 | 1608 | 1672 |
| 1 | 1864 | 1864 | 1928 | 1992 | 1992 | 2024 | 2088 | 2088 | 2152 | 2152 |
| 2 | 2280 | 2344 | 2344 | 2408 | 2472 | 2536 | 2536 | 2600 | 2664 | 2664 |
| 3 | 2984 | 2984 | 3112 | 3112 | 3240 | 3240 | 3368 | 3368 | 3496 | 3496 |
| 4 | 3624 | 3752 | 3752 | 3880 | 4008 | 4008 | 4136 | 4136 | 4264 | 4264 |
| 5 | 4584 | 4584 | 4776 | 4776 | 4776 | 4968 | 4968 | 5160 | 5160 | 5352 |
| 6 | 5352 | 5352 | 5544 | 5736 | 5736 | 5992 | 5992 | 5992 | 6200 | 6200 |
| 7 | 6200 | 6456 | 6456 | 6712 | 6712 | 6712 | 6968 | 6968 | 7224 | 7224 |
| 8 | 7224 | 7224 | 7480 | 7480 | 7736 | 7736 | 7992 | 7992 | 8248 | 8504 |
| 9 | 7992 | 8248 | 8248 | 8504 | 8760 | 8760 | 9144 | 9144 | 9144 | 9528 |
| 10 | 9144 | 9144 | 9144 | 9528 | 9528 | 9912 | 9912 | 10296 | 10296 | 10680 |
| 11 | 10296 | 10680 | 10680 | 11064 | 11064 | 11448 | 11448 | 11832 | 11832 | 12216 |
| 12 | 11832 | 11832 | 12216 | 12216 | 12576 | 12576 | 12960 | 12960 | 13536 | 13536 |
| 13 | 12960 | 13536 | 13536 | 14112 | 14112 | 14688 | 14688 | 14688 | 15264 | 15264 |
| 14 | 14688 | 14688 | 15264 | 15264 | 15840 | 15840 | 16416 | 16416 | 16992 | 16992 |
| 15 | 15840 | 15840 | 16416 | 16416 | 16992 | 16992 | 17568 | 17568 | 18336 | 18336 |
| 16 | 16416 | 16992 | 16992 | 17568 | 17568 | 18336 | 18336 | 19080 | 19080 | 19848 |
| 17 | 18336 | 19080 | 19080 | 19848 | 19848 | 20616 | 20616 | 20616 | 21384 | 21384 |
| 18 | 19848 | 20616 | 21384 | 21384 | 22152 | 22152 | 22920 | 22920 | 23688 | 23688 |
| 19 | 22152 | 22152 | 22920 | 22920 | 23688 | 24496 | 24496 | 25456 | 25456 | 25456 |
| 20 | 23688 | 24496 | 24496 | 25456 | 25456 | 26416 | 26416 | 27376 | 27376 | 28336 |
| 21 | 25456 | 26416 | 26416 | 27376 | 27376 | 28336 | 28336 | 29296 | 29296 | 30576 |
| 22 | 27376 | 28336 | 28336 | 29296 | 29296 | 30576 | 30576 | 31704 | 31704 | 32856 |
| 23 | 29296 | 29296 | 30576 | 30576 | 31704 | 31704 | 32856 | 32856 | 34008 | 34008 |
| 24 | 31704 | 31704 | 32856 | 32856 | 34008 | 34008 | 35160 | 35160 | 36696 | 36696 |
| 25 | 32856 | 32856 | 34008 | 34008 | 35160 | 35160 | 36696 | 36696 | 37888 | 37888 |
| 26 | 37888 | 37888 | 39232 | 40576 | 40576 | 40576 | 42368 | 42368 | 43816 | 43816 |
|  | 61 | 62 | 63 | 64 | 65 | 66 | 67 | 68 | 69 | 70 |
| 0 | 1672 | 1736 | 1736 | 1800 | 1800 | 1800 | 1864 | 1864 | 1928 | 1928 |
| 1 | 2216 | 2280 | 2280 | 2344 | 2344 | 2408 | 2472 | 2472 | 2536 | 2536 |
| 2 | 2728 | 2792 | 2856 | 2856 | 2856 | 2984 | 2984 | 3112 | 3112 | 3112 |
| 3 | 3624 | 3624 | 3624 | 3752 | 3752 | 3880 | 3880 | 4008 | 4008 | 4136 |
| 4 | 4392 | 4392 | 4584 | 4584 | 4584 | 4776 | 4776 | 4968 | 4968 | 4968 |
| 5 | 5352 | 5544 | 5544 | 5736 | 5736 | 5736 | 5992 | 5992 | 5992 | 6200 |
| 6 | 6456 | 6456 | 6456 | 6712 | 6712 | 6968 | 6968 | 6968 | 7224 | 7224 |
| 7 | 7480 | 7480 | 7736 | 7736 | 7992 | 7992 | 8248 | 8248 | 8504 | 8504 |
| 8 | 8504 | 8760 | 8760 | 9144 | 9144 | 9144 | 9528 | 9528 | 9528 | 9912 |
| 9 | 9528 | 9912 | 9912 | 10296 | 10296 | 10296 | 10680 | 10680 | 11064 | 11064 |
| 10 | 10680 | 11064 | 11064 | 11448 | 11448 | 11448 | 11832 | 11832 | 12216 | 12216 |
| 11 | 12216 | 12576 | 12576 | 12960 | 12960 | 13536 | 13536 | 13536 | 14112 | 14112 |
| 12 | 14112 | 14112 | 14112 | 14688 | 14688 | 15264 | 15264 | 15264 | 15840 | 15840 |
| 13 | 15840 | 15840 | 16416 | 16416 | 16992 | 16992 | 16992 | 17568 | 17568 | 18336 |
| 14 | 17568 | 17568 | 18336 | 18336 | 18336 | 19080 | 19080 | 19848 | 19848 | 19848 |
| 15 | 18336 | 19080 | 19080 | 19848 | 19848 | 20616 | 20616 | 20616 | 21384 | 21384 |
| 16 | 19848 | 19848 | 20616 | 20616 | 21384 | 21384 | 22152 | 22152 | 22152 | 22920 |
| 17 | 22152 | 22152 | 22920 | 22920 | 23688 | 23688 | 24496 | 24496 | 24496 | 25456 |
| 18 | 24496 | 24496 | 24496 | 25456 | 25456 | 26416 | 26416 | 27376 | 27376 | 27376 |
| 19 | 26416 | 26416 | 27376 | 27376 | 28336 | 28336 | 29296 | 29296 | 29296 | 30576 |
| 20 | 28336 | 29296 | 29296 | 29296 | 30576 | 30576 | 31704 | 31704 | 31704 | 32856 |
| 21 | 30576 | 31704 | 31704 | 31704 | 32856 | 32856 | 34008 | 34008 | 35160 | 35160 |
| 22 | 32856 | 34008 | 34008 | 34008 | 35160 | 35160 | 36696 | 36696 | 36696 | 37888 |
| 23 | 35160 | 35160 | 36696 | 36696 | 37888 | 37888 | 37888 | 39232 | 39232 | 40576 |
| 24 | 36696 | 37888 | 37888 | 39232 | 39232 | 40576 | 40576 | 42368 | 42368 | 42368 |
| 25 | 39232 | 39232 | 40576 | 40576 | 40576 | 42368 | 42368 | 43816 | 43816 | 43816 |
| 26 | 45352 | 45352 | 46888 | 46888 | 48936 | 48936 | 48936 | 51024 | 51024 | 52752 |
|  | 71 | 72 | 73 | 74 | 75 | 76 | 77 | 78 | 79 | 80 |
| 0 | 1992 | 1992 | 2024 | 2088 | 2088 | 2088 | 2152 | 2152 | 2216 | 2216 |
| 1 | 2600 | 2600 | 2664 | 2728 | 2728 | 2792 | 2792 | 2856 | 2856 | 2856 |
| 2 | 3240 | 3240 | 3240 | 3368 | 3368 | 3368 | 3496 | 3496 | 3496 | 3624 |
| 3 | 4136 | 4264 | 4264 | 4392 | 4392 | 4392 | 4584 | 4584 | 4584 | 4776 |
| 4 | 5160 | 5160 | 5160 | 5352 | 5352 | 5544 | 5544 | 5544 | 5736 | 5736 |
| 5 | 6200 | 6200 | 6456 | 6456 | 6712 | 6712 | 6712 | 6968 | 6968 | 6968 |
| 6 | 7480 | 7480 | 7736 | 7736 | 7736 | 7992 | 7992 | 8248 | 8248 | 8248 |
| 7 | 8760 | 8760 | 8760 | 9144 | 9144 | 9144 | 9528 | 9528 | 9528 | 9912 |
| 8 | 9912 | 9912 | 10296 | 10296 | 10680 | 10680 | 10680 | 11064 | 11064 | 11064 |
| 9 | 11064 | 11448 | 11448 | 11832 | 11832 | 11832 | 12216 | 12216 | 12576 | 12576 |
| 10 | 12576 | 12576 | 12960 | 12960 | 12960 | 13536 | 13536 | 13536 | 14112 | 14112 |
| 11 | 14112 | 14688 | 14688 | 14688 | 15264 | 15264 | 15840 | 15840 | 15840 | 16416 |
| 12 | 16416 | 16416 | 16416 | 16992 | 16992 | 17568 | 17568 | 17568 | 18336 | 18336 |
| 13 | 18336 | 18336 | 19080 | 19080 | 19080 | 19848 | 19848 | 19848 | 20616 | 20616 |
| 14 | 20616 | 20616 | 20616 | 21384 | 21384 | 22152 | 22152 | 22152 | 22920 | 22920 |
| 15 | 22152 | 22152 | 22152 | 22920 | 22920 | 23688 | 23688 | 23688 | 24496 | 24496 |
| 16 | 22920 | 23688 | 23688 | 24496 | 24496 | 24496 | 25456 | 25456 | 25456 | 26416 |
| 17 | 25456 | 26416 | 26416 | 26416 | 27376 | 27376 | 27376 | 28336 | 28336 | 29296 |
| 18 | 28336 | 28336 | 29296 | 29296 | 29296 | 30576 | 30576 | 30576 | 31704 | 31704 |
| 19 | 30576 | 30576 | 31704 | 31704 | 32856 | 32856 | 32856 | 34008 | 34008 | 34008 |
| 20 | 32856 | 34008 | 34008 | 34008 | 35160 | 35160 | 35160 | 36696 | 36696 | 36696 |
| 21 | 35160 | 36696 | 36696 | 36696 | 37888 | 37888 | 39232 | 39232 | 39232 | 40576 |
| 22 | 37888 | 39232 | 39232 | 40576 | 40576 | 40576 | 42368 | 42368 | 42368 | 43816 |
| 23 | 40576 | 40576 | 42368 | 42368 | 43816 | 43816 | 43816 | 45352 | 45352 | 45352 |
| 24 | 43816 | 43816 | 45352 | 45352 | 45352 | 46888 | 46888 | 46888 | 48936 | 48936 |
| 25 | 45352 | 45352 | 46888 | 46888 | 46888 | 48936 | 48936 | 48936 | 51024 | 51024 |
| 26 | 52752 | 52752 | 55056 | 55056 | 55056 | 55056 | 57336 | 57336 | 57336 | 59256 |
|  | 81 | 82 | 83 | 84 | 85 | 86 | 87 | 88 | 89 | 90 |
| 0 | 2280 | 2280 | 2280 | 2344 | 2344 | 2408 | 2408 | 2472 | 2472 | 2536 |
| 1 | 2984 | 2984 | 2984 | 3112 | 3112 | 3112 | 3240 | 3240 | 3240 | 3240 |
| 2 | 3624 | 3624 | 3752 | 3752 | 3880 | 3880 | 3880 | 4008 | 4008 | 4008 |
| 3 | 4776 | 4776 | 4776 | 4968 | 4968 | 4968 | 5160 | 5160 | 5160 | 5352 |
| 4 | 5736 | 5992 | 5992 | 5992 | 5992 | 6200 | 6200 | 6200 | 6456 | 6456 |
| 5 | 7224 | 7224 | 7224 | 7480 | 7480 | 7480 | 7736 | 7736 | 7736 | 7992 |
| 6 | 8504 | 8504 | 8760 | 8760 | 8760 | 9144 | 9144 | 9144 | 9144 | 9528 |
| 7 | 9912 | 9912 | 10296 | 10296 | 10296 | 10680 | 10680 | 10680 | 11064 | 11064 |
| 8 | 11448 | 11448 | 11448 | 11832 | 11832 | 12216 | 12216 | 12216 | 12576 | 12576 |
| 9 | 12960 | 12960 | 12960 | 13536 | 13536 | 13536 | 13536 | 14112 | 14112 | 14112 |
| 10 | 14112 | 14688 | 14688 | 14688 | 14688 | 15264 | 15264 | 15264 | 15840 | 15840 |
| 11 | 16416 | 16416 | 16992 | 16992 | 16992 | 17568 | 17568 | 17568 | 18336 | 18336 |
| 12 | 18336 | 19080 | 19080 | 19080 | 19080 | 19848 | 19848 | 19848 | 20616 | 20616 |
| 13 | 20616 | 21384 | 21384 | 21384 | 22152 | 22152 | 22152 | 22920 | 22920 | 22920 |
| 14 | 22920 | 23688 | 23688 | 24496 | 24496 | 24496 | 25456 | 25456 | 25456 | 25456 |
| 15 | 24496 | 25456 | 25456 | 25456 | 26416 | 26416 | 26416 | 27376 | 27376 | 27376 |
| 16 | 26416 | 26416 | 27376 | 27376 | 27376 | 28336 | 28336 | 28336 | 29296 | 29296 |
| 17 | 29296 | 29296 | 30576 | 30576 | 30576 | 30576 | 31704 | 31704 | 31704 | 32856 |
| 18 | 31704 | 32856 | 32856 | 32856 | 34008 | 34008 | 34008 | 35160 | 35160 | 35160 |
| 19 | 35160 | 35160 | 35160 | 36696 | 36696 | 36696 | 37888 | 37888 | 37888 | 39232 |
| 20 | 37888 | 37888 | 39232 | 39232 | 39232 | 40576 | 40576 | 40576 | 42368 | 42368 |
| 21 | 40576 | 40576 | 42368 | 42368 | 42368 | 43816 | 43816 | 43816 | 45352 | 45352 |
| 22 | 43816 | 43816 | 45352 | 45352 | 45352 | 46888 | 46888 | 46888 | 48936 | 48936 |
| 23 | 46888 | 46888 | 46888 | 48936 | 48936 | 48936 | 51024 | 51024 | 51024 | 51024 |
| 24 | 48936 | 51024 | 51024 | 51024 | 52752 | 52752 | 52752 | 52752 | 55056 | 55056 |
| 25 | 51024 | 52752 | 52752 | 52752 | 55056 | 55056 | 55056 | 55056 | 57336 | 57336 |
| 26 | 59256 | 59256 | 61664 | 61664 | 61664 | 63776 | 63776 | 63776 | 66592 | 66592 |
|  | 91 | 92 | 93 | 94 | 95 | 96 | 97 | 98 | 99 | 100 |
| 0 | 2536 | 2536 | 2600 | 2600 | 2664 | 2664 | 2728 | 2728 | 2728 | 2792 |
| 1 | 3368 | 3368 | 3368 | 3496 | 3496 | 3496 | 3496 | 3624 | 3624 | 3624 |
| 2 | 4136 | 4136 | 4136 | 4264 | 4264 | 4264 | 4392 | 4392 | 4392 | 4584 |
| 3 | 5352 | 5352 | 5352 | 5544 | 5544 | 5544 | 5736 | 5736 | 5736 | 5736 |
| 4 | 6456 | 6456 | 6712 | 6712 | 6712 | 6968 | 6968 | 6968 | 6968 | 7224 |
| 5 | 7992 | 7992 | 8248 | 8248 | 8248 | 8504 | 8504 | 8760 | 8760 | 8760 |
| 6 | 9528 | 9528 | 9528 | 9912 | 9912 | 9912 | 10296 | 10296 | 10296 | 10296 |
| 7 | 11064 | 11448 | 11448 | 11448 | 11448 | 11832 | 11832 | 11832 | 12216 | 12216 |
| 8 | 12576 | 12960 | 12960 | 12960 | 13536 | 13536 | 13536 | 13536 | 14112 | 14112 |
| 9 | 14112 | 14688 | 14688 | 14688 | 15264 | 15264 | 15264 | 15264 | 15840 | 15840 |
| 10 | 15840 | 16416 | 16416 | 16416 | 16992 | 16992 | 16992 | 16992 | 17568 | 17568 |
| 11 | 18336 | 18336 | 19080 | 19080 | 19080 | 19080 | 19848 | 19848 | 19848 | 19848 |
| 12 | 20616 | 21384 | 21384 | 21384 | 21384 | 22152 | 22152 | 22152 | 22920 | 22920 |
| 13 | 23688 | 23688 | 23688 | 24496 | 24496 | 24496 | 25456 | 25456 | 25456 | 25456 |
| 14 | 26416 | 26416 | 26416 | 27376 | 27376 | 27376 | 28336 | 28336 | 28336 | 28336 |
| 15 | 28336 | 28336 | 28336 | 29296 | 29296 | 29296 | 29296 | 30576 | 30576 | 30576 |
| 16 | 29296 | 30576 | 30576 | 30576 | 30576 | 31704 | 31704 | 31704 | 31704 | 32856 |
| 17 | 32856 | 32856 | 34008 | 34008 | 34008 | 35160 | 35160 | 35160 | 35160 | 36696 |
| 18 | 36696 | 36696 | 36696 | 37888 | 37888 | 37888 | 37888 | 39232 | 39232 | 39232 |
| 19 | 39232 | 39232 | 40576 | 40576 | 40576 | 40576 | 42368 | 42368 | 42368 | 43816 |
| 20 | 42368 | 42368 | 43816 | 43816 | 43816 | 45352 | 45352 | 45352 | 46888 | 46888 |
| 21 | 45352 | 46888 | 46888 | 46888 | 46888 | 48936 | 48936 | 48936 | 48936 | 51024 |
| 22 | 48936 | 48936 | 51024 | 51024 | 51024 | 51024 | 52752 | 52752 | 52752 | 55056 |
| 23 | 52752 | 52752 | 52752 | 55056 | 55056 | 55056 | 55056 | 57336 | 57336 | 57336 |
| 24 | 55056 | 57336 | 57336 | 57336 | 57336 | 59256 | 59256 | 59256 | 61664 | 61664 |
| 25 | 57336 | 59256 | 59256 | 59256 | 61664 | 61664 | 61664 | 61664 | 63776 | 63776 |
| 26 | 66592 | 68808 | 68808 | 68808 | 71112 | 71112 | 71112 | 73712 | 73712 | 75376 |
|  | 101 | 102 | 103 | 104 | 105 | 106 | 107 | 108 | 109 | 110 |
| 0 | 2792 | 2856 | 2856 | 2856 | 2984 | 2984 | 2984 | 2984 | 2984 | 3112 |
| 1 | 3752 | 3752 | 3752 | 3752 | 3880 | 3880 | 3880 | 4008 | 4008 | 4008 |
| 2 | 4584 | 4584 | 4584 | 4584 | 4776 | 4776 | 4776 | 4776 | 4968 | 4968 |
| 3 | 5992 | 5992 | 5992 | 5992 | 6200 | 6200 | 6200 | 6200 | 6456 | 6456 |
| 4 | 7224 | 7224 | 7480 | 7480 | 7480 | 7480 | 7736 | 7736 | 7736 | 7992 |
| 5 | 8760 | 9144 | 9144 | 9144 | 9144 | 9528 | 9528 | 9528 | 9528 | 9528 |
| 6 | 10680 | 10680 | 10680 | 10680 | 11064 | 11064 | 11064 | 11448 | 11448 | 11448 |
| 7 | 12216 | 12576 | 12576 | 12576 | 12960 | 12960 | 12960 | 12960 | 13536 | 13536 |
| 8 | 14112 | 14112 | 14688 | 14688 | 14688 | 14688 | 15264 | 15264 | 15264 | 15264 |
| 9 | 15840 | 16416 | 16416 | 16416 | 16416 | 16992 | 16992 | 16992 | 16992 | 17568 |
| 10 | 17568 | 18336 | 18336 | 18336 | 18336 | 18336 | 19080 | 19080 | 19080 | 19080 |
| 11 | 20616 | 20616 | 20616 | 21384 | 21384 | 21384 | 21384 | 22152 | 22152 | 22152 |
| 12 | 22920 | 23688 | 23688 | 23688 | 23688 | 24496 | 24496 | 24496 | 24496 | 25456 |
| 13 | 26416 | 26416 | 26416 | 26416 | 27376 | 27376 | 27376 | 27376 | 28336 | 28336 |
| 14 | 29296 | 29296 | 29296 | 29296 | 30576 | 30576 | 30576 | 30576 | 31704 | 31704 |
| 15 | 30576 | 31704 | 31704 | 31704 | 31704 | 32856 | 32856 | 32856 | 34008 | 34008 |
| 16 | 32856 | 32856 | 34008 | 34008 | 34008 | 34008 | 35160 | 35160 | 35160 | 35160 |
| 17 | 36696 | 36696 | 36696 | 37888 | 37888 | 37888 | 39232 | 39232 | 39232 | 39232 |
| 18 | 40576 | 40576 | 40576 | 40576 | 42368 | 42368 | 42368 | 42368 | 43816 | 43816 |
| 19 | 43816 | 43816 | 43816 | 45352 | 45352 | 45352 | 46888 | 46888 | 46888 | 46888 |
| 20 | 46888 | 46888 | 48936 | 48936 | 48936 | 48936 | 48936 | 51024 | 51024 | 51024 |
| 21 | 51024 | 51024 | 51024 | 52752 | 52752 | 52752 | 52752 | 55056 | 55056 | 55056 |
| 22 | 55056 | 55056 | 55056 | 57336 | 57336 | 57336 | 57336 | 59256 | 59256 | 59256 |
| 23 | 57336 | 59256 | 59256 | 59256 | 59256 | 61664 | 61664 | 61664 | 61664 | 63776 |
| 24 | 61664 | 61664 | 63776 | 63776 | 63776 | 63776 | 66592 | 66592 | 66592 | 66592 |
| 25 | 63776 | 63776 | 66592 | 66592 | 66592 | 66592 | 68808 | 68808 | 68808 | 71112 |
| 26 | 75376 | 75376 | 75376 | 75376 | 75376 | 75376 | 75376 | 75376 | 75376 | 75376 |

7.1.7.2.2	Transport blocks mapped to two-layer spatial multiplexing

For, the TBS is given by the (,) entry of Table 7.1.7.2.1-1.

For, a baseline TBS_L1 is taken from the (,) entry of Table 7.1.7.2.1-1, which is then translated into TBS_L2 using the mapping rule shown in Table 7.1.7.2.2-1. The TBS is given by TBS_L2.

Table 7.1.7.2.2-1: One-layer to two-layer TBS translation table

| TBS_L1 | TBS_L2 | TBS_L1 | TBS_L2 | TBS_L1 | TBS_L2 | TBS_L1 | TBS_L2 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1544 | 3112 | 3752 | 7480 | 10296 | 20616 | 28336 | 57336 |
| 1608 | 3240 | 3880 | 7736 | 10680 | 21384 | 29296 | 59256 |
| 1672 | 3368 | 4008 | 7992 | 11064 | 22152 | 30576 | 61664 |
| 1736 | 3496 | 4136 | 8248 | 11448 | 22920 | 31704 | 63776 |
| 1800 | 3624 | 4264 | 8504 | 11832 | 23688 | 32856 | 66592 |
| 1864 | 3752 | 4392 | 8760 | 12216 | 24496 | 34008 | 68808 |
| 1928 | 3880 | 4584 | 9144 | 12576 | 25456 | 35160 | 71112 |
| 1992 | 4008 | 4776 | 9528 | 12960 | 25456 | 36696 | 73712 |
| 2024 | 4008 | 4968 | 9912 | 13536 | 27376 | 37888 | 76208 |
| 2088 | 4136 | 5160 | 10296 | 14112 | 28336 | 39232 | 78704 |
| 2152 | 4264 | 5352 | 10680 | 14688 | 29296 | 40576 | 81176 |
| 2216 | 4392 | 5544 | 11064 | 15264 | 30576 | 42368 | 84760 |
| 2280 | 4584 | 5736 | 11448 | 15840 | 31704 | 43816 | 87936 |
| 2344 | 4776 | 5992 | 11832 | 16416 | 32856 | 45352 | 90816 |
| 2408 | 4776 | 6200 | 12576 | 16992 | 34008 | 46888 | 93800 |
| 2472 | 4968 | 6456 | 12960 | 17568 | 35160 | 48936 | 97896 |
| 2536 | 5160 | 6712 | 13536 | 18336 | 36696 | 51024 | 101840 |
| 2600 | 5160 | 6968 | 14112 | 19080 | 37888 | 52752 | 105528 |
| 2664 | 5352 | 7224 | 14688 | 19848 | 39232 | 55056 | 110136 |
| 2728 | 5544 | 7480 | 14688 | 20616 | 40576 | 57336 | 115040 |
| 2792 | 5544 | 7736 | 15264 | 21384 | 42368 | 59256 | 119816 |
| 2856 | 5736 | 7992 | 15840 | 22152 | 43816 | 61664 | 124464 |
| 2984 | 5992 | 8248 | 16416 | 22920 | 45352 | 63776 | 128496 |
| 3112 | 6200 | 8504 | 16992 | 23688 | 46888 | 66592 | 133208 |
| 3240 | 6456 | 8760 | 17568 | 24496 | 48936 | 68808 | 137792 |
| 3368 | 6712 | 9144 | 18336 | 25456 | 51024 | 71112 | 142248 |
| 3496 | 6968 | 9528 | 19080 | 26416 | 52752 | 73712 | 146856 |
| 3624 | 7224 | 9912 | 19848 | 27376 | 55056 | 75376 | 149776 |

7.1.7.2.3	Transport blocks mapped for DCI Format 1C

The TBS is given by theentry of Table 7.1.7.2.3-1.

Table 7.1.7.2.3-1: Transport Block Size Table for DCI format 1C

|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| TBS | 40 | 56 | 72 | 120 | 136 | 144 | 176 | 208 | 224 | 256 | 280 | 296 | 328 | 336 | 392 | 488 |
|  | 16 | 17 | 18 | 19 | 20 | 21 | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 | 30 | 31 |
| TBS | 552 | 600 | 632 | 696 | 776 | 840 | 904 | 1000 | 1064 | 1128 | 1224 | 1288 | 1384 | 1480 | 1608 | 1736 |

7.1.7.3	Redundancy Version determination for Format 1C

If the DCI Format 1C CRC is scrambled by P-RNTI or RA-RNTI, then

- the UE shall set the Redundancy  Version to 0

Else if the DCI Format 1C CRC is scrambled by SI-RNTI, then

the UE shall set the Redundancy  Version as defined in [8].

理解：Redundancy  Version在321协议的5.3..1最后面，SIB1和SI的冗余版本号是不一样的，但是计算下来冗余版本号都是0,2,3,1

7.2	UE procedure for reporting channel quality indication (CQI), precoding matrix indicator (PMI) and rank indication (RI)

The time and frequency resources that can be used by the UE to report CQI, PMI, and RI are controlled by the eNB. For spatial multiplexing, as given in [3], the UE shall determine a RI corresponding to the number of useful transmission layers. For transmit diversity as given in [3], RI is equal to one.

A UE in transmission mode 8 is configured with PMI/RI reporting if the parameter pmi-RI-Report is configured by higher layer signalling; otherwise, it is configured without PMI/RI reporting.

理解：在TM8模式下，并且高层使能了pmi-RI-Report，则为PMI/RI上报，其他的情况为非PMI/RI上报。在PMI/RI上报模式下，UE是根据CSI-RS来计算CQI的。

CQI, PMI, and RI reporting is periodic or aperiodic.

A UE shall transmit periodic CQI/PMI, or RI reporting on PUCCH as defined hereafter in subframes with no PUSCH allocation. A UE shall transmit periodic CQI/PMI or RI reporting on PUSCH as defined hereafter in subframes with PUSCH allocation, where the UE shall use the same PUCCH-based periodic CQI/PMI or RI reporting format on PUSCH.

A UE shall transmit aperiodic CQI/PMI, and RI reporting on PUSCH if the conditions specified hereafter are met. For aperiodic CQI reporting, RI reporting is transmitted only if configured CQI/PMI/RI feedback type supports RI reporting.

The CQI transmissions on PUCCH and PUSCH for various scheduling modes are summarized in the following table:

Table 7.2-1: Physical Channels for Aperiodic or Periodic CQI reporting

| Scheduling Mode | Periodic CQI reporting channels | Aperiodic CQI reporting channel |
| --- | --- | --- |
| Frequency non-selective | PUCCH |  |
| Frequency selective | PUCCH | PUSCH |

In case both periodic and aperiodic reporting would occur in the same subframe, the UE shall only transmit the aperiodic report in that subframe.

When reporting RI the UE reports a single instance of the number of useful transmission layers. For each RI reporting interval when the UE is configured in transmission mode 4 or when the UE is configured in transmission mode 8 with PMI/RI reporting, a UE shall determine a RI from the supported set of RI values for the corresponding eNodeB antenna configuration and UE category and report the number in each RI report. For each RI reporting interval when the UE is configured in transmission mode 3, a UE shall determine RI for the corresponding eNodeB antenna configuration and UE category in each reporting interval and report the detected number in each RI report to support selection between transmit diversity and large delay CDD.

When reporting PMI the UE reports either a single or a multiple PMI report.  The number of RBs represented by a single UE PMI report can be  or a smaller subset of RBs. The number of RBs represented by a single PMI report is semi-statically configured by higher layer signalling. A UE is restricted to report PMI and RI within a precoder codebook subset specified by a bitmap parameter codebookSubsetRestriction configured by higher layer signalling. For a specific precoder codebook and associated transmission mode, the bitmap can specify all possible precoder codebook subsets from which the UE can assume the eNB may be using when the UE is configured in the relevant transmission mode. Codebook subset restriction is supported for transmission modes 3, 4, 5, 6 and for transmission mode 8 if the UE is configured with PMI/RI reporting. The resulting number of bits for each transmission mode is given in Table 7.2-1b. The bitmap forms the bit sequence where  is the LSB and is the MSB and where a bit value of zero indicates that the PMI and RI reporting is not allowed to correspond to precoder(s) associated with the bit. The association of bits to precoders for the relevant transmission modes are given as follows:

- Transmission mode 3

  - 2 antenna ports: bit is associated with the precoder in Table 6.3.4.2.3-1 of [3] corresponding to  layers and codebook index 0 while bit  is associated with the precoder for 2 antenna ports in Section 6.3.4.3 of [3].

理解：a1代表2层，使用码本0，也就是码本

a0代表1层，码本0，也就是发射分集。

  - 4 antenna ports: bit is associated with the precoders in Table 6.3.4.2.3-2 of [3] corresponding to  layers and codebook indices 12, 13, 14, and 15 while bit  is associated with the precoder for 4 antenna ports in Section 6.3.4.3 of [3].

- Transmission mode 4

  - 2 antenna ports: see Table 7.2-1c

理解：6个Bit对应6种，去掉TM3中使用的码本

  - 4 antenna ports: bit  is associated with the precoder for  layers and with codebook index in Table 6.3.4.2.3-2 of [3].

理解：UE被限制使用码本子集来上报PMI和RI，限制的码本子集通过高层参数AntennaInfoDedicated-> codebookSubsetRestriction来配置。列举了不同TM模式、不同发射天线的的限制集码本的BitMap

- Transmission modes 5 and 6

  - 2 antenna ports: bit  is associated with the precoder for  layer with codebook index in Table 6.3.4.2.3-1 of [3].

  - 4 antenna ports: bit  is associated with the precoder for  layer with codebook index in Table 6.3.4.2.3-2 of [3].

- Transmission mode 8

  - 2 antenna ports: see Table 7.2-1c

  - 4 antenna ports: bit  is associated with the precoder for  layers and with codebook index in Table 6.3.4.2.3-2 of [3], .

Table 7.2-1b: Number of bits in codebook subset restriction bitmap for applicable transmission modes.

|  | Number of bits |  |
| --- | --- | --- |
|  | 2 antenna ports | 4 antenna ports |
| Transmission mode 3 | 2 | 4 |
| Transmission mode 4 | 6 | 64 |
| Transmission mode 5 | 4 | 16 |
| Transmission mode 6 | 4 | 16 |
| Transmission mode 8 | 6 | 32 |

Table 7.2-1c: Association of bits in codebookSubSetRestriction bitmap to precoders in the 2 antenna port codebook of Table 6.3.4.2.3-1 in [3].

| Codebook index | Number of layers |  |
| --- | --- | --- |
|  | 1 | 2 |
| 0 | a0 | - |
| 1 | a1 | a4 |
| 2 | a2 | a5 |
| 3 | a3 | - |

The set of subbands (S) a UE shall evaluate for CQI reporting spans the entire downlink system bandwidth.  A subband is a set of k contiguous PRBs where k is a function of system bandwidth.  Note the last subband in set S may have fewer than k contiguous PRBs depending on .  The number of subbands for system bandwidth given by is defined by.  The subbands shall be indexed in the order of increasing frequency and non-increasing sizes starting at the lowest frequency.

- For transmission modes 1, 2, 3 and 5, as well as transmission mode 8 if the UE is configured without PMI/RI reporting, transmission mode 4 with RI=1, and transmission mode 8 with PMI/RI reporting and RI=1, a single 4-bit wideband CQI is reported according to Table 7.2.3-1.

- For transmission modes 3 and 4, as well as transmission mode 8 if the UE is configured with PMI/RI reporting, CQI is calculated assuming transmission of one codeword for RI=1 and two codewords for RI > 1.

- For RI > 1 with transmission mode 4 and with transmission mode 8 if the UE is configured with PMI/RI reporting, PUSCH based triggered reporting includes reporting a wideband CQI which comprises:

  - A 4-bit wideband CQI for codeword 0 according to Table 7.2.3-1

  - A 4-bit wideband CQI for codeword 1 according to Table 7.2.3-1

- For RI > 1 with transmission mode 4 and with transmission mode 8 if the UE is configured with PMI/RI reporting, PUCCH based reporting includes reporting a 4-bit wideband CQI for codeword 0 according to Table 7.2.3-1 and a wideband spatial differential CQI. The wideband spatial differential CQI value comprises:

  - A 3-bit wideband spatial differential CQI value for codeword 1 offset level

    - Codeword 1 offset level = wideband CQI index for codeword 0 – wideband CQI index for codeword 1.

  - The mapping from the 3-bit wideband spatial differential CQI value to the offset level is shown in Table 7.2-2.

Table 7.2-2 Mapping spatial differential CQI value to offset level

| Spatial differential CQI value | Offset level |
| --- | --- |
| 0 | 0 |
| 1 | 1 |
| 2 | 2 |
| 3 | 3 |
| 4 | -4 |
| 5 | -3 |
| 6 | -2 |
| 7 | -1 |

7.2.1	Aperiodic CQI/PMI/RI Reporting using PUSCH

A UE shall perform aperiodic CQI, PMI and RI reporting using the PUSCH in subframe n+k , upon receiving in subframe n either:

- a DCI format 0, or

- a Random Access Response Grant,

if the respective CQI request field is set to 1 and is not reserved.

When the CQI request field from a DCI format 0 is set to 1, for FDD k=4, and for TDD UL/DL configuration 1-6, k is given in Table 8-2. For TDD UL/DL configuration 0, if the MSB of the UL index is set to 1 and LSB of the UL index is set to 0, k is given in Table 8-2; or if MSB of the UL index is set to 0 and LSB of the UL index is set to 1, k is equal to 7; or if both MSB and LSB of the UL index is set to 1, k is given in Table 8-2.

When the CQI request field from a Random Access Response Grant is set to 1 and is not reserved, k is equal to  if the UL delay field in section 6.2 is set to zero, where  is given in section 6.1.1. The UE shall postpone aperiodic CQI, PMI and RI reporting to the next available UL subframe if the UL delay field is set to 1.

The minimum reporting interval for aperiodic reporting of CQI and PMI and RI is 1 subframe. The subband size for CQI shall be the same for transmitter-receiver configurations with and without precoding.

When aperiodic CQI/PMI/RI report with no transport block associated as defined in section 8.6.2 and positive SR is transmitted in the same subframe, the UE shall transmit SR, and, if applicable, ACK/NAK, on PUCCH resources as described in Section 10.1

A UE is semi-statically configured by higher layers to feed back CQI and PMI and corresponding RI on the same PUSCH using one of the following reporting modes given in Table 7.2.1-1 and described below.

Table 7.2.1-1: CQI and PMI Feedback Types for PUSCH reporting Modes

|  |  | PMI Feedback Type |  |  |
| --- | --- | --- | --- | --- |
|  |  | No PMI | Single PMI | Multiple PMI |
| PUSCH CQI Feedback Type |  |  |  |  |
|  | Wideband |  |  | Mode 1-2 |
|  | (wideband CQI) |  |  |  |
|  | UE Selected | Mode 2-0 |  | Mode 2-2 |
|  | (subband CQI) |  |  |  |
|  | Higher Layer-configured | Mode 3-0 | Mode 3-1 |  |
|  | (subband CQI) |  |  |  |

For each of the transmission modes defined in Section 7.1, the following reporting modes are supported on PUSCH:

Transmission mode 1	: Modes 2-0, 3-0

Transmission mode 2	: Modes 2-0, 3-0

Transmission mode 3	: Modes 2-0, 3-0

Transmission mode 4	: Modes 1-2, 2-2, 3-1

Transmission mode 5	: Mode 3-1

Transmission mode 6	: Modes 1-2, 2-2, 3-1

Transmission mode 7	: Modes 2-0, 3-0

Transmission mode 8	: Modes 1-2, 2-2, 3-1 if the UE is configured with PMI/RI reporting; modes 2-0, 3-0 if the UE is configured without PMI/RI reporting

The aperiodic CQI reporting mode is given by the parameter cqi-ReportModeAperiodic which is configured by higher-layer signalling.

For , PUSCH reporting modes are not supported.

RI is only reported for transmission mode 3, transmission mode 4, and transmission mode 8 if the UE is configured with PMI/RI reporting.

A RI report on an aperiodic reporting mode is valid only for CQI/PMI report on that aperiodic reporting mode

- Wideband feedback

  - Mode 1-2 description:

    - For each subband a preferred precoding matrix is selected from the codebook subset assuming transmission only in the subband

    - A UE shall report one wideband CQI value per codeword which is calculated assuming the use of the corresponding selected precoding matrix in each subband and transmission on set S subbands.

    - The UE shall report the selected precoding matrix indicator for each set S subband.

    - Subband size is given by Table 7.2.1-3.

    - For transmission mode 4 and transmission mode 8, the reported PMI and CQI values are calculated conditioned on the reported RI. For other transmission modes they are reported conditioned on rank 1.

- Higher Layer-configured subband feedback

  - Mode 3-0 description:

    - A UE shall report a wideband CQI value which is calculated assuming transmission on set S subbands

    - The UE shall also report one subband CQI value for each set S subband. The subband CQI value is calculated assuming transmission only in the subband

    - Both the wideband and subband CQI represent channel quality for the first codeword, even when RI>1.

    - For transmission mode 3 the reported CQI values are calculated conditioned on the reported RI. For other transmission modes they are reported conditioned on rank 1.

  - Mode 3-1 description:

    - A single precoding matrix is selected from the codebook subset assuming transmission on set S subbands

    - A UE shall report one subband CQI value per codeword for each set S subband which are calculated assuming the use of the single precoding matrix in all subbands and assuming transmission in the corresponding subband.

    - A UE shall report a wideband CQI value per codeword which is calculated assuming the use of the single precoding matrix in all subbands and transmission on set S subbands

    - The UE shall report the single selected precoding matrix indicator

    - For transmission mode 4 and transmission mode 8, the reported PMI and CQI values are calculated conditioned on the reported RI. For other transmission modes they are reported conditioned on rank 1.

  - Subband CQI value for each codeword are encoded differentially with respect to their respective wideband CQI using 2-bits as defined by

    - Subband differential CQI offset level = subband CQI index – wideband CQI index. The mapping from the 2-bit subband differential CQI value to the offset level is shown in Table 7.2.1-2.

Table 7.2.1-2:  Mapping subband differential CQI value to offset level

| Subband differential CQI value | Offset level |
| --- | --- |
| 0 | 0 |
| 1 | 1 |
| 2 | 2 |
| 3 | -1 |

  - Supported subband size (k) is given in Table 7.2.1-3.

Table 7.2.1-3: Subband Size (k) vs. System Bandwidth

| System Bandwidth | Subband Size |
| --- | --- |
|  | (k) |
| 6 - 7 | NA |
| 8 - 10 | 4 |
| 11 - 26 | 4 |
| 27 - 63 | 6 |
| 64 - 110 | 8 |

- UE-selected subband feedback

  - Mode 2-0 description:

    - The UE shall select a set of M preferred subbands of size k (where k and M are given in Table 7.2.1-5 for each system bandwidth range) within the set of subbands S.

    - The UE shall also report one CQI value reflecting transmission only over the M selected subbands determined in the previous step. The CQI represents channel quality for the first codeword, even when RI>1.

    - Additionally, the UE shall also report one wideband CQI value which is calculated assuming transmission on set S subbands. The wideband CQI represents channel quality for the first codeword, even when RI>1.

    - For transmission mode 3 the reported CQI values are calculated conditioned on the reported RI. For other transmission modes they are reported conditioned on rank 1.

  - Mode 2-2 description:

    - The UE shall perform joint selection of the set of M preferred subbands of size k within the set of subbands S and a preferred single precoding matrix selected from the codebook subset that is preferred to be used for transmission over the M selected subbands.

    - The UE shall report one CQI value per codeword reflecting transmission only over the selected M preferred subbands and using the same selected single precoding matrix in each of the M subbands.

    - The UE shall also report the selected single precoding matrix indicator preferred for the M selected subbands.

    - A single precoding matrix is selected from the codebook subset assuming transmission on set S subbands

    - A UE shall report a wideband CQI value per codeword which is calculated assuming the use of the single precoding matrix in all subbands and transmission on set S subbands

    - A UE shall also report the selected single precoding matrix indicator for all set S subbands.

    - For transmission mode 4 and transmission mode 8, the reported PMI and CQI values are calculated conditioned on the reported RI. For other transmission modes they are reported conditioned on rank 1.

  - For all UE-selected subband feedback modes the UE shall report the positions of the M selected subbands using a combinatorial index r defined as

    - where the set , () contains the M sorted subband indices and  is the extended binomial coefficient, resulting in unique label .

  - The CQI value for the M selected subbands for each codeword is encoded differentially using 2-bits relative to its respective wideband CQI as defined by

    - Differential CQI offset level = M selected subbands CQI index – wideband CQI index

    - The mapping from the 2-bit differential CQI value to the offset level is shown in Table 7.2.1-4.

Table 7.2.1-4:  Mapping differential CQI value to offset level

| Differential CQI value | Offset level |
| --- | --- |
| 0 | 1 |
| 1 | 2 |
| 2 | 3 |
| 3 | 4 |

  - Supported subband size k and M values include those shown in Table 7.2.1-5. In Table 7.2.1-5 the k and M values are a function of system bandwidth.

  - The number of bits to denote the position of the M selected subbands is .

Table 7.2.1-5: Subband Size (k) and Number of Subbands (M) in S vs. Downlink System Bandwidth

| System Bandwidth | Subband Size k (RBs) | M |
| --- | --- | --- |
| 6 – 7 | NA | NA |
| 8 – 10 | 2 | 1 |
| 11 – 26 | 2 | 3 |
| 27 – 63 | 3 | 5 |
| 64 – 110 | 4 | 6 |

7.2.2	Periodic CQI/PMI/RI Reporting using PUCCH

A UE is semi-statically configured by higher layers to periodically feed back different CQI, PMI, and RI on the PUCCH using the reporting modes given in Table 7.2.2-1 and described below.

Table 7.2.2-1: CQI and PMI Feedback Types for PUCCH reporting Modes

|  |  | PMI Feedback Type |  |
| --- | --- | --- | --- |
|  |  | No PMI | Single PMI |
| PUCCH CQI Feedback Type |  |  |  |
|  | Wideband | Mode 1-0 | Mode 1-1 |
|  | (wideband CQI) |  |  |
|  | UE Selected | Mode 2-0 | Mode 2-1 |
|  | (subband CQI) |  |  |

For each of the transmission modes defined in Section 7.1, the following reporting modes are supported on PUCCH:

Transmission mode 1	: Modes 1-0, 2-0

Transmission mode 2	: Modes 1-0, 2-0

Transmission mode 3	: Modes 1-0, 2-0

Transmission mode 4	: Modes 1-1, 2-1

Transmission mode 5	: Modes 1-1, 2-1

Transmission mode 6	: Modes 1-1, 2-1

Transmission mode 7	: Modes 1-0, 2-0

Transmission mode 8	: Modes 1-1, 2-1 if the UE is configured with PMI/RI reporting; modes 1-0, 2-0 if the UE is configured without PMI/RI reporting

The periodic CQI reporting mode is given by the parameter cqi-FormatIndicatorPeriodic which is configured by higher-layer signalling.

For the UE-selected subband CQI, a CQI report in a certain subframe describes the channel quality in a particular part or in particular parts of the bandwidth described subsequently as bandwidth part (BP) or parts. The bandwidth parts shall be indexed in the order of increasing frequency and non-increasing sizes starting at the lowest frequency.

- There are a total of N subbands for a system bandwidth given by wheresubbands are of size k. If  then one of the subbands is of size.

- A bandwidth part j is frequency-consecutive and consists of subbands where J bandwidth parts span S or  as given in Table 7.2.2-2. If  then  is. If J>1 then  is either  or , depending on, k and J.

- Each bandwidth part j, where 0 ≤ j ≤ J-1, is scanned in sequential order according to increasing frequency.

- For UE selected subband feedback a single subband out of  subbands of a bandwidth part is selected along with a corresponding L-bit label indexed in the order of increasing frequency, where.

The CQI and PMI payload sizes of each PUCCH reporting mode are given in Table 7.2.2-3.

Four CQI/PMI and RI reporting types with distinct periods and offsets are supported for each PUCCH reporting mode as given in Table 7.2.2-3:

- Type 1 report supports CQI feedback for the UE selected sub-bands

- Type 2 report supports wideband CQI and PMI feedback.

- Type 3 report supports RI feedback

- Type 4 report supports wideband CQI

The periodicity  (in subframes) and offset  (in subframes) for CQI/PMI reporting are determined based on the parameter cqi-pmi-ConfigIndex () given in Table 7.2.2-1A for FDD and table 7.2.2-1C for TDD. The periodicity  and relative offset   for RI reporting are determined based on the parameter ri-ConfigIndex () given in Table 7.2.2-1B. Both cqi-pmi-ConfigIndex and ri-ConfigIndex are configured by higher layer signalling.  The relative reporting offset for RI  takes values from the set

In the case where wideband CQI/PMI reporting is configured:

- The reporting instances for wideband CQI/PMI are subframes satisfying .

- In case RI reporting is configured, the reporting interval of the RI reporting is an integer multiple  of period  (in subframes).

  - The reporting instances for RI are subframes satisfying .

  - In case of collision of RI and wideband CQI/PMI the wideband CQI/PMI is dropped.

In the case where both wideband CQI/PMI and subband CQI reporting are configured:

  - The reporting instances for wideband CQI/PMI and subband CQI are subframes satisfying .

    - The wideband CQI/PMI report has period , and is reported on the subframes satisfying  . The integer  is defined as , where  is the number of bandwidth parts.

    - Between every two consecutive wideband CQI/PMI reports, the remaining  reporting instances are used in sequence for subband CQI reports on  full cycles of bandwidth parts except when the gap between two consecutive wideband CQI/PMI reports contains less than  reporting instances due to a system frame number transition to 0, in which case the UE shall not transmit the remainder of the subband CQI reports which have not been transmitted before the second of the two wideband CQI/PMI reports. Each full cycle of bandwidth parts shall be in increasing order starting from bandwidth part 0 to bandwidth part . The parameter is configured by higher-layer signalling.

理解：由高层配置CQI-ReportConfig-> cqi-ReportPeriodic->k来确定。此和小k表示的含义不一样，小k代表子带的Rb个数。

  - In case RI reporting is configured, the reporting interval of RI is  times the wideband CQI/PMI period , and RI is reported on the same PUCCH cyclic shift resource as both the wideband CQI/PMI and subband CQI reports.

    - The reporting instances for RI are subframes satisfying .In case of collision between RI and wideband CQI/PMI or subband CQI, the wideband CQI/PMI or subband CQI is dropped.

See section 10.1 regarding UE behaviour for collision between CQI/PMI/RI and ACK/NACK and the corresponding PUCCH format assignment.

The CQI/PMI or RI report shall be transmitted on the PUCCH resource  as defined in [3], where  is UE specific and configured by higher layers.

In case of collision between CQI/PMI/RI and positive SR in a same subframe, CQI/PMI/RI is dropped.

Table 7.2.2-1A: Mapping of  to  and  for FDD.

|  | Value of | Value of |
| --- | --- | --- |
| 0 1 | 2 |  |
| 2 6 | 5 | – 2 |
| 7 16 | 10 | – 7 |
| 17 36 | 20 | – 17 |
| 37 76 | 40 | – 37 |
| 77 156 | 80 | – 77 |
| 157 316 | 160 | – 157 |
| = 317 | Reserved |  |
| 318 349 | 32 | – 318 |
| 350 413 | 64 | – 350 |
| 414 541 | 128 | – 414 |
| 542 1023 | Reserved |  |

Table 7.2.2-1B: Mapping of  to  and .

|  | Value of | Value of |
| --- | --- | --- |
| 0 160 | 1 | − |
| 161 321 | 2 | − ( – 161) |
| 322 482 | 4 | − ( – 322) |
| 483 643 | 8 | − ( – 483) |
| 644 804 | 16 | − ( – 644) |
| 805 965 | 32 | − ( – 805) |
| 966 1023 | Reserved |  |

Table 7.2.2-1C: Mapping of  to  and  for TDD.

|  | Value of | Value of |
| --- | --- | --- |
| = 0 | 1 |  |
| 1 5 | 5 | – 1 |
| 6 15 | 10 | – 6 |
| 16 35 | 20 | – 16 |
| 36 75 | 40 | – 36 |
| 76 155 | 80 | – 76 |
| 156 315 | 160 | – 156 |
| 316 1023 | Reserved |  |

For TDD periodic CQI/PMI reporting, the following periodicity values apply depending on the TDD UL/DL configuration [3]:

  - The reporting period of  is only applicable to TDD UL/DL configurations 0, 1, 3, 4, and 6, where all UL subframes in a radio frame are used for CQI/PMI reporting.

  - The reporting period of  is only applicable to TDD UL/DL configurations 0, 1, 2, and 6.

  - The reporting periods of  are applicable to all TDD UL/DL configurations.

For , Mode 2-0 and Mode 2-1 are not supported.

A RI report in a periodic reporting mode is valid only for CQI/PMI report on that periodic reporting mode.

For the calculation of CQI/PMI conditioned on the last reported RI, in the absence of a last reported RI the UE shall conduct the CQI/PMI calculation conditioned on the lowest possible RI as given by the bitmap parameter codebookSubsetRestriction .

- Wideband feedback

  - Mode 1-0 description:

    - In the subframe where RI is reported (only for transmission mode 3):

      - A UE shall determine a RI assuming transmission on set S subbands.

      - The UE shall report a type 3 report consisting of one RI.

    - In the subframe where CQI is reported:

      - A UE shall report a type 4 report consisting of one wideband CQI value which is calculated assuming transmission on set S subbands. The wideband CQI represents channel quality for the first codeword, even when RI>1.

      - For transmission mode 3 the CQI is calculated conditioned on the last reported periodic RI. For other transmission modes it is calculated conditioned on transmission rank 1

  - Mode 1-1 description:

    - In the subframe where RI is reported (only for transmission mode 4 and transmission mode 8):

      - A UE shall determine a RI assuming transmission on set S subbands.

      - The UE shall report a type 3 report consisting of one RI

    - In the subframe where CQI/PMI is reported:

      - A single precoding matrix is selected from the codebook subset assuming transmission on set S subbands

      - A UE shall report a type 2 report on each respective successive reporting opportunity consisting of

        - A single wideband CQI value which is calculated assuming the use of a single precoding matrix in all subbands and transmission on set S subbands.

        - The selected single precoding matrix indicator (wideband PMI)

        - When RI>1, a 3-bit wideband spatial differential CQI, which is shown in Table 7.2-2.

      - For transmission mode 4 and transmission mode 8, the PMI and CQI are calculated conditioned on the last reported periodic RI. For other transmission modes they are calculated conditioned on transmission rank 1.

- UE Selected subband feedback

  - Mode 2-0 description:

    - In the subframe where RI is reported (only for transmission mode 3):

      - A UE shall determine a RI assuming transmission on set S subbands.

      - The UE shall report a type 3 report consisting of one RI.

    - In the subframe where wideband CQI is reported:

      - The UE shall report a type 4 report on each respective successive reporting opportunity consisting of one wideband CQI value which is calculated assuming transmission on set S subbands. The wideband CQI represents channel quality for the first codeword, even when RI>1.

      - For transmission mode 3 the CQI is calculated conditioned on the last reported periodic RI. For other transmission modes it is calculated conditioned on transmission rank 1.

    - In the subframe where CQI for the selected subbands is reported:

      - The UE shall select the preferred subband within the set of  subbands in each of the J bandwidth parts where J is given in Table 7.2.2-2.

      - The UE shall report a type 1 report consisting of one CQI value reflecting transmission only over the selected subband of a bandwidth part determined in the previous step along with the corresponding preferred subband L-bit label. A type 1 report for each bandwidth part will in turn be reported in respective successive reporting opportunities. The CQI represents channel quality for the first codeword, even when RI>1.

      - For transmission mode 3 the preferred subband selection and CQI values are calculated conditioned on the last reported periodic RI. For other transmission modes they are calculated conditioned on transmission rank 1.

  - Mode 2-1 description:

    - In the subframe where RI is reported (only for transmission mode 4 and transmission mode 8):

      - A UE shall determine a RI assuming transmission on set S subbands.

      - The UE shall report a type 3 report consisting of one RI.

    - In the subframe where wideband CQI/PMI is reported:

      - A single precoding matrix is selected from the codebook subset assuming transmission on set S subbands.

      - A UE shall report a type 2 report on each respective successive reporting opportunity consisting of:

        - A wideband CQI value which is calculated assuming the use of a single precoding matrix in all subbands and transmission on set S subbands.

        - The selected single precoding matrix indicator (wideband PMI).

        - When RI>1, and additional 3-bit wideband spatial differential CQI, which is shown in Table 7.2-2.

      - For transmission mode 4 and transmission mode 8, the PMI and CQI values are calculated conditioned on the last reported periodic RI. For other transmission modes they are calculated conditioned on transmission rank 1.

    - In the subframe where CQI for the selected subbands is reported:

      - The UE shall select the preferred subband within the set of Nj subbands in each of the J bandwidth parts where J is given in Table 7.2.2-2.

      - The UE shall report a type 1 report per bandwidth part on each respective successive reporting opportunity consisting of:

        - CQI value for codeword 0 reflecting transmission only over the selected subband of a bandwidth part determined in the previous step along with the corresponding preferred subband L-bit label.

        - When RI>1, an additional 3-bit subband spatial differential CQI value for codeword 1 offset level

          - Codeword 1 offset level = subband CQI index for codeword 0 – subband CQI index for codeword 1.

          - Assuming the use of the most recently reported single precoding matrix in all subbands and transmission on set S subbands.

        - The mapping from the 3-bit subband spatial differential CQI value to the offset level is shown in Table 7.2-2.

      - For transmission mode 4 and transmission mode 8, the subband selection and CQI values are calculated conditioned on the last reported periodic wideband PMI and RI. For other transmission modes they are calculated conditioned on the last reported PMI and transmission rank 1.

Table 7.2.2-2: Subband Size (k) and Bandwidth Parts (J) vs. Downlink System Bandwidth

| System Bandwidth | Subband Size k (RBs) | Bandwidth Parts (J) |
| --- | --- | --- |
| 6 – 7 | NA | NA |
| 8 – 10 | 4 | 1 |
| 11 – 26 | 4 | 2 |
| 27 – 63 | 6 | 3 |
| 64 – 110 | 8 | 4 |

If parameter ttiBundling provided by higher layers is set to TRUE and if an UL-SCH in subframe bundling operation collides with a periodic CQI/PMI/RI reporting instance, then the UE shall drop the periodic CQI/PMI/RI report in that subframe and shall not multiplex periodic CQI/PMI and/or rank indicator in the PUSCH transmission in that subframe.

Table 7.2.2-3: PUCCH Report Type Payload size per Reporting Mode

| PUCCH Report Type | Reported | Mode State | PUCCH Reporting Modes |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Mode 1-1 | Mode 2-1 | Mode 1-0 | Mode 2-0 |
|  |  |  | (bits/BP) | (bits/BP) | (bits/BP) | (bits/BP) |
| 1 | Sub-band CQI | RI = 1 | NA | 4+L | NA | 4+L |
|  |  | RI > 1 | NA | 7+L | NA | 4+L |
| 2 | Wideband CQI/PMI | 2 TX Antennas RI = 1 | 6 | 6 | NA | NA |
|  |  | 4 TX Antennas RI = 1 | 8 | 8 | NA | NA |
|  |  | 2 TX Antennas RI > 1 | 8 | 8 | NA | NA |
|  |  | 4 TX Antennas RI > 1 | 11 | 11 | NA | NA |
| 3 | RI | 2-layer spatial multiplexing | 1 | 1 | 1 | 1 |
|  |  | 4-layer spatial multiplexing | 2 | 2 | 2 | 2 |
| 4 | Wideband CQI | RI = 1 or RI>1 | NA | NA | 4 | 4 |

7.2.3	Channel quality indicator (CQI) definition

The CQI indices and their interpretations are given in Table 7.2.3-1.

- 理解：带有CQI参考资源的下行PDSCH信号可以理解是PDSCH上有参考测量信道CSI-RS，用于终端计算CQI，终端还可以通过小区参考信号CRS进行测量CQI。一般情况：如果基站配置没有PMI/RI，终端基于CRS 来进行信道测量，如果基站配置有PMI/RI，则终端基于CSI-RS 来进行信道测量

Based on an unrestricted observation interval in time and frequency, the UE shall derive for each CQI value reported in uplink subframe n the highest CQI index between 1 and 15 in Table 7.2.3-1 which satisfies the following condition, or CQI index 0 if CQI index 1 does not satisfy the condition:

- A single PDSCH transport block with a combination of modulation scheme and transport block size corresponding to the CQI index, and occupying a group of downlink physical resource blocks termed the CQI reference resource, could be received with a transport block error probability not exceeding 0.1.

（根据反馈CQI来确定下行的调制方式和TB块大小，要求传输块的误码率不超过0.1）

A combination of modulation scheme and transport block size corresponds to a CQI index if:

- the combination could be signalled for transmission on the PDSCH in the CQI reference resource according to the relevant Transport Block Size table, and

- the modulation scheme is indicated by the CQI index, and

- the combination of transport block size and modulation scheme when applied to the reference resource results in the effective channel code rate which is the closest possible to the code rate indicated by the CQI index. If more than one combination of transport block size and modulation scheme results in a effective channel code rate equally close to the code rate indicated by the CQI index, only the combination with the smallest of such transport block sizes is relevant.

The CQI reference resource is defined as follows:

- In the frequency domain, the CQI reference resource is defined by the group of downlink physical resource blocks corresponding to the band to which the derived CQI value relates.

- 理解：CQI的参考资源在频域上是根据带宽定义的一组下行物理资源。在时域上是一个单独的下行帧，并且发射功率也是由要求的，的计算公式和普通的下行子帧的计算公式不一样。相当于带CSI-RS的PDSCH子帧的功率会比不带的功率大。由高层CQI-ReportConfig-> nomPDSCH-RS-EPRE-Offset来确定。

- In the time domain, the CQI reference resource is defined by a single downlink subframe n-nCQI_ref,

  - where for periodic CQI reporting nCQI_ref  is the smallest value greater than or equal to 4, such that it corresponds to a valid downlink subframe;

  - where for aperiodic CQI reporting nCQI_ref  is such that the reference resource is in the same valid downlink subframe as the corresponding CQI request in a DCI format 0.

  - where for aperiodic CQI reporting nCQI_ref  is equal to 4 and downlink subframe n-nCQI_ref corresponds to a valid downlink subframe, where downlink subframe n-nCQI_ref is received after the subframe with the corresponding CQI request in a Random Access Response Grant.

- A downlink subframe shall be considered to be valid if:

    - it is configured as a downlink subframe for that UE, and

    - it is not an MBSFN subframe, and

    - it does not contain a DwPTS field in case the length of DwPTS is  and less, and

    - it does not fall within a configured measurement gap for that UE.

- If there is no valid downlink subframe for the CQI reference resource, CQI reporting is omitted in uplink subframe n.

- In the layer domain, the CQI reference resource is defined by any RI and PMI on which the CQI is conditioned.

In the CQI reference resource, the UE shall assume the following for the purpose of deriving the CQI index:

- The first 3 OFDM symbols are occupied by control signalling

- No resource elements used by primary or secondary synchronisation signals or PBCH

- CP length of the non-MBSFN subframes

- Redundancy Version 0

- the PDSCH transmission scheme given by Table 7.2.3-0 depending on the transmission mode currently configured for the UE (which may be the default mode).

- The ratio of PDSCH EPRE to cell-specific RS EPRE is as given in Section 5.2 with the exception of which shall be assumed to be

  - [dB] for any modulation scheme, if the UE is configured with transmission mode 2 with 4 cell-specific antenna ports, or transmission mode 3 with 4 cell-specific antenna ports and the associated RI is equal to one;

  - [dB] for any modulation scheme and any number of layers, otherwise.

The shift is given by the parameter nomPDSCH-RS-EPRE-Offset which is configured by higher-layer signalling.

Table 7.2.3-0: PDSCH transmission scheme assumed for CQI reference resource

| Transmission mode | Transmission scheme of PDSCH |
| --- | --- |
| 1 | Single-antenna port, port 0 |
| 2 | Transmit diversity |
| 3 | Transmit diversity if the associated rank indicator is 1, otherwise large delay CDD |
| 4 | Closed-loop spatial multiplexing |
| 5 | Multi-user MIMO |
| 6 | Closed-loop spatial multiplexing with a single transmission layer |
| 7 | If the number of PBCH antenna ports is one, Single-antenna port, port 0; otherwise Transmit diversity |
| 8 | If the UE is configured without PMI/RI reporting: if the number of PBCH antenna ports is one, single-antenna port, port 0; otherwise transmit diversity If the UE is configured with PMI/RI reporting: closed-loop spatial multiplexing |

Table 7.2.3-1: 4-bit CQI Table

| CQI index | modulation | code rate x 1024 | efficiency |
| --- | --- | --- | --- |
| 0 | out of range |  |  |
| 1 | QPSK | 78 | 0.1523 |
| 2 | QPSK | 120 | 0.2344 |
| 3 | QPSK | 193 | 0.3770 |
| 4 | QPSK | 308 | 0.6016 |
| 5 | QPSK | 449 | 0.8770 |
| 6 | QPSK | 602 | 1.1758 |
| 7 | 16QAM | 378 | 1.4766 |
| 8 | 16QAM | 490 | 1.9141 |
| 9 | 16QAM | 616 | 2.4063 |
| 10 | 64QAM | 466 | 2.7305 |
| 11 | 64QAM | 567 | 3.3223 |
| 12 | 64QAM | 666 | 3.9023 |
| 13 | 64QAM | 772 | 4.5234 |
| 14 | 64QAM | 873 | 5.1152 |
| 15 | 64QAM | 948 | 5.5547 |

理解：此处的efficiency是指information bits per resource element，比如一个RE能承载的信息为5.5547，其余为Turbo冗余比特（使用64QAM，每个RE的信道比特或者说物理比特为6），这样就是efficency=5.5547。找一对最接近于选择的CQI index对应的code rate的调制方式和TBS。码率为code rate = efficiency /Qm

理解：CQI的值是手机上报的，具体如何上报CQI是手机厂商的具体算法，不同手机厂商不同的算法，一般是通过查表所得。不同的SINR下有不同的CQI表格，手机会根据BLER选择一个最大满足BLER《10%的CQI上报。而手机上报CQI会存在一定偏差，EnodeB侧有CQI  adapter的这个功能，可以对上报的CQI进行微调

| CQI索引 | 调制方式 | 编码速率*1024 | 频谱效率（bit/s/Hz） | 等效SNR阀值（BLER=10%） |
| --- | --- | --- | --- | --- |
| 1 | QPSK | 78 | 0.1523 | -6.71 |
| 2 | QPSK | 120 | 0.2344 | -5.11 |
| 3 | QPSK | 193 | 0.3770 | -3.15 |
| 4 | QPSK | 308 | 0.6016 | -0.879 |
| 5 | QPSK | 449 | 0.8770 | 0.701 |
| 6 | QPSK | 602 | 1.1758 | 2.529 |
| 7 | 16QAM | 378 | 1.4766 | 4.606 |
| 8 | 16QAM | 490 | 1.9141 | 6.431 |
| 9 | 16QAM | 616 | 2.4063 | 8.326 |
| 10 | 64QAM | 466 | 2.7305 | 10.3 |
| 11 | 64QAM | 567 | 3.3223 | 12.22 |
| 12 | 64QAM | 666 | 3.9023 | 14.01 |
| 13 | 64QAM | 772 | 4.5234 | 15.81 |
| 14 | 64QAM | 873 | 5.1152 | 17.68 |
| 15 | 64QAM | 948 | 5.5547 | 19.61 |

7.2.4	Precoding Matrix Indicator (PMI) definition

For transmission modes 4, 5, and 6, precoding feedback is used for channel dependent codebook based precoding and relies on UEs reporting precoding matrix indicator (PMI). For transmission mode 8, the UE shall report PMI if configured with PMI/RI reporting. A UE shall report PMI based on the feedback modes described in 7.2.1 and 7.2.2.  Each PMI value corresponds to a codebook index given in Table 6.3.4.2.3-1 or Table 6.3.4.2.3-2 of [3] as follows:

- For 2 antenna ports  and an associated RI value of 1, a PMI value of  corresponds to the codebook index  given in Table 6.3.4.2.3-1 of [3] with .

- For 2 antenna ports  and an associated RI value of 2, a PMI value of  corresponds to the codebook index given in Table 6.3.4.2.3-1 of [3] with .

- For 4 antenna ports , a PMI value of  corresponds to the codebook index  given in Table 6.3.4.2.3-2 of [3] with  equal to the associated RI value.

For other transmission modes, PMI reporting is not supported.

7.3	UE procedure for reporting ACK/NACK

For FDD, when both ACK/NACK and SR are transmitted in the same sub-frame a UE shall transmit the ACK/NACK on its assigned ACK/NACK PUCCH resource for a negative SR transmission and transmit the ACK/NACK on its assigned SR PUCCH resource for a positive SR transmission.

For TDD and all UL-DL configurations except configuration 5, two ACK/NACK feedback modes are supported by higher layer configuration.

- ACK/NACK bundling, and

- ACK/NACK multiplexing

For TDD UL-DL configuration 5, only ACK/NACK bundling is supported.

For TDD, the UE shall upon detection of a PDSCH transmission or a PDCCH indicating downlink SPS release (defined in section 9.2) within subframe(s) , where  and  is defined in Table 10.1-1 intended for the UE and for which ACK/NACK response shall be provided, transmit the ACK/NACK response in UL subframe n.

For TDD UL-DL configurations 1-6, the value of the Downlink Assignment Index (DAI) in DCI format 0, , detected by the UE according to Table 7.3-X in subframe , where  is defined in Table 7.3-Y, represents the total number of subframes with PDSCH transmissions and with PDCCH indicating downlink SPS release to the corresponding UE within all the subframe(s), where. The value  includes all PDSCH transmission with and without corresponding PDCCH within all the subframe(s) . In case neither PDSCH transmission, nor PDCCH indicating the downlink SPS resource release is intended to the UE, the UE can expect that the value of the DAI in DCI format 0, , if transmitted, is set to 4.

理解：假如一个上行子帧需要反馈4个下行子帧，则是在离上行子帧最近的下行子帧上发送DCI 0（或者比这个下行子帧更加近，例如子帧配比3的子帧2和子帧3、子帧配比4的子帧2，都是之前的第4帧）

在既没有PDCSH传输，也没有PDCCH指示SPS释放的情况下，则UE期待收到的DCI 0中的DAI为4，查表7.3-X可以知道可以表示0.

For TDD UL-DL configurations 1-6, the value of the DAI in DCI format 1/1A/1B/1D/2/2A/2B denotes the accumulative number of PDCCH(s) with assigned PDSCH transmission(s) and PDCCH indicating downlink SPS release up to the present subframe within subframe(s) , where , and shall be updated from subframe to subframe. Denote  as the value of the DAI in PDCCH with DCI format 1/1A/1B/1D/2/2A/2B detected by the UE according to Table 7.3-X in subframe, where  is the smallest value in the set  (defined in Table 10.1-1) such that the UE detects a DCI format 1/1A/1B/1D/2/2A/2B.

理解：是在下行子帧的DCI1/1A/1B/1D/2/2A/2B中指示，假如一个上行子帧需要反馈4个下行子帧，是在离上行子帧最近的那个下行子帧上指示。

For all TDD UL-DL configurations, denote  as the total number of PDCCH(s) with assigned PDSCH transmission(s) and PDCCH indicating downlink SPS release detected by the UE within the subframe(s) , where . Denote , which can be zero or one, as the number of PDSCH transmissions without a corresponding PDCCH within the subframe(s) , where .

理解：和的含义基本上相同，区别是是eNodeB告诉UE的，是UE接收时统计的，UE根据接收到的PDCCH进行统计。

理解：

：代表有PDCCH指派的PDSCH数量+没有PDCCH指派的PDSCH数量+下行SPS释放的PDCCH数量

：代表指派PDSCH的PDCCH的数量+下行SPS释放的的PDCCH数量

：代表指派PDSCH的PDCCH的数量加上指示下行SPS释放的的PDCCH数量，代表PDCCH总的数量。

U_dai = 含义同DL_DAI，但是为了区分同一个DL/UL_DAI值对应多种情况时需要引入，可见213的表7.3-X中的第三列，对应多种情况。

：代表没有PDCCH指示的PDSCH的数量，值为0或者1.

- 有上面的分析可以知道=+

- 在PUCCH上传输ACK时，使用公式来判决是否有下行指派丢失。

- 在PUSCH上传输ACK时，使用公式来判决是否有下行指派丢失

- R9版本中，在PUCCH上传输ACK时，最多使用2Bit，但是在PUSCH上传输ACK时，最多可以是4Bit，注意区别。

For TDD ACK/NACK bundling or ACK/NACK multiplexing and a subframe  with , the UE shall generate one or two ACK/NACK bits by performing a logical AND operation per codeword across  DL subframes associated with a single UL subframe, of all the corresponding  individual PDSCH transmission ACK/NACKs and individual ACK in response to received PDCCH indicating downlink SPS release, where  is the number of elements in the set  defined in Table 10.1-1. The UE shall detect if at least one downlink assignment has been missed, and for the case that the UE is transmitting on PUSCH the UE shall also determine the parameter. For TDD UL-DL configuration 0,  shall be 1 if UE detects the PDSCH transmission with or without corresponding PDCCH within the subframe , where .

- For the case that the UE is not transmitting on PUSCH in subframe n and TDD UL-DL configurations 1-6, if   and , the UE detects that at least one downlink assignment has been missed.

理解：如果ACK不是在PUSCH上反馈，说明没有上行调度，也就没有DCI 0，也就没有，只能根据来判决是否有下行指派丢掉。

- For the case that the UE is transmitting on PUSCH and the PUSCH transmission is adjusted based on a detected PDCCH with DCI format 0 intended for the UE and TDD UL-DL configurations 1-6, if  the UE detects that at least one downlink assignment has been missed and the UE shall generate NACK for all codewords where  is determined by the UE as . If the UE does not detect any downlink assignment missing,  is determined by the UE as . UE shall not transmit ACK/NACK if and .

理解：如果ACK是在PUSCH上传输， UE需要决定参数，如果没有发行下行指派丢失，。如果发现下行指派丢失，。在和的情况下UE不传输ACK\NACK.

- For the case that the UE is transmitting on PUSCH, and the PUSCH transmission is not based on a detected PDCCH with DCI format 0 intended for the UE and TDD UL-DL configurations 1-6, if   and , the UE detects that at least one downlink assignment has been missed and the UE shall generate NACK for all codewords. The UE determines  as the number of assigned subframes. The UE shall not transmit ACK/NACK if .

For TDD ACK/NACK bundling, when the UE is configured by transmission mode 3, 4 or 8 defined in Section 7.1 and ACK/NACK bits are transmitted on PUSCH, the UE shall always generate 2 ACK/NACK bits assuming both codeword 0 and 1 are enabled. For the case that the UE detects only the PDSCH transmission associated with codeword 0 within the bundled subframes, the UE shall generate NACK for codeword 1.

理解：在ACK\NACK bundling的情况下，TM3、TM4和TM8双码字的情况下，如果只有码字0上有数据，码字1上没有数据，UE将会产生2个Bit的ACK，码字1的NACK为1.

Table 7.3-X: Value of Downlink Assignment Index

| DAI MSB, LSB | or | Number of subframes with PDSCH transmission and with PDCCH indicating DL SPS release |
| --- | --- | --- |
| 0,0 | 1 | 1 or 5 or 9 |
| 0,1 | 2 | 2 or 6 |
| 1,0 | 3 | 3 or 7 |
| 1,1 | 4 | 0 or 4 or 8 |

Table 7.3-Y: Uplink association index k’ for TDD

| TDD UL/DL Configuration | DL subframe number n |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 |  | 4 | 5 | 6 |  | 7 |  | 8 |  | 9 |
| 1 |  |  | 6 | 4 |  |  |  |  |  | 6 |  | 4 |  |  |
| 2 |  |  | 4 |  |  |  |  |  |  | 4 |  |  |  |  |
| 3 |  |  | 4 | 4 |  | 4 |  |  |  |  |  |  |  |  |
| 4 |  |  | 4 | 4 |  |  |  |  |  |  |  |  |  |  |
| 5 |  |  | 4 |  |  |  |  |  |  |  |  |  |  |  |
| 6 |  |  | 7 | 7 |  | 5 |  |  |  | 7 |  | 7 |  |  |
| UL-DL Configuration |  |  |  | Subframe n |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 0 |  | 1 |  | 2 |  | 3 | 4 | 5 | 6 | 7 | 8 | 9 |
| 0 |  |  |  | - |  | - |  | 6 |  | - | 4 | - | - | 6 | - | 4 |
| 1 |  |  |  | - |  | - |  | 7, 6 |  | 4 | - | - | - | 7, 6 | 4 | - |
| 2 |  |  |  | - |  | - |  | 8, 7, 4, 6 |  | - | - | - | - | 8, 7, 4, 6 | - | - |
| 3 |  |  |  | - |  | - |  | 7, 6, 11 |  | 6, 5 | 5, 4 | - | - | - | - | - |
| 4 |  |  |  | - |  | - |  | 12, 8, 7, 11 |  | 6, 5, 4, 7 | - | - | - | - | - | - |
| 5 |  |  |  | - |  | - |  | 13, 12, 9, 8, 7, 5, 4, 11, 6 |  | - | - | - | - | - | - | - |
| 6 |  |  |  | - |  | - |  | 7 |  | 7 | 5 | - | - | 7 | 7 | - |

理解：

- 比较36.213中表10.1-1和表7.3-Y可以知道，DCI format 0发送所在的下行子帧（对应表7.3-Y）一定对应表10.1-1中HARQ反馈窗中最后的一个下行子帧（或者比该下行子帧还要靠后，例如子帧配比3的子帧2和子帧3、子帧配比4的子帧2，都是之前的第4帧）

- 指示UE在所有n-k子帧中，接收到没有PDCCH的PDSCH传输数量。由于SPS的周期最小为10ms，所以在一个ACK/NACK反馈窗中最多有一个SPS调度，所以的值只能为0或者1.

- 和的定义相同，但是含义不同。是eNodeB告诉UE的，是UE接收时统计的，UE根据接收到的PDCCH进行统计。

- 反映了UE在HARQ反馈窗口内实际在多少个子帧接收到了PDSCH和指示下行SPS释放的PDCCH

For TDD ACK/NACK multiplexing and a subframe with , spatial ACK/NACK bundling across multiple codewords within a DL subframe is performed by a logical AND operation of all the corresponding individual ACK/NACKs. In case the UE is transmitting on PUSCH, the UE shall determine the number of ACK/NAK feedback bits   and the ACK/NACK feedback bits  ,  to be transmitted in subframe n in case the UE is transmitting on PUSCH.

- If the PUSCH transmission is adjusted based on a detected PDCCH with DCI format 0 intended for the UE, then  unless   and  in which case the UE shall not transmit ACK/NACK. The spatially bundled ACK/NACK for a PDSCH transmission with a corresponding PDCCH or for a PDCCH indicating downlink SPS release in subframe  is associated with  where DAI(k) is the value of DAI in DCI format 1A/1B/1D/1/2/2A/2B detected in subframe . For the case with , the ACK/NACK associated with a PDSCH transmission without a corresponding PDCCH is mapped to . The ACK/NACK feedback bits without any detected PDSCH transmission or without detected PDCCH indicating downlink SPS release are set to NACK.

- If the PUSCH transmission is not adjusted based on a detected PDCCH with DCI format 0 intended for the UE, , and  is associated with the spatially bundled ACK/NACK for DL subframe , where. The ACK/NACK feedback bits without any detected PDSCH transmission or without detected PDCCH indicating downlink SPS release are set to NACK. The UE shall not transmit ACK/NACK if .

理解：当ACK在PUSCH上传输，传输的Bit数是不同的。当有DCI 0指示的PUSCH，Bit数，当没有当有DCI 0指示的PUSCH，Bit数为

For TDD when both ACK/NACK and SR are transmitted in the same sub-frame, a UE shall transmit the bundled ACK/NACK or the multiple ACK/NAK responses (according to section 10.1) on its assigned ACK/NACK PUCCH resources for a negative SR transmission. For a positive SR, the UE shall transmit  on its assigned SR PUCCH resource using PUCCH format 1b according to section 5.4.1 in [3]. The value of  are generated according to Table 7.3-1 from the  ACK/NACK responses including ACK in response to PDCCH indicating downlink SPS release by spatial ACK/NAK bundling across multiple codewords within each PDSCH transmission. For TDD UL-DL configurations 1-6, if  and , the UE detects that at least one downlink assignment has been missed.

Table 7.3-1: Mapping between multiple ACK/NACK responses and

| Number of ACK among multiple () ACK/NACK responses |  |
| --- | --- |
| 0 or None (UE detect at least one DL assignment is missed) | 0, 0 |
| 1 | 1, 1 |
| 2 | 1, 0 |
| 3 | 0, 1 |
| 4 | 1, 1 |
| 5 | 1, 0 |
| 6 | 0, 1 |
| 7 | 1, 1 |
| 8 | 1, 0 |
| 9 | 0, 1 |

For TDD when both ACK/NACK and CQI/PMI or RI are configured to be transmitted in the same sub-frame on PUCCH, a UE shall transmit CQI/PMI or RI and  using PUCCH format 2b for normal CP or PUCCH format 2 for extended CP, according to section 5.2.3.4 in [4] with  replaced by . The value of  are generated according to Table 7.3-1 from the  ACK/NACK responses including ACK in response to PDCCH indicating downlink SPS release by spatial ACK/NACK bundling across multiple codewords within each PDSCH transmission. For TDD UL-DL configurations 1-6, if  and , the UE detects that at least one downlink assignment has been missed.

When only ACK/NACK or only a positive SR is transmitted a UE shall use PUCCH Format 1a or 1b for the ACK/NACK resource and PUCCH Format 1 for the SR resource as defined in section 5.4.1 in [3].

8	Physical uplink shared channel related procedures

For FDD, there shall be 8 HARQ processes in the uplink for non-subframe bundling operation, i.e. normal HARQ operation, and 4 HARQ processes in the uplink for subframe bundling operation. The subframe bundling operation is configured by the parameter ttiBundling provided by higher layers.

理解：在非ttiBundling 的情况下，FDD为8个进程数，而在ttiBundling的情况下，FDD为4个进程数。

In case higher layers configure the use of subframe bundling for FDD and TDD, the subframe bundling operation is only applied to UL-SCH, such that four consecutive uplink subframes are used.

理解：子帧绑定操作仅仅应用到UL-SCH，例如4个连续的上行子帧使用子帧绑定。

For FDD and normal HARQ operation, the UE shall upon detection of a PDCCH with DCI format 0 and/or a PHICH transmission in subframe n intended for the UE, adjust the corresponding PUSCH transmission in subframe n+4 according to the PDCCH and PHICH information.

For FDD and subframe bundling operation, the UE shall upon detection of a PDCCH with DCI format 0 in subframe n intended for the UE, and/or a PHICH transmission in subframe n-5 intended for the UE, adjust the corresponding first PUSCH transmission in the bundle in subframe n+4 according to the PDCCH and PHICH information.

理解：在ttiBundling 的情况下，如果是上行自适应，DCI0来上行调度，在N帧DCI0进行上行调度，N+4帧进行第一个绑定的PUSCH传输。如果是上行非自适应，在n-5帧收到PHICH的ACK\NACK反馈，将在n+4帧进行第一个绑定的PUSCH传输，相当于间隔9个子帧，如下图所示。

For FDD and TDD, the NDI as signalled on PDCCH, the RV as determined in section 8.6.1, and the TBS as determined in section 8.6.2, shall be delivered to higher layers.

For TDD, the number of HARQ processes shall be determined by the DL/UL configuration (Table 4.2-2 of [3]), as indicated in table 8-1.

只有上行行子帧配比0、1和6有ttiBundling，因为只有这三种子帧配比上行子帧的个数大于或等于4.

Table 8-1: Number of synchronous UL HARQ processes for TDD

| TDD UL/DL configuration | Number of HARQ processes for normal HARQ operation | Number of HARQ processes for subframe bundling operation |
| --- | --- | --- |
| 0 | 7 | 3 |
| 1 | 4 | 2 |
| 2 | 2 | N/A |
| 3 | 3 | N/A |
| 4 | 2 | N/A |
| 5 | 1 | N/A |
| 6 | 6 | 3 |

For TDD UL/DL configurations 1-6 and normal HARQ operation, the UE shall upon detection of a PDCCH with DCI format 0 and/or a PHICH transmission in subframe n intended for the UE, adjust the corresponding PUSCH transmission in subframe n+k, with k given in Table 8-2, according to the PDCCH and PHICH information.

For TDD UL/DL configuration 0 and normal HARQ operation the UE shall upon detection of a PDCCH with DCI format 0 and/or a PHICH transmission in subframe n intended for the UE, adjust the corresponding PUSCH transmission in subframe n+k if the MSB of the UL index in the DCI format 0 is set to 1 or PHICH is received in subframe n=0 or 5 in the resource corresponding to , as defined in Section 9.1.2, with k given in Table 8-2. If, for TDD UL/DL configuration 0 and normal HARQ operation, the LSB of the UL index in the DCI format 0 is set to 1 in subframe n or a PHICH is received in subframe n=0 or 5 in the resource corresponding to , as defined in Section 9.1.2, or PHICH is received in subframe n=1 or 6, the UE shall adjust the corresponding PUSCH transmission in subframe n+7. If, for TDD UL/DL configuration 0, both the MSB and LSB of the UL index in the DCI format 0 are set in subframe n, the UE shall adjust the corresponding PUSCH transmission in both subframes n+ k and n+7, with k given in Table 8-2

理解：TDD 上下行子帧配比0的情况下，下行子帧少于上行子帧数。一个下行子帧可能需要同时给2个上行子帧发送UL grant。为了实现该功能，DCI0新增了2Bit的字段：UL index。子帧配比0下上行子帧4和子帧9对应，其他的子帧对应。对TDD UL/DL configuration 0而言，如果UE在子帧n收到的UL grant的MSB置为1，或在子帧0或5接收到的PHICH对应（反馈的不是子帧4或9的ACK/NACK信息），则UE会在子帧n + k发送对应的PUSCH，其中k的值见36.213的Table 8-2。

对TDD UL/DL configuration 0而言，如果UE在子帧n收到的UL grant的LSB置为1，或在子帧0或5接收到的PHICH对应，或在子帧1或6接收到PHICH，则UE会在子帧n + 7发送对应的PUSCH。

对TDD UL/DL configuration 0而言，如果UE在子帧n收到的UL grant的MSB和LSB均置为1，则UE会在子帧n + k和n + 7都发送对应的PUSCH。（见36.213的8.0节）

For TDD UL/DL configurations 1 and 6 and subframe bundling operation, the UE shall upon detection of a PDCCH with DCI format 0 in subframe n intended for the UE, and/or a PHICH transmission intended for the UE in subframe n-l with l given in Table 8-2a, adjust the corresponding first PUSCH transmission in the bundle in subframe n+k, with k given in Table 8-2, according to the PDCCH and PHICH information.

理解：和FDD的基本相同，如果是上行非自适应，在n-l帧收到PHICH的ACK\NACK反馈，将在n+k帧进行第一个绑定的PUSCH传输，相当于间隔l+k个子帧，其中k由Table 8-2可以知道，l由Table 8-2a可以知道。

For TDD UL/DL configuration 0 and subframe bundling operation, the UE shall upon detection of a PDCCH with DCI format 0 in subframe n intended for the UE, and/or a PHICH transmission intended for the UE in subframe n-l with l given in Table 8-2a, adjust the corresponding first PUSCH transmission in the bundle in subframe n+k, if the MSB of the UL index in the DCI format 0 is set to 1 or if , as defined in Section 9.1.2, with k given in Table 8-2, according to the PDCCH and PHICH information. If, for TDD UL/DL configuration 0 and subframe bundling operation, the LSB of the UL index in the DCI format 0 is set to 1 in subframe n or if , as defined in Section 9.1.2, the UE shall adjust the corresponding first PUSCH transmission in the bundle in subframe n+7, according to the PDCCH and PHICH information.

理解：上行行子帧配比0、ttiBundling的情况下，是没有一个下行帧给2个上行帧调度的情况，和非ttiBundling的情况是有区别的。

Table 8-2 k for TDD configurations 0-6

| TDD UL/DL Configuration | DL subframe number n |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 |
| 0 | 4 | 6 |  |  |  | 4 | 6 |  |  |  |
| 1 |  | 6 |  |  | 4 |  | 6 |  |  | 4 |
| 2 |  |  |  | 4 |  |  |  |  | 4 |  |
| 3 | 4 |  |  |  |  |  |  |  | 4 | 4 |
| 4 |  |  |  |  |  |  |  |  | 4 | 4 |
| 5 |  |  |  |  |  |  |  |  | 4 |  |
| 6 | 7 | 7 |  |  |  | 7 | 7 |  |  | 5 |

Table 8-2a l for TDD configurations 0, 1 and 6

| TDD UL/DL Configuration | DL subframe number n |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 |
| 0 | 9 | 6 |  |  |  | 9 | 6 |  |  |  |
| 1 |  | 2 |  |  | 3 |  | 2 |  |  | 3 |
| 6 | 5 | 5 |  |  |  | 6 | 6 |  |  | 8 |

If a UE is configured by higher layers to decode PDCCHs with the CRC scrambled by the C-RNTI, the UE shall decode the PDCCH according to the combination defined in table 8-3 and transmit the corresponding PUSCH. The scrambling initialization of this PUSCH corresponding to these PDCCHs and the PUSCH retransmission for the same transport block is by C-RNTI.

# Table 8-3: PDCCH configured by C-RNTI

| DCI format | Search Space |
| --- | --- |
| DCI format 0 | Common and UE specific by C-RNTI |

If a UE is configured by higher layers to decode PDCCHs with the CRC scrambled by the C-RNTI and is also configured to receive random access procedures initiated by PDCCH orders, the UE shall decode the PDCCH according to the combination defined in table 8-4.

# Table 8-4: PDCCH configured as PDCCH order to initiate random access procedure

| DCI format | Search Space |
| --- | --- |
| DCI format 1A | Common and UE specific by C-RNTI |

If a UE is configured by higher layers to decode PDCCHs with the CRC scrambled by the SPS C-RNTI, the UE shall decode the PDCCH according to the combination defined in table 8-5 and transmit the corresponding PUSCH. The scrambling initialization of this PUSCH corresponding to these PDCCHs and PUSCH retransmission for the same transport block is by SPS C-RNTI. The scrambling initialization of initial transmission of this PUSCH without a corresponding PDCCH and the PUSCH retransmission for the same transport block is by SPS C-RNTI.

# Table 8-5: PDCCH configured by SPS C-RNTI

| DCI format | Search Space |
| --- | --- |
| DCI format 0 | Common and UE specific by C-RNTI |

If a UE is configured by higher layers to decode PDCCHs with the CRC scrambled by the Temporary C-RNTI regardless of whether UE is configured or not configured to decode PDCCHs with the CRC scrambled by the C-RNTI, the UE shall decode the PDCCH according to the combination defined in table 8-6 and transmit the corresponding PUSCH. The scrambling initialization of PUSCH corresponding to these PDCCH is by Temporary C-RNTI.

If a Temporary C-RNTI is set by higher layers, the scrambling of PUSCH corresponding to the Random Access Response Grant in Section 6.2 and the PUSCH retransmission for the same transport block is by Temporary C-RNTI. Else, the scrambling of PUSCH corresponding to the Random Access Response Grant in Section 6.2 and the PUSCH retransmission for the same transport block is by C-RNTI.

# Table 8-6: PDCCH configured by Temporary C-RNTI

| DCI format | Search Space |
| --- | --- |
| DCI format 0 | Common |

If a UE is configured by higher layers to decode PDCCHs with the CRC scrambled by the TPC-PUCCH-RNTI, the UE shall decode the PDCCH according to the combination defined in table 8-7. The notation 3/3A implies that the UE shall receive either DCI format 3 or DCI format 3A depending on the configuration.

# Table 8-7: PDCCH configured by TPC-PUCCH-RNTI

| DCI format | Search Space |
| --- | --- |
| DCI format 3/3A | Common |

If a UE is configured by higher layers to decode PDCCHs with the CRC scrambled by the TPC-PUSCH-RNTI, the UE shall decode the PDCCH according to the combination defined in table 8.8. The notation 3/3A implies that the UE shall receive either DCI format 3 or DCI format 3A depending on the configuration.

# Table 8-8: PDCCH configured by TPC-PUSCH-RNTI

| DCI format | Search Space |
| --- | --- |
| DCI format 3/3A | Common |

理解：上行共享信道相关处理过程中，初始随机接入RA过程使用PDCCH的DCI1A，其它的C-RNTI、SPS C-RNTI、Temp C-RNTI使用的都是PDCCH的DCI0，TPC-PUCCH-RNTI、TPC-PUSCH-RNTI使用DCI3/3A，从上面可以看出上行共享信道使用的DCI有DCI0、DCI1A和DCI3/3A

8.1	Resource Allocation for PDCCH DCI Format 0

The resource allocation information indicates to a scheduled UE a set of contiguously allocated virtual resource block indices denoted by . A resource allocation field in the scheduling grant consists of a resource indication value (RIV) corresponding to a starting resource block () and a length in terms of contiguously allocated resource blocks ( 1).  The resource indication value is defined by

if  then

else

A UE shall discard PUSCH resource allocation in the corresponding PDCCH with DCI format 0 if consistent control information is not detected.

理解：RIV是用有限的Bit数来表示RB的起始位置和长度，减少信令的一种方式

8.2	UE sounding procedure

The following Sounding Reference Symbol (SRS) parameters are UE specific semi-statically configurable by higher layers:

- Transmission comb , as defined in Section 5.5.3.2 of [3]

- Starting physical resource block assignment , as defined in Section 5.5.3.2 of [3]

- duration: single or indefinite (until disabled), as defined in [11]理解：SRS的传输持续时间，FALSE表示一个符号，TRUE表示不确定

- srs-ConfigIndex ISRS for SRS periodicity and SRS subframe offset , as defined in Table 8.2-1 and Table 8.2-2

- SRS bandwidth , as defined in Section 5.5.3.2 of [3]

- Frequency hopping bandwidth, , as defined in Section 5.5.3.2 of [3]

- Cyclic shift , as defined in Section 5.5.3.1 of [3]

The cell specific SRS transmission bandwidths  are configured by higher layers. The allowable values are given in Section 5.5.3.2 of [3].

The cell specific SRS transmission sub-frames are configured by higher layers. The allowable values are given in Section 5.5.3.3 of [3].

When antenna selection is enabled for a UE that supports transmit antenna selection, the index , of the UE antenna that transmits the SRS at time nSRS is given by

理解：终端的两个天线上都需要发送SRS的情况下，SRS的传输在两个天线间来回切换，同一时刻只有一个天线发送SRS。

, for both partial and full sounding bandwidth, and when frequency hopping is disabled (i.e., ),

1.不跳频时天线的选择是根据SRS的发送次数来选择天线的，2个天线来回切换。

,

when frequency hopping is enabled (i.e., ),

where values BSRS, bhop, Nb, and nSRS are given in Section 5.5.3.2 of [3], and (where  regardless of the  value), except when a single SRS transmission is configured for the UE.

For TDD, when one SC-FDMA symbol exists in UpPTS, it can be used for SRS transmission. When two SC-FDMA symbols exist in UpPTS, both can be used for SRS transmission and both can be assigned to the same UE.

A UE shall not transmit SRS whenever SRS and PUCCH format 2/2a/2b transmissions happen to coincide in the same subframe. (理解：当SRS和format2系列的PUCCH碰撞时，SRS丢掉)

A UE shall not transmit SRS whenever SRS transmission and PUCCH transmission carrying ACK/NACK and/or positive SR happen to coincide in the same subframe if the parameter ackNackSRS-SimultaneousTransmission  is FALSE. A UE shall transmit SRS whenever SRS transmission and PUCCH transmission carrying ACK/NACK and/or positive SR happen to coincide in the same subframe if the parameter ackNackSRS-SimultaneousTransmission  is TRUE.

In UpPTS, whenever SRS transmission instance overlaps with the PRACH region for preamble format 4 or exceeds the range of uplink system bandwidth configured in the cell, the UE shall not transmit SRS.

The parameter ackNackSRS-SimultaneousTransmission provided by higher layers determines if a UE is configured to support the transmission of ACK/NACK on PUCCH and SRS in one subframe. If it is configured to support the transmission of ACK/NACK on PUCCH and SRS in one subframe, then in the cell specific SRS subframes UE shall transmit ACK/NACK and SR using the shortened PUCCH format as defined in Section 5.4.1 of [3], where the ACK/NACK or the SR symbol corresponding to the SRS location is punctured. This shortened PUCCH format shall be used in a cell specific SRS subframe even if the UE does not transmit SRS in that subframe. The cell specific SRS subframes are defined in Section 5.5.3.3 of [3]. Otherwise, the UE shall use the normal PUCCH format 1/1a/1b as defined in Section 5.4.1 of [3] for the transmission of ACK/NACK and SR.

The UE specific SRS configuration for SRS periodicity,, and SRS subframe offset,, is defined in Table 8.2-1 and Table 8.2-2, for FDD and TDD, respectively. The periodicity  of the SRS transmission is selected from the set  {2, 5, 10, 20, 40, 80, 160, 320} ms or subframes. For the SRS periodicity  of 2 ms in TDD, two SRS resources are configured in a half frame containing UL subframe(s).

SRS transmission instances for TDD with  and for FDD are the subframes satisfying  , where for FDD  is the subframe index within the frame, for TDD  is defined in Table 8.2-3. The SRS transmission instances for TDD with  are the subframes satisfying .

A UE shall not transmit SRS whenever SRS and a PUSCH transmission corresponding to a Random Access Response Grant or a retransmission of the same transport block as part of the contention based random access procedure coincide in the same subframe.

Table 8.2-1: UE Specific SRS Periodicity  and Subframe Offset Configuration , FDD

| SRS Configuration Index ISRS | SRS Periodicity (ms) | SRS Subframe Offset |
| --- | --- | --- |
| 0 – 1 | 2 | ISRS |
| 2 – 6 | 5 | ISRS – 2 |
| 7 – 16 | 10 | ISRS – 7 |
| 17 – 36 | 20 | ISRS – 17 |
| 37 – 76 | 40 | ISRS – 37 |
| 77 – 156 | 80 | ISRS – 77 |
| 157 – 316 | 160 | ISRS – 157 |
| 317 – 636 | 320 | ISRS – 317 |
| 637 – 1023 | reserved | reserved |

Table 8.2-2: UE Specific SRS Periodicity  and Subframe Offset Configuration , TDD

| SRS Configuration Index ISRS | SRS Periodicity (ms) | SRS Subframe Offset |
| --- | --- | --- |
| 0 | 2 | 0, 1 |
| 1 | 2 | 0, 2 |
| 2 | 2 | 1, 2 |
| 3 | 2 | 0, 3 |
| 4 | 2 | 1, 3 |
| 5 | 2 | 0, 4 |
| 6 | 2 | 1, 4 |
| 7 | 2 | 2, 3 |
| 8 | 2 | 2, 4 |
| 9 | 2 | 3, 4 |
| 10 – 14 | 5 | ISRS – 10 |
| 15 – 24 | 10 | ISRS – 15 |
| 25 – 44 | 20 | ISRS – 25 |
| 45 – 84 | 40 | ISRS – 45 |
| 85 – 164 | 80 | ISRS – 85 |
| 165 – 324 | 160 | ISRS – 165 |
| 325 – 644 | 320 | ISRS – 325 |
| 645 – 1023 | reserved | reserved |

Table 8.2-3:  for TDD

|  | subframe index n |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 |  | 2 | 3 | 4 | 5 | 6 |  | 7 | 8 | 9 |
|  |  | 1st symbol of UpPTS | 2nd symbol of UpPTS |  |  |  |  | 1st symbol of UpPTS | 2nd symbol of UpPTS |  |  |  |
| in case UpPTS length of 2 symbols |  | 0 | 1 | 2 | 3 | 4 |  | 5 | 6 | 7 | 8 | 9 |
| in case UpPTS length of 1 symbol |  | 1 |  | 2 | 3 | 4 |  | 6 |  | 7 | 8 | 9 |

8.3	UE ACK/NACK procedure

For Frame Structure type 1, an ACK/NACK received on the PHICH assigned to a UE in subframe i is associated with the PUSCH transmission in subframe i-4.

For Frame Structure type 2 UL/DL configuration 1-6, an ACK/NACK received on the PHICH assigned to a UE in subframe i is associated with the PUSCH transmission in the subframe i-k as indicated by the following table 8.3-1.

For Frame Structure type 2 UL/DL configuration 0, an ACK/NACK received on the PHICH in the resource corresponding to , as defined in Section 9.1.2, assigned to a UE in subframe i is associated with the PUSCH transmission in the subframe i-k as indicated by the following table 8.3-1. If, for Frame Structure type 2 UL/DL configuration 0, an ACK/NACK received on the PHICH in the resource corresponding to , as defined in Section 9.1.2, assigned to a UE in subframe i is associated with the PUSCH transmission in the subframe i-6.

理解：TDD 上下行子帧配比0的情况下，下行子帧少于上行子帧数，子帧0需要对应上行子帧3和子帧4的反馈，子帧5需要对应上行子帧8和子帧9的反馈。子帧配比0下上行子帧4和子帧9对应，其他的子帧对应如果UE在对应的子帧n收到了PHICH，则该PHICH对应UE上行子帧n - k发送的PUSCH数据，其中k的值见的Table 8.3-1；如果UE在对应的子帧n收到了PHICH，则该PHICH对应UE上行子帧n - 6发送的PUSCH数据

Table 8.3-1 k for TDD configurations 0-6

| TDD UL/DL Configuration | DL subframe number i |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 |
| 0 | 7 | 4 |  |  |  | 7 | 4 |  |  |  |
| 1 |  | 4 |  |  | 6 |  | 4 |  |  | 6 |
| 2 |  |  |  | 6 |  |  |  |  | 6 |  |
| 3 | 6 |  |  |  |  |  |  |  | 6 | 6 |
| 4 |  |  |  |  |  |  |  |  | 6 | 6 |
| 5 |  |  |  |  |  |  |  |  | 6 |  |
| 6 | 6 | 4 |  |  |  | 7 | 4 |  |  | 6 |

The physical layer in the UE shall deliver indications to the higher layers as follows:

For downlink subframe i, if a transport block was transmitted in the associated PUSCH subframe then:

    - if ACK is decoded on the PHICH in subframe i, ACK shall be delivered to the higher layers;

    - else NACK shall be delivered to the higher layers.

8.4	UE PUSCH Hopping procedure

The UE shall perform PUSCH frequency hopping if the single bit frequency hopping (FH) field in a corresponding PDCCH with DCI format 0 is set to 1 otherwise no PUSCH frequency hopping is performed.

A UE performing PUSCH frequency hopping shall determine its PUSCH resource allocation (RA) for the first slot of a subframe (S1) including the lowest index PRB () in subframe n from the resource allocation field in the latest PDCCH with DCI format 0 for the same transport block. If there is no PDCCH for the same transport block, the UE shall determine its hopping type based on

- the hopping information in the most recent semi-persistent scheduling assignment PDCCH, when the initial PUSCH for the same transport block is semi-persistently scheduled or

- the random access response grant for the same transport block, when the PUSCH is initiated by the random access response grant.

理解：

The resource allocation field in DCI format 0 excludes either 1 or 2 bits used for hopping information as indicated by Table 8.4-1 below where the number of PUSCH resource blocks is defined as

表示RRC消息中PUSCH-Config中的pusch-HoppingOffset项，表示调频偏移。

表示RRC消息中PUSCH-Config中的n-SB项，表示子带数。

Type1和Type2的类型由1个或2个跳频Bit确定，由表8.4-2来确定。

For type 1 and type 2 PUSCH hopping,  if  is an odd number where  defined in [3].  in other cases.  The size of the resource allocation field in DCI format 0 after excluding either 1 or 2 bits shall be , where NUL_hop = 1 or 2 bits. The number of contiguous RBs that can be assigned to a type-1 hopping user is limited to . The number of contiguous RBs that can be assigned to a type-2 hopping user is limited to min(,), where the number of sub-bands  is given by higher layers.

理解：Type1和Type2的分配的连续RB数都有上限，如果是Type1，上限是，如果是Type2，上限是min(,)。

A UE performing PUSCH frequency hopping shall use one of two possible PUSCH frequency hopping types based on the hopping information.  PUSCH hopping type 1 is described in section 8.4.1 and type 2 is described in section 8.4.2.

Table 8.4-1: Number of Hopping Bits NUL_hop vs. System Bandwidth

| System BW | #Hopping bits for 2nd slot RA (NUL_hop) |
| --- | --- |
| 6-49 | 1 |
| 50-110 | 2 |

The parameter Hopping-mode provided by higher layers determines if PUSCH frequency hopping is “inter-subframe” or “intra and inter-subframe”.

8.4.1  	Type 1 PUSCH Hopping

For PUSCH hopping type 1 the hopping bit or bits indicated in Table 8.4-1 determine as defined in Table 8.4-2. The lowest index PRB () of the 1st slot RA in subframe i is defined as , where , and is obtained from the uplink scheduling grant as in Section 8.4 and Section 8.1.

The lowest index PRB () of the 2nd slot RA in subframe i is defined as .

The set of physical resource blocks to be used for PUSCH transmission are contiguously allocated resource blocks from PRB index  for the 1st slot, and from PRB index  for the 2nd slot, respectively, where is obtained from the uplink scheduling grant as in Section 8.4 and Section 8.1.

理解：第一个时隙的RB_Start和RB_Num由DCI0中的RIV值计算得到，但是第二个时隙的RB_Start由公式计算得到，由表8.4-2得到，其中由推出。

第二个时隙的RB_Num和第一个时隙的相同，都是由RIV值计算得到。

If the Hopping-mode is "inter-subframe", the 1st slot RA is applied to even CURRENT_TX_NB, and the 2nd slot RA is applied to odd CURRENT_TX_NB, where CURRENT_TX_NB is defined in [8].

理解：如果是子帧间跳频，时隙0的RB位置用在偶数MAC PDU传输的次数中，时隙1RB位置用在奇数MAC PDU传输的次数中。

8.4.2  	Type 2 PUSCH Hopping

In PUSCH hopping type 2 the set of physical resource blocks to be used for transmission in slot  is given by the scheduling grant together with a predefined pattern according to [3] section 5.3.4. If the system frame number is not acquired by the UE yet, the UE shall not transmit PUSCH with type-2 hopping and  for TDD, where  is defined in [3].

理解：跳频模式用高层Hopping-mode指定，帧间跳频还是帧内/帧间调频。根据RB的起始位置不同，又有2种跳频类型，类型1跳频和类型2跳频，根据跳频信息比特NUL_hop 指示，根据表8.4-2来确定确定调频类型，同时也获取跳频间隔。

Table 8.4-2: PDCCH DCI Format 0 Hopping Bit Definition

| System BW | Number of Hopping bits | Information in hopping bits |  |
| --- | --- | --- | --- |
| 6 – 49 | 1 | 0 | , |
|  |  | 1 | Type 2 PUSCH Hopping |
| 50 – 110 | 2 | 00 |  |
|  |  | 01 |  |
|  |  | 10 |  |
|  |  | 11 | Type 2 PUSCH Hopping |

8.5	UE Reference Symbol procedure

If UL sequence hopping is configured in the cell, it applies to all reference symbols (SRS, PUSCH and PUCCH RS).

8.6	Modulation order, redundancy version and transport block size determination

To determine the modulation order, redundancy version and transport block size for the physical uplink shared channel, the UE shall first

- read the “modulation and coding scheme and redundancy version” field (), and

- check the “CQI request” bit, and

- compute the total number of allocated PRBs () based on the procedure defined in Section 8.1, and

- compute the number of coded symbols for control information.

8.6.1		Modulation order and redundancy version determination

For, the modulation order () is determined as follows:

- If the UE is capable of supporting 64QAM in PUSCH and has not been configured by higher layers to transmit only QPSK and 16QAM, the modulation order is given by in Table 8.6.1-1.

- If the UE is not capable of supporting 64QAM in PUSCH or has been configured by higher layers to transmit only QPSK and 16QAM, is first read from Table 8.6.1-1. The modulation order is set to .

- If the parameter ttiBundling provided by higher layers is set to TRUE, then the resource allocation size is restricted to  and the modulation order is set to .

理解：如果高层配置ttiBundling使能，则调制方式固定为QPSK，物理Rb个数为

For, if, the “CQI request” bit in DCI format 0 is set to 1 and , the modulation order is set to . Otherwise, the modulation order shall be determined from the DCI transported in the latest PDCCH with DCI format 0 for the same transport block using . If there is no PDCCH with DCI format 0 for the same transport block using , the modulation order shall be determined from

理解：当MCS为29，同时CQI request位置1和的情况下，调制方式固定为QPSK。

- the most recent semi-persistent scheduling assignment PDCCH, when the initial PUSCH for the same transport block is semi-persistently scheduled, or,

- the random access response grant for the same transport block, when the PUSCH is initiated by the random access response grant.

The UE shall useand Table 8.6.1-1 to determine the redundancy version (rvidx) to use in the physical uplink shared channel.

Table 8.6.1-1: Modulation, TBS index and redundancy version table for PUSCH

| MCS Index | Modulation Order | TBS Index | Redundancy Version rvidx |
| --- | --- | --- | --- |
| 0 | 2 | 0 | 0 |
| 1 | 2 | 1 | 0 |
| 2 | 2 | 2 | 0 |
| 3 | 2 | 3 | 0 |
| 4 | 2 | 4 | 0 |
| 5 | 2 | 5 | 0 |
| 6 | 2 | 6 | 0 |
| 7 | 2 | 7 | 0 |
| 8 | 2 | 8 | 0 |
| 9 | 2 | 9 | 0 |
| 10 | 2 | 10 | 0 |
| 11 | 4 | 10 | 0 |
| 12 | 4 | 11 | 0 |
| 13 | 4 | 12 | 0 |
| 14 | 4 | 13 | 0 |
| 15 | 4 | 14 | 0 |
| 16 | 4 | 15 | 0 |
| 17 | 4 | 16 | 0 |
| 18 | 4 | 17 | 0 |
| 19 | 4 | 18 | 0 |
| 20 | 4 | 19 | 0 |
| 21 | 6 | 19 | 0 |
| 22 | 6 | 20 | 0 |
| 23 | 6 | 21 | 0 |
| 24 | 6 | 22 | 0 |
| 25 | 6 | 23 | 0 |
| 26 | 6 | 24 | 0 |
| 27 | 6 | 25 | 0 |
| 28 | 6 | 26 | 0 |
| 29 | reserved |  | 1 |
| 30 |  |  | 2 |
| 31 |  |  | 3 |

8.6.2		Transport block size determination

For, the UE shall first determine the TBS index () usingand Table 8.6.1-1. The UE shall then follow the procedure in Section 7.1.7.2.1 to determine the transport block size.

For, if, the “CQI request” bit in DCI format 0 is set to 1 and, then there is no transport block for the UL-SCH and only the control information feedback for the current PUSCH reporting mode is transmitted by the UE. 理解：当MCS为29，同时CQI request位置1和的情况下，PUSCH上只传输控制信息，不传输数据信息。

Otherwise, the transport block size shall be determined from the initial PDCCH for the same transport block using . If there is no initial PDCCH with DCI format 0 for the same transport block using , the transport block size shall be determined from

- the most recent semi-persistent scheduling assignment PDCCH, when the initial PUSCH for the same transport block is semi-persistently scheduled, or,

- the random access response grant for the same transport block, when the PUSCH is initiated by the random access response grant.

8.6.3		Control information MCS offset determination

,  and   shall be configured to values according to Table 8.6.3-1,2,3  with the higher layer signalled indexes , , and , respectively.

理解：当上行业务信道和上行控制信息联合在PUSCH中传输的时候，需要进一步定义控制信息占用的RE位置与个数。由212中5.2.2.6中ACK、RI和CQI占用的Bit数需要使用，控制控制信息占用的Bit数。

Table 8.6.3-1: Mapping of HARQ-ACK offset values and the index signalled by higher layers

| 0 | 2.000 |
| --- | --- |
| 1 | 2.500 |
| 2 | 3.125 |
| 3 | 4.000 |
| 4 | 5.000 |
| 5 | 6.250 |
| 6 | 8.000 |
| 7 | 10.000 |
| 8 | 12.625 |
| 9 | 15.875 |
| 10 | 20.000 |
| 11 | 31.000 |
| 12 | 50.000 |
| 13 | 80.000 |
| 14 | 126.000 |
| 15 | reserved |

Table 8.6.3-2: Mapping of RI offset values and the index signalled by higher layers

| 0 | 1.250 |
| --- | --- |
| 1 | 1.625 |
| 2 | 2.000 |
| 3 | 2.500 |
| 4 | 3.125 |
| 5 | 4.000 |
| 6 | 5.000 |
| 7 | 6.250 |
| 8 | 8.000 |
| 9 | 10.000 |
| 10 | 12.625 |
| 11 | 15.875 |
| 12 | 20.000 |
| 13 | reserved |
| 14 | reserved |
| 15 | reserved |

Table 8.6.3-3: Mapping of CQI offset values and the index signalled by higher layers

| 0 | reserved |
| --- | --- |
| 1 | reserved |
| 2 | 1.125 |
| 3 | 1.250 |
| 4 | 1.375 |
| 5 | 1.625 |
| 6 | 1.750 |
| 7 | 2.000 |
| 8 | 2.250 |
| 9 | 2.500 |
| 10 | 2.875 |
| 11 | 3.125 |
| 12 | 3.500 |
| 13 | 4.000 |
| 14 | 5.000 |
| 15 | 6.250 |

8.7	UE Transmit Antenna Selection

UE transmit antenna selection is configured by higher layers.

If UE transmit antenna selection is disabled or not supported by the UE, the UE shall transmit from UE port 0.

If closed-loop UE transmit antenna selection is enabled by higher layers the UE shall perform transmit antenna selection in response to the most recent command received via DCI Format 0 in section 5.3.3.2 [4].

If open-loop UE transmit antenna selection is enabled by higher layers, the transmit antenna to be selected by the UE is not specified.

理解：1.UE的传输天线选择是根据高层配置的，如果UE不支持天线选择或者没有使能，UE使用端口0传输。

2.UE的选择开环和闭环是根据高层参数RadioResourceConfigDedicated-> PhysicalConfigDedicated-> antennaInfo-> AntennaInfoDedicated来确定。

3. 如果是选择开环，UE自己选择天线，不受基站的控制。

9	Physical downlink control channel procedures

9.1	UE procedure for determining physical downlink control channel assignment

9.1.1  	PDCCH Assignment Procedure

The control region consists of a set of CCEs, numbered from 0 to  according to Section 6.8.1 in [3], where  is the total number of CCEs in the control region of subframe . The UE shall monitor a set of PDCCH candidates for control information in every non-DRX subframe, where monitoring implies attempting to decode each of the PDCCHs in the set according to all the monitored DCI formats.

The set of PDCCH candidates to monitor are defined in terms of search spaces, where a search space  at aggregation level  is defined by a set of PDCCH candidates. The CCEs corresponding to PDCCH candidate m of the search space  are given by

where is defined below,  and .  is the number of PDCCH candidates to monitor in the given search space.

理解：L表示聚合度，表示盲检区间，盲检区域包括个盲检点，每个盲检点包括L个CCE。由于每个UE盲检出来的是整个PDCCH占用的CCE个数，这些CCE中有本UE和其他UE的DCI，本UE只需要找到自己所需要DCI所在的CCE位置就行。

The UE shall monitor one common search space at each of the aggregation levels 4 and 8 and one UE-specific search space at each of the aggregation levels 1, 2, 4, 8. The common and UE-specific search spaces may overlap.

The aggregation levels defining the search spaces are listed in Table 9.1.1-1. The DCI formats that the UE shall monitor depend on the configured transmission mode as defined in Section 7.1.

Table 9.1.1-1: PDCCH candidates monitored by a UE.

| Search space |  |  | Number of PDCCH candidates |
| --- | --- | --- | --- |
| Type | Aggregation level | Size [in CCEs] |  |
| UE-specific | 1 | 6 | 6 |
|  | 2 | 12 | 6 |
|  | 4 | 8 | 2 |
|  | 8 | 16 | 2 |
| Common | 4 | 16 | 4 |
|  | 8 | 16 | 2 |

For the common search spaces, is set to 0 for the two aggregation levels  and .

For the UE-specific search space  at aggregation level, the variable is defined by

where , ,  and ,  is the slot number within a radio frame. The RNTI value used for  is defined in section 7.1 in downlink and section 8 in uplink.

9.1.2  	PHICH Assignment Procedure

For scheduled PUSCH transmissions in subframe n, a UE shall determine the corresponding PHICH resource in subframe , where  is always 4 for FDD and is given in table 9.1.2-1 for TDD. For subframe bundling operation, the corresponding PHICH resource is associated with the last subframe in the bundle.

Table 9.1.2-1:  for TDD

| TDD UL/DL Configuration | UL subframe index n |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 |
| 0 |  |  | 4 | 7 | 6 |  |  | 4 | 7 | 6 |
| 1 |  |  | 4 | 6 |  |  |  | 4 | 6 |  |
| 2 |  |  | 6 |  |  |  |  | 6 |  |  |
| 3 |  |  | 6 | 6 | 6 |  |  |  |  |  |
| 4 |  |  | 6 | 6 |  |  |  |  |  |  |
| 5 |  |  | 6 |  |  |  |  |  |  |  |
| 6 |  |  | 4 | 6 | 6 |  |  | 4 | 7 |  |

The PHICH resource is identified by the index pairwhere is the PHICH group number and is the orthogonal sequence index within the group as defined by:

where

  - is mapped from the cyclic shift for DMRS field (according to Table 9.1.2-2) in the most recent DCI format 0 [4] for the transport block associated with the corresponding PUSCH transmission. shall be set to zero, if there is no PDCCH with DCI format 0 for the same transport block, and

    - if the initial PUSCH for the same transport block is semi-persistently scheduled, or

    - if the initial PUSCH for the same transport block is scheduled by the random access response grant .

  - is the spreading factor size used for PHICH modulation as described in section 6.9.1 in [3].

  - is the lowest PRB index in the first slot of the corresponding PUSCH transmission

  - is the number of PHICH groups configured by higher layers as described in section 6.9 of [3],

计算需要知道，是在MIB中广播的。

理解：上行PUSCH发送数据，在下行PHICH上反馈ACK\NACK，ACK\NACK所在的位置（包括组号和序列号）是和PUSCH的RB位置和DCI0中的相关的，当DCI0确定后，组号和序列号就都确定了。

Table 9.1.2-2: Mapping between  and the cyclic shift for DMRS field in DCI format 0 in [4]

| Cyclic Shift for DMRS Field in DCI format 0 in [4] |  |
| --- | --- |
| 000 | 0 |
| 001 | 1 |
| 010 | 2 |
| 011 | 3 |
| 100 | 4 |
| 101 | 5 |
| 110 | 6 |
| 111 | 7 |

9.2	PDCCH validation for semi-persistent scheduling

A UE shall validate a Semi-Persistent Scheduling assignment PDCCH only if all the following conditions are met:

-    the CRC parity bits obtained for the PDCCH payload are scrambled with the Semi-Persistent Scheduling C-RNTI

-    the new data indicator field is set to ‘0’. In case of DCI formats 2, 2A and 2B, the new data indicator field refers to the one for the enabled transport block.

Validation is achieved if all the fields for the respective used DCI format are set according to Table 9.2-1 or Table 9.2-1A.

If validation is achieved, the UE shall consider the received DCI information accordingly as a valid semi-persistent activation or release.

If validation is not achieved, the received DCI format shall be considered by the UE as having been received with a non-matching CRC.

Table 9.2-1: Special fields for Semi-Persistent Scheduling Activation PDCCH Validation

|  | DCI format 0 | DCI format 1/1A | DCI format 2/2A/2B |
| --- | --- | --- | --- |
| TPC command for scheduled PUSCH | set to ‘00’ | N/A | N/A |
| Cyclic shift DM RS | set to ‘000’ | N/A | N/A |
| Modulation and coding scheme and redundancy version | MSB is set to ‘0’ | N/A | N/A |
| HARQ process number | N/A | FDD: set to ‘000’ TDD: set to ‘0000’ | FDD: set to ‘000’ TDD: set to ‘0000’ |
| Modulation and coding scheme | N/A | MSB is set to ‘0’ | For the enabled transport block: MSB is set to ‘0’ |
| Redundancy version | N/A | set to ‘00’ | For the enabled transport block: set to ‘00’ |

Table 9.2-1A: Special fields for Semi-Persistent Scheduling Release PDCCH Validation

|  | DCI format 0 | DCI format 1A |
| --- | --- | --- |
| TPC command for scheduled PUSCH | set to ‘00’ | N/A |
| Cyclic shift DM RS | set to ‘000’ | N/A |
| Modulation and coding scheme and redundancy version | set to ‘11111’ | N/A |
| Resource block assignment and hopping resource allocation | Set to all ‘1’s | N/A |
| HARQ process number | N/A | FDD: set to ‘000’ TDD: set to ‘0000’ |
| Modulation and coding scheme | N/A | set to ‘11111’ |
| Redundancy version | N/A | set to ‘00’ |
| Resource block assignment | N/A | Set to all ‘1’s |

For the case that the DCI format indicates a semi-persistent downlink scheduling activation, the TPC command for PUCCH field shall be used as an index to one of the four PUCCH resource indices configured by higher layers, with the mapping defined in Table 9.2-2

理解：DCI0用于通知激活上行PUSCH的半持续调度，DCI1、DCI1A、DCI2和DCI2A用于激活下行PDSCH的半持续调度，不同的DCI对应不同的传输模式，对应表Table 7.1-6。表9.2-1给出了半持续调度的激活方式。DCI0用于通知释放上行PUSCH的半持续调度，DCI1A用于通知释放下行PDSCH的半持续调度，表9.2-1A给出了半持续调度的释放方式。对于下行半持续调度激活需要通过PUCCH的ACK\NACK进行确认，PUCCH的资源索引是通过高层配置的4个PUCCH资源中的一个，具体采用那个PUCCH资源是通过DCI1A中的PUCCH功率控制比特信息域进行指示，对应的资源索引如表9.2-2.

4个PUCCH资源是通过SPS-ConfigDL的n1PUCCH-AN-PersistentList来配置的，List中有4个PUCCH资源。

Table 9.2-2: PUCCH Resource Index for Downlink Semi-Persistent Scheduling

| Value of ‘TPC command for PUCCH’ |  |
| --- | --- |
| ‘00’ | The first PUCCH resource index configured by the higher layers |
| ‘01’ | The second PUCCH resource index configured by the higher layers |
| ‘10’ | The third PUCCH resource index configured by the higher layers |
| ‘11’ | The fourth PUCCH resource index configured by the higher layers |

10	Physical uplink control channel procedures

10.1	UE procedure for determining physical uplink control channel assignment

Uplink control information (UCI) in subframe  shall be transmitted

- on PUCCH using format 1/1a/1b or 2/2a/2b if the UE is not transmitting on PUSCH in subframe

- on PUSCH if the UE is transmitting on PUSCH in subframe unless the PUSCH transmission corresponds to a Random Access Response Grant or a retransmission of the same transport block as part of the contention based random access procedure, in which case UCI is not transmitted

Throughout this section, subframes are numbered in monotonically increasing order; if the last subframe of a radio frame is denoted as , the first subframe of the next radio frame is denoted as .

Using the PUCCH formats defined in section 5.4.1 and 5.4.2 in [3], the following combinations of uplink control information on PUCCH are supported:

- Format 1a for 1-bit HARQ-ACK  or in case of FDD for 1-bit HARQ-ACK with positive SR

- Format 1b for 2-bit HARQ-ACK or for 2-bit HARQ-ACK with positive SR

- Format 1b for HARQ-ACK with channel selection

- Format 1 for positive SR

- Format 2 for a CQI/PMI or a RI report when not multiplexed with HARQ-ACK

- Format 2a for a CQI/PMI or a RI report multiplexed with 1-bit HARQ-ACK for normal cyclic prefix

- Format 2b for a CQI/PMI or a RI report multiplexed with 2-bit HARQ-ACK for normal cyclic prefix

- Format 2 for a CQI/PMI or a RI report multiplexed with HARQ-ACK for extended cyclic prefix

The scrambling initialization of PUCCH format 2, 2a and 2b is by C-RNTI.

In case of collision between a CQI/PMI/RI and an HARQ-ACK in a same subframe without PUSCH, the CQI/PMI/RI is multiplexed with HARQ-ACK on PUCCH if the parameter simultaneousAckNackAndCQI provided by higher layers is set TRUE, otherwise the CQI/PMI/RI is dropped.

In case of collision between a periodic CQI/PMI/RI and an HARQ-ACK in a same subframe with PUSCH, the periodic CQI/PMI/RI is multiplexed with the HARQ-ACK in the PUSCH transmission in that subframe.

For TDD, two ACK/NACK feedback modes are supported by higher layer configuration.

- ACK/NACK bundling and

- ACK/NACK multiplexing

For TDD UL-DL configuration 5, only ACK/NACK bundling is supported.

TDD ACK/NACK bundling is performed per codeword across  multiple DL subframes associated with a single UL subframe n, where  is the number of elements in the set  defined in Table 10.1-1, by a logical AND operation of all the individual PDSCH transmission (with and without corresponding PDCCH) ACK/NACKs and ACK in response to PDCCH indicating downlink SPS release. The bundled 1 or 2 ACK/NACK bits are transmitted using PUCCH format 1a or PUCCH format 1b, respectively.

For TDD ACK/NACK multiplexing and a subframe n with, where  is the number of elements in the set  defined in Table 10.1-1, spatial ACK/NACK bundling across multiple codewords within a DL subframe is performed by a logical AND operation of all the corresponding individual ACK/NACKs and PUCCH format 1b with channel selection is used. For TDD ACK/NACK multiplexing and a subframe n with , spatial ACK/NACK bundling across multiple codewords within a DL subframe is not performed, 1 or 2 ACK/NACK bits are transmitted using PUCCH format 1a or PUCCH format 1b, respectively.

For FDD, the UE shall use PUCCH resource  for transmission of HARQ-ACK in subframe , where

- for a PDSCH transmission indicated by the detection of a corresponding PDCCH in subframe , or for a PDCCH indicating downlink SPS release (defined in section 9.2) in subframe ,  the UE shall use , where  is the number of the first CCE used for transmission of the corresponding DCI assignment and  is configured by higher layers.

- for a PDSCH transmission where there is not a corresponding PDCCH detected in subframe , the value of  is determined according to higher layer configuration and Table 9.2-2.

For TDD ACK/NACK bundling or TDD ACK/NACK multiplexing and a subframe n with  where  is the number of elements in the set  defined in Table 10.1-1, the UE shall use PUCCH resource  for transmission of HARQ-ACK in subframe , where

- If there is PDSCH transmission indicated by the detection of corresponding PDCCH or there is PDCCH indicating downlink SPS release within subframe(s) , where  and  (defined in Table 10.1-1) is a set of M elements  depending on the subframe n and the UL-DL configuration (defined in Table 4.2-2 in [3]), the UE first selects a value out of {0, 1, 2, 3} which makes  and shall use , where  is configured by higher layers, , and  is the number of the first CCE used for transmission of the corresponding PDCCH in subframe and the corresponding m, where  is the smallest value in set such that UE detects a PDCCH in subframe .

- If there is only a PDSCH transmission where there is not a corresponding PDCCH detected within subframe(s) , where and  is defined in Table 10.1-1, the value of  is determined according to higher layer configuration and Table 9.2-2.

Table 10.1-1: Downlink association set index:  for TDD

| UL-DL Configuration | Subframe n |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 |
| 0 | - | - | 6 | - | 4 | - | - | 6 | - | 4 |
| 1 | - | - | 7, 6 | 4 | - | - | - | 7, 6 | 4 | - |
| 2 | - | - | 8, 7, 4, 6 | - | - | - | - | 8, 7, 4, 6 | - | - |
| 3 | - | - | 7, 6, 11 | 6, 5 | 5, 4 | - | - | - | - | - |
| 4 | - | - | 12, 8, 7, 11 | 6, 5, 4, 7 | - | - | - | - | - | - |
| 5 | - | - | 13, 12, 9, 8, 7, 5, 4, 11, 6 | - | - | - | - | - | - | - |
| 6 | - | - | 7 | 7 | 5 | - | - | 7 | 7 | - |

For TDD ACK/NACK multiplexing and sub-frame  with , where  is the number of elements in the set  defined in Table 10.1-1, denote  as the ACK/NACK resource derived from sub-frame  and HARQ-ACK(i) as the ACK/NACK/DTX response from sub-frame , where (defined in Table 10.1-1) and .

- For a PDSCH transmission or a PDCCH indicating downlink SPS release in sub-frame  where , the ACK/NACK resource , where is selected from {0, 1, 2, 3} such that , ,  is the number of the first CCE used for transmission of the corresponding PDCCH in subframe , and  is configured by higher layers.

- For a PDSCH transmission where there is not a corresponding PDCCH detected in subframe , the value of  is determined according to higher layer configuration and Table 9.2-2.

The UE shall transmit  on an ACK/NACK resource in sub-frame  using PUCCH format 1b according to section 5.4.1 in [3]. The value of  and the ACK/NACK resource  are generated by channel selection according to Table 10.1-2, 10.1-3, and 10.1-4 for M = 2, 3, and 4, respectively. In case  are mapped to “N/A,” then the UE shall not transmit ACK/NACK response in sub-frame.

Table 10.1-2: Transmission of ACK/NACK multiplexing for M = 2

| HARQ-ACK(0), HARQ-ACK(1) |  |  |
| --- | --- | --- |
| ACK, ACK |  | 1, 1 |
| ACK, NACK/DTX |  | 0, 1 |
| NACK/DTX, ACK |  | 0, 0 |
| NACK/DTX, NACK |  | 1, 0 |
| NACK, DTX |  | 1, 0 |
| DTX, DTX | N/A | N/A |

Table 10.1-3: Transmission of ACK/NACK multiplexing for M = 3

| HARQ-ACK(0), HARQ-ACK(1), HARQ-ACK(2) |  |  |
| --- | --- | --- |
| ACK, ACK, ACK |  | 1, 1 |
| ACK, ACK, NACK/DTX |  | 1, 1 |
| ACK, NACK/DTX, ACK |  | 1, 1 |
| ACK, NACK/DTX, NACK/DTX |  | 0, 1 |
| NACK/DTX, ACK, ACK |  | 1, 0 |
| NACK/DTX, ACK, NACK/DTX |  | 0, 0 |
| NACK/DTX, NACK/DTX, ACK |  | 0, 0 |
| DTX, DTX, NACK |  | 0, 1 |
| DTX, NACK, NACK/DTX |  | 1, 0 |
| NACK, NACK/DTX, NACK/DTX |  | 1, 0 |
| DTX, DTX, DTX | N/A | N/A |

Table 10.1-4: Transmission of ACK/NACK multiplexing for M = 4

| HARQ-ACK(0), HARQ-ACK(1), HARQ-ACK(2), HARQ-ACK(3) |  |  |
| --- | --- | --- |
| ACK, ACK, ACK, ACK |  | 1, 1 |
| ACK, ACK, ACK, NACK/DTX |  | 1, 0 |
| NACK/DTX,NACK/DTX,NACK,DTX |  | 1, 1 |
| ACK, ACK, NACK/DTX, ACK |  | 1, 0 |
| NACK, DTX, DTX, DTX |  | 1, 0 |
| ACK, ACK, NACK/DTX, NACK/DTX |  | 1, 0 |
| ACK, NACK/DTX, ACK, ACK |  | 0, 1 |
| NACK/DTX, NACK/DTX, NACK/DTX, NACK |  | 1, 1 |
| ACK, NACK/DTX, ACK, NACK/DTX |  | 0, 1 |
| ACK, NACK/DTX, NACK/DTX, ACK |  | 0, 1 |
| ACK, NACK/DTX, NACK/DTX, NACK/DTX |  | 1, 1 |
| NACK/DTX, ACK, ACK, ACK |  | 0, 1 |
| NACK/DTX, NACK, DTX, DTX |  | 0, 0 |
| NACK/DTX, ACK, ACK, NACK/DTX |  | 1, 0 |
| NACK/DTX, ACK, NACK/DTX, ACK |  | 1, 0 |
| NACK/DTX, ACK, NACK/DTX, NACK/DTX |  | 0, 1 |
| NACK/DTX, NACK/DTX, ACK, ACK |  | 0, 1 |
| NACK/DTX, NACK/DTX, ACK, NACK/DTX |  | 0, 0 |
| NACK/DTX, NACK/DTX, NACK/DTX, ACK |  | 0, 0 |
| DTX, DTX, DTX, DTX | N/A | N/A |

ACK/NACK repetition is enabled or disabled by a UE specific parameter ackNackRepetition configured by higher layers. Once enabled, the UE shall repeat any ACK/NACK transmission with a repetition factor , where  is provided by higher layers and includs the initial ACK/NACK transmission, until ACK/NACK repetition is disabled by higher layers. For a PDSCH transmission without a corresponding PDCCH detected, the UE shall transmit the corresponding ACK/NACK response  times using PUCCH resource configured by higher layers. For a PDSCH transmission with a corresponding PDCCH detected, or for a PDCCH indicating downlink SPS release, the UE shall first transmit the corresponding ACK/NACK response once using PUCCH resource derived from the corresponding PDCCH CCE index (as described in Section 10.1), and repeat the transmission of the corresponding ACK/NACK response   times always using PUCCH resource , where is configured by higher layers.

For TDD, ACK/NACK repetition is only applicable for ACK/NACK bundling and is not applicable for ACK/NACK multiplexing.

理解：TDD制式下，ANRep只能在ACK\NACK的bundling下进行，复用模式不能使用ANRep。

1.  UE在上行子帧的PUCCH上只会发送ACK/NACK（对应子帧n - 4上检测到的PDSCH传输）；

2.  UE不会在上行子帧上发送任何其它信号。

3. 对应在下行子帧上检测到PDSCH传输，UE不会发送任何重复的ACK/NACK，即不做ACK/NACK repetition处理，但第一个ACK/NACK还是要发送的。也就是说，此时对应下行子帧的PDSCH传输，只会发送一个ACK/NACK。（注：用于发送重复ACK/NACK的PUCCH 1资源只有一个或一对，该资源不能同时给2个下行子帧发送的PDSCH反馈ACK/NACK）

The scheduling request (SR) shall be transmitted on the PUCCH resource as defined in [3], where  is UE specific and configured by higher layers. The SR configuration for SR transmission periodicity and SR subframe offset  is defined in Table 10.1-5 by the parameter sr-ConfigIndex  given by higher layers.

SR transmission instances are the uplink subframes satisfying .

Table 10.1-5: UE-specific SR periodicity and subframe offset configuration

| SR configuration Index | SR periodicity (ms) | SR subframe offset |
| --- | --- | --- |
| 0 – 4 | 5 |  |
| 5 – 14 | 10 |  |
| 15 – 34 | 20 |  |
| 35 – 74 | 40 |  |
| 75 – 154 | 80 |  |
| 155 – 156 | 2 |  |
| 157 | 1 |  |

10.2	Uplink ACK/NACK timing

For FDD, the UE shall upon detection of a PDSCH transmission in subframe n-4 intended for the UE and for which an ACK/NACK shall be provided, transmit the ACK/NACK response in subframe n. If ACK/NACK repetition is enabled, upon detection of a PDSCH transmission in subframe n-4 intended for the UE and for which ACK/NACK response shall be provided, and if the UE is not repeating the transmission of any ACK/NACK in subframe  corresponding to a PDSCH transmission in subframes , … , , the UE:

- shall transmit only the ACK/NACK response (corresponding to the detected PDSCH transmission in subframe ) on PUCCH in subframes , , …, ;

- shall not transmit any other signal in subframes , , …, ; and

- shall not transmit any ACK/NACK response repetitions corresponding to any detected PDSCH transmission in subframes , …, .

For TDD, the UE shall upon detection of a PDSCH transmission within subframe(s) , where  and  is defined in Table 10.1-1 intended for the UE and for which ACK/NACK response shall be provided, transmit the ACK/NACK response in UL subframe n. If ACK/NACK repetition is enabled, upon detection of a PDSCH transmission within subframe(s) , where  and  is defined in Table 10.1-1 intended for the UE and for which ACK/NACK response shall be provided, and if the UE is not repeating the transmission of any ACK/NACK in subframe  corresponding to a PDSCH transmission in a DL subframe earlier than subframe , the UE:

- shall transmit only the ACK/NACK response (corresponding to the detected PDSCH transmission in subframe) on PUCCH in UL subframe and the next UL subframes denoted as , …,;

- shall not transmit any other signal in UL subframe , , …,; and

- shall not transmit any ACK/NACK response repetitions corresponding to any detected PDSCH transmission in subframes  , where , is the set defined in Table 10.1-1 corresponding to UL subframe , and .

For TDD, ACK/NACK bundling, if the UE detects that at least one downlink assignment has been missed as described in Section 7.3, the UE shall not transmit ACK/NACK in case the UE is not transmitting on PUSCH.

The uplink timing for the ACK corresponding to a detected PDCCH indicating downlink SPS release shall be the same as the uplink timing for the ACK/NACK corresponding to a detected PDSCH, as defined above.

11 	Physical multicast channel related procedures

11.1	UE procedure for receiving the physical multicast channel

The UE shall decode the PMCH when configured by higher layers. The UE may assume that an eNB transmission on the PMCH is performed according to Section 6.5 of [3].

Thefor the PMCH is configured by higher layers. The UE shall usefor the PMCH and Table 7.1.7.1-1 to determine the modulation order () and TBS index () used in the PMCH. The UE shall then follow the procedure in Section 7.1.7.2.1 to determine the transport block size, assuming is equal to.

11.2	UE procedure for receiving MCCH change notification

If a UE is configured by higher layers to decode PDCCHs with the CRC scrambled by the M-RNTI, the UE shall decode the PDCCH according to the combination defined in table 11.2-1.

# Table 11.2-1: PDCCH configured by M-RNTI

| DCI format | Search Space |
| --- | --- |
| DCI format 1C | Common |

The 8-bit information for MCCH change notification [11], as signalled on the PDCCH, shall be delivered to higher layers.

Annex A (informative):
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | TSG # | TSG Doc. | CR | Rev | Subject/Comment | Old | New |
| 2006-09 |  |  |  |  | Draft version created |  | 0.0.0 |
| 2006-10 |  |  |  |  | Endorsed by RAN1 | 0.0.0 | 0.1.0 |
| 2007-01 |  |  |  |  | Inclusion of decisions from RAN1#46bis and RAN1#47 | 0.1.0 | 0.1.1 |
| 2007-01 |  |  |  |  | Endorsed by RAN1 | 0.1.1 | 0.2.0 |
| 2007-02 |  |  |  |  | Inclusion of decisions from RAN1#47bis | 0.2.0 | 0.2.1 |
| 2007-02 |  |  |  |  | Endorsed by RAN1 | 0.2.1 | 0.3.0 |
| 2007-02 |  |  |  |  | Editor’s version including decisions from RAN1#48 & RAN1#47bis | 0.3.0 | 0.3.1 |
| 2007-03 |  |  |  |  | Updated Editor’s version | 0.3.1 | 0.3.2 |
| 2007-03 | RAN#35 | RP-070171 |  |  | For information at RAN#35 | 0.3.2 | 1.0.0 |
| 2007-03 |  |  |  |  | Random access text modified to better reflect RAN1 scope | 1.0.0 | 1.0.1 |
| 2007-03 |  |  |  |  | Updated Editor’s version | 1.0.1 | 1.0.2 |
| 2007-03 |  |  |  |  | Endorsed by RAN1 | 1.0.2 | 1.1.0 |
| 2007-05 |  |  |  |  | Updated Editor’s version | 1.1.0 | 1.1.1 |
| 2007-05 |  |  |  |  | Updated Editor’s version | 1.1.1 | 1.1.2 |
| 2007-05 |  |  |  |  | Endorsed by RAN1 | 1.1.2 | 1.2.0 |
| 2007-08 |  |  |  |  | Updated Editor’s version | 1.2.0 | 1.2.1 |
| 2007-08 |  |  |  |  | Updated Editor’s version – uplink power control from RAN1#49bis | 1.2.1 | 1.2.2 |
| 2007-08 |  |  |  |  | Endorsed by RAN1 | 1.2.2 | 1.3.0 |
| 2007-09 |  |  |  |  | Updated Editor’s version reflecting RAN#50 decisions | 1.3.0 | 1.3.1 |
| 2007-09 |  |  |  |  | Updated Editor’s version reflecting comments | 1.3.1 | 1.3.2 |
| 2007-09 |  |  |  |  | Updated Editor’s version reflecting further comments | 1.3.2 | 1.3.3 |
| 2007-09 |  |  |  |  | Updated Editor’s version reflecting further comments | 1.3.3 | 1.3.4 |
| 2007-09 |  |  |  |  | Updated Edtior’s version reflecting further comments | 1.3.4 | 1.3.5 |
| 2007-09 | RAN#37 | RP-070731 |  |  | Endorsed by RAN1 | 1.3.5 | 2.0.0 |
| 2007-09 | RAN#37 | RP-070737 |  |  | For approval at RAN#37 | 2.0.0 | 2.1.0 |
| 12/09/07 | RAN_37 | RP-070737 | - | - | Approved version | 2.1.0 | 8.0.0 |
| 28/11/07 | RAN_38 | RP-070949 | 0001 | 2 | Update of 36.213 | 8.0.0 | 8.1.0 |
| 05/03/08 | RAN_39 | RP-080145 | 0002 | - | Update of TS36.213 according to changes listed in cover sheet | 8.1.0 | 8.2.0 |
| 28/05/08 | RAN_40 | RP-080434 | 0003 | 1 | PUCCH timing and other formatting and typo corrections | 8.2.0 | 8.3.0 |
| 28/05/08 | RAN_40 | RP-080434 | 0006 | 1 | PUCCH power control for non-unicast information | 8.2.0 | 8.3.0 |
| 28/05/08 | RAN_40 | RP-080434 | 0008 | - | UE ACK/NACK Procedure | 8.2.0 | 8.3.0 |
| 28/05/08 | RAN_40 | RP-080434 | 0009 | - | UL ACK/NACK timing for TDD | 8.2.0 | 8.3.0 |
| 28/05/08 | RAN_40 | RP-080434 | 0010 | - | Specification of UL control channel assignment | 8.2.0 | 8.3.0 |
| 28/05/08 | RAN_40 | RP-080434 | 0011 | - | Precoding Matrix for 2Tx Open-loop SM | 8.2.0 | 8.3.0 |
| 28/05/08 | RAN_40 | RP-080434 | 0012 | - | Clarifications on UE selected CQI reports | 8.2.0 | 8.3.0 |
| 28/05/08 | RAN_40 | RP-080434 | 0013 | 1 | UL HARQ Operation and Timing | 8.2.0 | 8.3.0 |
| 28/05/08 | RAN_40 | RP-080434 | 0014 | - | SRS power control | 8.2.0 | 8.3.0 |
| 28/05/08 | RAN_40 | RP-080434 | 0015 | 1 | Correction of UE PUSCH frequency hopping procedure | 8.2.0 | 8.3.0 |
| 28/05/08 | RAN_40 | RP-080434 | 0017 | 4 | Blind PDCCH decoding | 8.2.0 | 8.3.0 |
| 28/05/08 | RAN_40 | RP-080434 | 0019 | 1 | Tx Mode vs DCI format is clarified | 8.2.0 | 8.3.0 |
| 28/05/08 | RAN_40 | RP-080434 | 0020 | - | Resource allocation for distributed VRB | 8.2.0 | 8.3.0 |
| 28/05/08 | RAN_40 | RP-080434 | 0021 | 2 | Power Headroom | 8.2.0 | 8.3.0 |
| 28/05/08 | RAN_40 | RP-080434 | 0022 | - | Clarification for RI reporting in PUCCH and PUSCH reporting modes | 8.2.0 | 8.3.0 |
| 28/05/08 | RAN_40 | RP-080434 | 0025 | - | Correction of the description of PUSCH power control for TDD | 8.2.0 | 8.3.0 |
| 28/05/08 | RAN_40 | RP-080434 | 0026 | - | UL ACK/NACK procedure for TDD | 8.2.0 | 8.3.0 |
| 28/05/08 | RAN_40 | RP-080434 | 0027 | - | Indication of radio problem detection | 8.2.0 | 8.3.0 |
| 28/05/08 | RAN_40 | RP-080434 | 0028 | - | Definition of Relative Narrowband TX Power Indicator | 8.2.0 | 8.3.0 |
| 28/05/08 | RAN_40 | RP-080434 | 0029 | - | Calculation of ΔTF(i) for UL-PC | 8.2.0 | 8.3.0 |
| 28/05/08 | RAN_40 | RP-080434 | 0030 | - | CQI reference and set S definition, CQI mode removal, and Miscellanious | 8.2.0 | 8.3.0 |
| 28/05/08 | RAN_40 | RP-080434 | 0031 | - | Modulation order and TBS determination for PDSCH and PUSCH | 8.2.0 | 8.3.0 |
| 28/05/08 | RAN_40 | RP-080434 | 0032 | - | On Sounding RS | 8.2.0 | 8.3.0 |
| 28/05/08 | RAN_40 | RP-080426 | 0033 | - | Multiplexing of rank and CQI/PMI reports on PUCCH | 8.2.0 | 8.3.0 |
| 28/05/08 | RAN_40 | RP-080466 | 0034 | - | Timing advance command responding time | 8.2.0 | 8.3.0 |
| 09/09/08 | RAN_41 | RP-080670 | 37 | 2 | SRS hopping pattern for closed loop antenna selection | 8.3.0 | 8.4.0 |
| 09/09/08 | RAN_41 | RP-080670 | 39 | 2 | Clarification on uplink power control | 8.3.0 | 8.4.0 |
| 09/09/08 | RAN_41 | RP-080670 | 41 | - | Clarification on DCI formats using resource allocation type 2 | 8.3.0 | 8.4.0 |
| 09/09/08 | RAN_41 | RP-080670 | 43 | 2 | Clarification on tree structure of CCE aggregations | 8.3.0 | 8.4.0 |
| 09/09/08 | RAN_41 | RP-080670 | 46 | 2 | Correction of the description of PUCCH power control for TDD | 8.3.0 | 8.4.0 |
| 09/09/08 | RAN_41 | RP-080670 | 47 | 1 | Removal of CR0009 | 8.3.0 | 8.4.0 |
| 09/09/08 | RAN_41 | RP-080670 | 48 | 1 | Correction of mapping of cyclic shift value to PHICH modifier | 8.3.0 | 8.4.0 |
| 09/09/08 | RAN_41 | RP-080670 | 49 | - | TBS disabling for DCI formats 2 and 2A | 8.3.0 | 8.4.0 |
| 09/09/08 | RAN_41 | RP-080670 | 50 | - | Correction of maximum TBS sizes | 8.3.0 | 8.4.0 |
| 09/09/08 | RAN_41 | RP-080670 | 51 | - | Completion of the table specifying the number of bits for the periodic feedback | 8.3.0 | 8.4.0 |
| 09/09/08 | RAN_41 | RP-080670 | 54 | - | Clarification of RNTI for PUSCH/PUCCH power control with DCI formats 3/3A | 8.3.0 | 8.4.0 |
| 09/09/08 | RAN_41 | RP-080670 | 55 | 1 | Clarification on mapping of Differential CQI fields | 8.3.0 | 8.4.0 |
| 09/09/08 | RAN_41 | RP-080670 | 59 | 1 | PUSCH Power Control | 8.3.0 | 8.4.0 |
| 09/09/08 | RAN_41 | RP-080670 | 60 | - | RB restriction and modulation order for CQI-only transmission on PUSCH | 8.3.0 | 8.4.0 |
| 09/09/08 | RAN_41 | RP-080670 | 61 | - | Modulation order determination for uplink retransmissions | 8.3.0 | 8.4.0 |
| 09/09/08 | RAN_41 | RP-080670 | 62 | 2 | Introducing missing L1 parameters into 36.213 | 8.3.0 | 8.4.0 |
| 09/09/08 | RAN_41 | RP-080670 | 63 | 2 | Correcting the range and representation of delta_TF_PUCCH | 8.3.0 | 8.4.0 |
| 09/09/08 | RAN_41 | RP-080670 | 64 | 1 | Adjusting TBS sizes to for VoIP | 8.3.0 | 8.4.0 |
| 09/09/08 | RAN_41 | RP-080670 | 67 | - | Correction to the downlink resource allocation | 8.3.0 | 8.4.0 |
| 09/09/08 | RAN_41 | RP-080670 | 68 | - | Removal of special handling for PUSCH mapping in PUCCH region | 8.3.0 | 8.4.0 |
| 09/09/08 | RAN_41 | RP-080670 | 69 | - | Correction to the formulas for uplink power control | 8.3.0 | 8.4.0 |
| 09/09/08 | RAN_41 | RP-080670 | 70 | 1 | Definition of Bit Mapping for DCI Signalling | 8.3.0 | 8.4.0 |
| 09/09/08 | RAN_41 | RP-080670 | 71 | - | Clarification on PUSCH TPC commands | 8.3.0 | 8.4.0 |
| 09/09/08 | RAN_41 | RP-080670 | 72 | 1 | Reference for CQI/PMI Reporting Offset | 8.3.0 | 8.4.0 |
| 09/09/08 | RAN_41 | RP-080670 | 74 | - | Correction to the downlink/uplink timing | 8.3.0 | 8.4.0 |
| 09/09/08 | RAN_41 | RP-080670 | 75 | - | Correction to the time alignment command | 8.3.0 | 8.4.0 |
| 09/09/08 | RAN_41 | RP-080670 | 77 | 1 | Correction of offset signalling of UL Control information MCS | 8.3.0 | 8.4.0 |
| 09/09/08 | RAN_41 | RP-080670 | 78 | 2 | DCI format1C | 8.3.0 | 8.4.0 |
| 09/09/08 | RAN_41 | RP-080670 | 80 | - | Correction to Precoder Cycling for Open-loop Spatial Multiplexing | 8.3.0 | 8.4.0 |
| 09/09/08 | RAN_41 | RP-080670 | 81 | 1 | Clarifying Periodic CQI Reporting using PUCCH | 8.3.0 | 8.4.0 |
| 09/09/08 | RAN_41 | RP-080670 | 84 | 1 | CQI reference measurement period | 8.3.0 | 8.4.0 |
| 09/09/08 | RAN_41 | RP-080670 | 86 | - | Correction on downlink multi-user MIMO | 8.3.0 | 8.4.0 |
| 09/09/08 | RAN_41 | RP-080670 | 87 | - | PUCCH Reporting | 8.3.0 | 8.4.0 |
| 09/09/08 | RAN_41 | RP-080670 | 88 | 1 | Handling of Uplink Grant in Random Access Response | 8.3.0 | 8.4.0 |
| 09/09/08 | RAN_41 | RP-080670 | 89 | - | Correction to UL Hopping operation | 8.3.0 | 8.4.0 |
| 09/09/08 | RAN_41 | RP-080670 | 90 | - | DRS EPRE | 8.3.0 | 8.4.0 |
| 09/09/08 | RAN_41 | RP-080670 | 92 | - | Uplink ACK/NACK mapping for TDD | 8.3.0 | 8.4.0 |
| 09/09/08 | RAN_41 | RP-080670 | 93 | - | UL SRI Parameters Configuration | 8.3.0 | 8.4.0 |
| 09/09/08 | RAN_41 | RP-080670 | 94 | - | Miscellaneous updates for 36.213 | 8.3.0 | 8.4.0 |
| 09/09/08 | RAN_41 | RP-080670 | 95 | - | Clarifying Requirement for Max PDSCH Coding Rate | 8.3.0 | 8.4.0 |
| 09/09/08 | RAN_41 | RP-080670 | 96 | - | UE Specific SRS Configuration | 8.3.0 | 8.4.0 |
| 09/09/08 | RAN_41 | RP-080670 | 97 | - | DCI Format 1A changes needed for scheduling Broadcast Control | 8.3.0 | 8.4.0 |
| 09/09/08 | RAN_41 | RP-080670 | 98 | - | Processing of TPC bits in the random access response | 8.3.0 | 8.4.0 |
| 09/09/08 | RAN_41 | RP-080670 | 100 | 1 | Support of multi-bit ACK/NAK transmission in TDD | 8.3.0 | 8.4.0 |
| 03/12/08 | RAN_42 | RP-081075 | 82 | 3 | Corrections to RI for CQI reporting | 8.4.0 | 8.5.0 |
| 03/12/08 | RAN_42 | RP-081075 | 83 | 2 | Moving description of large delay CDD to 36.211 | 8.4.0 | 8.5.0 |
| 03/12/08 | RAN_42 | RP-081075 | 102 | 3 | Reception of DCI formats | 8.4.0 | 8.5.0 |
| 03/12/08 | RAN_42 | RP-081075 | 105 | 8 | Alignment of RAN1/RAN2 specification | 8.4.0 | 8.5.0 |
| 03/12/08 | RAN_42 | RP-081075 | 107 | 1 | General correction of reset of power control and random access response message | 8.4.0 | 8.5.0 |
| 03/12/08 | RAN_42 | RP-081075 | 108 | 2 | Final details on codebook subset restrictions | 8.4.0 | 8.5.0 |
| 03/12/08 | RAN_42 | RP-081075 | 109 | - | Correction on the definition of Pmax | 8.4.0 | 8.5.0 |
| 03/12/08 | RAN_42 | RP-081075 | 112 | 2 | CQI/PMI reference measurement periods | 8.4.0 | 8.5.0 |
| 03/12/08 | RAN_42 | RP-081075 | 113 | - | Correction of introduction of shortened SR | 8.4.0 | 8.5.0 |
| 03/12/08 | RAN_42 | RP-081075 | 114 | - | RAN1/2 specification alignment on HARQ operation | 8.4.0 | 8.5.0 |
| 03/12/08 | RAN_42 | RP-081075 | 115 | - | Introducing other missing L1 parameters in 36.213 | 8.4.0 | 8.5.0 |
| 03/12/08 | RAN_42 | RP-081075 | 116 | - | PDCCH blind decoding | 8.4.0 | 8.5.0 |
| 03/12/08 | RAN_42 | RP-081075 | 117 | - | PDCCH search space | 8.4.0 | 8.5.0 |
| 03/12/08 | RAN_42 | RP-081075 | 119 | - | Delta_TF for PUSCH | 8.4.0 | 8.5.0 |
| 03/12/08 | RAN_42 | RP-081075 | 120 | - | Delta_preamble_msg3 parameter values and TPC command in RA response | 8.4.0 | 8.5.0 |
| 03/12/08 | RAN_42 | RP-081075 | 122 | 1 | Correction of offset signaling of uplink control information MCS | 8.4.0 | 8.5.0 |
| 03/12/08 | RAN_42 | RP-081075 | 124 | - | Miscellaneous Corrections | 8.4.0 | 8.5.0 |
| 03/12/08 | RAN_42 | RP-081075 | 125 | - | Clarification of the uplink index in TDD mode | 8.4.0 | 8.5.0 |
| 03/12/08 | RAN_42 | RP-081075 | 126 | - | Clarification of the uplink transmission configurations | 8.4.0 | 8.5.0 |
| 03/12/08 | RAN_42 | RP-081075 | 127 | 2 | Correction to the PHICH index assignment | 8.4.0 | 8.5.0 |
| 03/12/08 | RAN_42 | RP-081075 | 128 | - | Clarification of type-2 PDSCH resource allocation for format 1C | 8.4.0 | 8.5.0 |
| 03/12/08 | RAN_42 | RP-081075 | 129 | - | Clarification of uplink grant in random access response | 8.4.0 | 8.5.0 |
| 03/12/08 | RAN_42 | RP-081075 | 130 | - | UE sounding procedure | 8.4.0 | 8.5.0 |
| 03/12/08 | RAN_42 | RP-081075 | 134 | - | Change for determining DCI format 1A TBS table column indicator for broadcast control | 8.4.0 | 8.5.0 |
| 03/12/08 | RAN_42 | RP-081075 | 135 | - | Clarifying UL VRB Allocation | 8.4.0 | 8.5.0 |
| 03/12/08 | RAN_42 | RP-081075 | 136 | 1 | Correction for Aperiodic CQI | 8.4.0 | 8.5.0 |
| 03/12/08 | RAN_42 | RP-081075 | 137 | 1 | Correction for Aperiodic CQI Reporting | 8.4.0 | 8.5.0 |
| 03/12/08 | RAN_42 | RP-081075 | 138 | 1 | Correction to PUCCH CQI reporting mode for N^DL_RB <= 7 | 8.4.0 | 8.5.0 |
| 03/12/08 | RAN_42 | RP-081075 | 140 | 1 | On sounding procedure in TDD | 8.4.0 | 8.5.0 |
| 03/12/08 | RAN_42 | RP-081075 | 141 | 1 | Alignment of RAN1/RAN3 specification | 8.4.0 | 8.5.0 |
| 03/12/08 | RAN_42 | RP-081075 | 143 | 1 | TTI bundling | 8.4.0 | 8.5.0 |
| 03/12/08 | RAN_42 | RP-081075 | 144 | 1 | ACK/NACK transmission on PUSCH for LTE TDD | 8.4.0 | 8.5.0 |
| 03/12/08 | RAN_42 | RP-081075 | 145 | 1 | Timing relationship between PHICH and its associated PUSCH | 8.4.0 | 8.5.0 |
| 03/12/08 | RAN_42 | RP-081075 | 147 | 1 | Definition of parameter for downlink reference signal transmit power | 8.4.0 | 8.5.0 |
| 03/12/08 | RAN_42 | RP-081075 | 148 | 1 | Radio link monitoring | 8.4.0 | 8.5.0 |
| 03/12/08 | RAN_42 | RP-081075 | 149 | 1 | Correction in 36.213 related to TDD downlink HARQ processes | 8.4.0 | 8.5.0 |
| 03/12/08 | RAN_42 | RP-081075 | 151 | - | Nominal PDSCH-to-RS EPRE Offset for CQI Reporting | 8.4.0 | 8.5.0 |
| 03/12/08 | RAN_42 | RP-081075 | 152 | 1 | Support of UL ACK/NAK repetition in Rel-8 | 8.4.0 | 8.5.0 |
| 03/12/08 | RAN_42 | RP-081075 | 155 | - | Clarification of misconfiguration of aperiodic CQI and SR | 8.4.0 | 8.5.0 |
| 03/12/08 | RAN_42 | RP-081075 | 156 | 1 | Correction of control information multiplexing in subframe bundling mode | 8.4.0 | 8.5.0 |
| 03/12/08 | RAN_42 | RP-081075 | 157 | - | Correction to the PHICH index assignment | 8.4.0 | 8.5.0 |
| 03/12/08 | RAN_42 | RP-081075 | 158 | 1 | UE transmit antenna selection | 8.4.0 | 8.5.0 |
| 03/12/08 | RAN_42 | RP-081075 | 159 | - | Clarification of spatial different CQI for CQI reporting Mode 2-1 | 8.4.0 | 8.5.0 |
| 03/12/08 | RAN_42 | RP-081075 | 160 | 1 | Corrections for TDD ACK/NACK bundling and multiplexing | 8.4.0 | 8.5.0 |
| 03/12/08 | RAN_42 | RP-081075 | 161 | - | Correction to RI for Open-Loop Spatial Multiplexing | 8.4.0 | 8.5.0 |
| 03/12/08 | RAN_42 | RP-081075 | 162 | - | Correction of differential CQI | 8.4.0 | 8.5.0 |
| 03/12/08 | RAN_42 | RP-081075 | 163 | - | Inconsistency between PMI definition and codebook index | 8.4.0 | 8.5.0 |
| 03/12/08 | RAN_42 | RP-081075 | 164 | - | PDCCH validation for semi-persistent scheduling | 8.4.0 | 8.5.0 |
| 03/12/08 | RAN_42 | RP-081075 | 165 | 1 | Correction to the UE behavior of PUCCH CQI piggybacked on PUSCH | 8.4.0 | 8.5.0 |
| 03/12/08 | RAN_42 | RP-081075 | 166 | - | Correction on SRS procedure when shortened PUCCH format is used | 8.4.0 | 8.5.0 |
| 03/12/08 | RAN_42 | RP-081075 | 167 | 1 | Transmission overlapping of physical channels/signals with PDSCH for transmission mode 7 | 8.4.0 | 8.5.0 |
| 03/12/08 | RAN_42 | RP-081075 | 169 | - | Clarification of SRS and SR transmission | 8.4.0 | 8.5.0 |
| 03/12/08 | RAN_42 | RP-081075 | 171 | - | Clarification on UE behavior when skipping decoding | 8.4.0 | 8.5.0 |
| 03/12/08 | RAN_42 | RP-081075 | 172 | 1 | PUSCH Hopping operation corrections | 8.4.0 | 8.5.0 |
| 03/12/08 | RAN_42 | RP-081075 | 173 | - | Clarification on message 3 transmission timing | 8.4.0 | 8.5.0 |
| 03/12/08 | RAN_42 | RP-081075 | 174 | - | MCS handling for DwPTS | 8.4.0 | 8.5.0 |
| 03/12/08 | RAN_42 | RP-081075 | 175 | - | Clarification of UE-specific time domain position for SR transmission | 8.4.0 | 8.5.0 |
| 03/12/08 | RAN_42 | RP-081075 | 176 | 1 | Physical layer parameters for CQI reporting | 8.4.0 | 8.5.0 |
| 03/12/08 | RAN_42 | RP-081075 | 177 | - | A-periodic CQI clarification for TDD UL/DL configuration 0 | 8.4.0 | 8.5.0 |
| 03/12/08 | RAN_42 | RP-081075 | 179 | 1 | Correction to the definitions of rho_A and rho_B (downlink power allocation) | 8.4.0 | 8.5.0 |
| 03/12/08 | RAN_42 | RP-081075 | 180 | - | Clarification of uplink A/N resource indication | 8.4.0 | 8.5.0 |
| 03/12/08 | RAN_42 | RP-081075 | 181 | - | PDCCH format 0 for message 3 adaptive retransmission and transmission of control information in message 3 during contention based random access procedure | 8.4.0 | 8.5.0 |
| 03/12/08 | RAN_42 | RP-081075 | 182 | - | To Fix the Discrepancy of Uplink Power Control and Channel Coding of Control Information in PUSCH | 8.4.0 | 8.5.0 |
| 03/12/08 | RAN_42 | RP-081122 | 183 | 1 | CQI reporting for antenna port 5 | 8.4.0 | 8.5.0 |
| 03/12/08 | RAN_42 | RP-081110 | 168 | 1 | Clarification on path loss definition | 8.4.0 | 8.5.0 |
| 04/03/09 | RAN_43 | RP-090236 | 184 | 1 | Corrections to Transmitted Rank Indication | 8.5.0 | 8.6.0 |
| 04/03/09 | RAN_43 | RP-090236 | 185 | 4 | Corrections to transmission modes | 8.5.0 | 8.6.0 |
| 04/03/09 | RAN_43 | RP-090236 | 186 | 2 | Delta_TF configuration for control only PUSCH | 8.5.0 | 8.6.0 |
| 04/03/09 | RAN_43 | RP-090236 | 187 | 1 | Correction to concurrent SRS and ACK/NACK transmission | 8.5.0 | 8.6.0 |
| 04/03/09 | RAN_43 | RP-090236 | 191 | 1 | PDCCH release for semi-persistent scheduling | 8.5.0 | 8.6.0 |
| 04/03/09 | RAN_43 | RP-090236 | 192 | 1 | Correction on ACKNACK transmission on PUSCH for LTE TDD | 8.5.0 | 8.6.0 |
| 04/03/09 | RAN_43 | RP-090236 | 193 | - | Correction to subband differential CQI value to offset level mapping for aperiodic CQI reporting | 8.5.0 | 8.6.0 |
| 04/03/09 | RAN_43 | RP-090236 | 194 | - | Correction for DRS Collision handling | 8.5.0 | 8.6.0 |
| 04/03/09 | RAN_43 | RP-090236 | 196 | 2 | Alignment of RAN1/RAN4 specification on UE maximum output power | 8.5.0 | 8.6.0 |
| 04/03/09 | RAN_43 | RP-090236 | 197 | - | Transmission scheme for transmission mode 7 with SPS C-RNTI | 8.5.0 | 8.6.0 |
| 04/03/09 | RAN_43 | RP-090236 | 198 | - | Clarifying bandwidth parts for periodic CQI reporting and CQI refererence period | 8.5.0 | 8.6.0 |
| 04/03/09 | RAN_43 | RP-090236 | 199 | 2 | Correction to the ACK/NACK bundling in case of transmission mode 3 and 4 | 8.5.0 | 8.6.0 |
| 04/03/09 | RAN_43 | RP-090236 | 200 | - | ACK/NAK repetition for TDD ACK/NAK multiplexing | 8.5.0 | 8.6.0 |
| 04/03/09 | RAN_43 | RP-090236 | 201 | - | Clarifying UL ACK/NAK transmission in TDD | 8.5.0 | 8.6.0 |
| 04/03/09 | RAN_43 | RP-090236 | 202 | - | Corrections to UE Transmit Antenna Selection | 8.5.0 | 8.6.0 |
| 04/03/09 | RAN_43 | RP-090236 | 203 | - | Correction to UE PUSCH hopping procedure | 8.5.0 | 8.6.0 |
| 04/03/09 | RAN_43 | RP-090236 | 204 | - | Correction to PHICH resource association in TTI bundling | 8.5.0 | 8.6.0 |
| 04/03/09 | RAN_43 | RP-090236 | 205 | - | Clarification of the length of resource assignment | 8.5.0 | 8.6.0 |
| 04/03/09 | RAN_43 | RP-090236 | 206 | - | Correction on ACK/NACK transmission for downlink SPS resource release | 8.5.0 | 8.6.0 |
| 04/03/09 | RAN_43 | RP-090236 | 207 | - | Introduction of additional values of wideband CQI/PMI periodicities | 8.5.0 | 8.6.0 |
| 04/03/09 | RAN_43 | RP-090236 | 208 | 2 | Correction to CQI/PMI/RI reporting field | 8.5.0 | 8.6.0 |
| 04/03/09 | RAN_43 | RP-090236 | 209 | 2 | Correction to rho_A definition for CQI calculation | 8.5.0 | 8.6.0 |
| 04/03/09 | RAN_43 | RP-090236 | 210 | - | Correction to erroneous cases in PUSCH linear block codes | 8.5.0 | 8.6.0 |
| 04/03/09 | RAN_43 | RP-090236 | 211 | 1 | Removing RL monitoring start and stop | 8.5.0 | 8.6.0 |
| 04/03/09 | RAN_43 | RP-090236 | 214 | 1 | Correction to type-1 and type-2 PUSCH hopping | 8.5.0 | 8.6.0 |
| 04/03/09 | RAN_43 | RP-090236 | 215 | - | Contradicting statements on determination of CQI subband size | 8.5.0 | 8.6.0 |
| 04/03/09 | RAN_43 | RP-090236 | 216 | - | Corrections to SRS | 8.5.0 | 8.6.0 |
| 04/03/09 | RAN_43 | RP-090236 | 219 | 2 | Miscellaneous corrections on TDD ACKNACK | 8.5.0 | 8.6.0 |
| 04/03/09 | RAN_43 | RP-090236 | 221 | 1 | CR for Redundancy Version mapping function for DCI 1C | 8.5.0 | 8.6.0 |
| 04/03/09 | RAN_43 | RP-090236 | 223 | - | Scrambling of PUSCH corresponding to Random Access Response Grant | 8.5.0 | 8.6.0 |
| 04/03/09 | RAN_43 | RP-090236 | 225 | - | Removal of SRS with message 3 | 8.5.0 | 8.6.0 |
| 04/03/09 | RAN_43 | RP-090236 | 226 | 3 | PRACH retransmission timing | 8.5.0 | 8.6.0 |
| 04/03/09 | RAN_43 | RP-090236 | 227 | - | Clarifying error handling of PDSCH and PUSCH assignments | 8.5.0 | 8.6.0 |
| 04/03/09 | RAN_43 | RP-090236 | 228 | - | Clarify PHICH index mapping | 8.5.0 | 8.6.0 |
| 04/03/09 | RAN_43 | RP-090236 | 229 | - | Correction of CQI timing | 8.5.0 | 8.6.0 |
| 04/03/09 | RAN_43 | RP-090236 | 230 | - | Alignment of CQI parameter names with RRC | 8.5.0 | 8.6.0 |
| 04/03/09 | RAN_43 | RP-090236 | 231 | 1 | Removal of ‘Off’ values for periodic reporting in L1 | 8.5.0 | 8.6.0 |
| 04/03/09 | RAN_43 | RP-090236 | 232 | - | Default value of RI | 8.5.0 | 8.6.0 |
| 04/03/09 | RAN_43 | RP-090236 | 233 | 1 | Clarification of uplink timing adjustments | 8.5.0 | 8.6.0 |
| 04/03/09 | RAN_43 | RP-090236 | 234 | - | Clarification on ACK/NAK repetition | 8.5.0 | 8.6.0 |
| 27/05/09 | RAN_44 | RP-090529 | 235 | 1 | Correction to the condition of resetting accumulated uplink power correction | 8.6.0 | 8.7.0 |
| 27/05/09 | RAN_44 | RP-090529 | 236 | - | Correction to the random access channel parameters received from higher layer | 8.6.0 | 8.7.0 |
| 27/05/09 | RAN_44 | RP-090529 | 237 | - | Correction on TDD ACKNACK | 8.6.0 | 8.7.0 |
| 27/05/09 | RAN_44 | RP-090529 | 238 | 1 | Correction on CQI reporting | 8.6.0 | 8.7.0 |
| 27/05/09 | RAN_44 | RP-090529 | 239 | - | Correction on the HARQ process number | 8.6.0 | 8.7.0 |
| 27/05/09 | RAN_44 | RP-090529 | 241 | 1 | CR correction of the description on TTI-bundling | 8.6.0 | 8.7.0 |
| 27/05/09 | RAN_44 | RP-090529 | 242 | 1 | Clarify latest and initial PDCCH for PDSCH and PUSCH transmisisons, and NDI for SPS activation | 8.6.0 | 8.7.0 |
| 27/05/09 | RAN_44 | RP-090529 | 243 | - | Clarify DRS EPRE | 8.6.0 | 8.7.0 |
| 27/05/09 | RAN_44 | RP-090529 | 244 | 1 | Clarification on TPC commands for SPS | 8.6.0 | 8.7.0 |
| 15/09/09 | RAN_45 | RP-090888 | 245 | 1 | Correction to PUSCH hopping and PHICH mapping procedures | 8.7.0 | 8.8.0 |
| 15/09/09 | RAN_45 | RP-090888 | 246 | - | Clarification on subband indexing in periodic CQI reporting | 8.7.0 | 8.8.0 |
| 15/09/09 | RAN_45 | RP-090888 | 247 | 2 | Correction to DVRB operation in TDD transmission mode 7 | 8.7.0 | 8.8.0 |
| 15/09/09 | RAN_45 | RP-090888 | 249 | - | Clarification of concurrent ACKNACK and periodic PMI/RI transmission on PUCCH for TDD | 8.7.0 | 8.8.0 |
| 15/09/09 | RAN_45 | RP-090888 | 250 | - | Clarify Inter-cell synchronization text | 8.7.0 | 8.8.0 |
| 01/12/09 | RAN_46 | RP-091172 | 248 | 1 | Introduction of LTE positioning | 8.8.0 | 9.0.0 |
| 01/12/09 | RAN_46 | RP-091172 | 254 | - | Clarification of PDSCH and PRS in combination for LTE positioning | 8.8.0 | 9.0.0 |
| 01/12/09 | RAN_46 | RP-091177 | 255 | 5 | Editorial corrections to 36.213 | 8.8.0 | 9.0.0 |
| 01/12/09 | RAN_46 | RP-091257 | 256 | 1 | Introduction of enhanced dual layer transmission | 8.8.0 | 9.0.0 |
| 01/12/09 | RAN_46 | RP-091177 | 257 | 1 | Add shorter SR periodicity | 8.8.0 | 9.0.0 |
| 01/12/09 | RAN_46 | RP-091256 | 258 | - | Introduction of LTE MBMS | 8.8.0 | 9.0.0 |
| 17/12/09 | RAN_46 | RP-091257 | 256 | 1 | Correction by MCC due to wrong implementation of CR0256r1 – Sentence is added to Single-antenna port scheme section 7.1.1 | 9.0.0 | 9.0.1 |
| 16/03/10 | RAN_47 | RP-100211 | 259 | 3 | UE behavior when collision of antenna port 7/8 with PBCH or SCH happened and when distributed VRB is used with antenna port 7 | 9.0.1 | 9.1.0 |
| 16/03/10 | RAN_47 | RP-100210 | 260 | 1 | MCCH change notification using DCI format 1C | 9.0.1 | 9.1.0 |
| 16/03/10 | RAN_47 | RP-100211 | 263 | - | Correction on PDSCH EPRE and UE-specific RS EPRE for Rel-9 enhanced DL transmissions | 9.0.1 | 9.1.0 |
| 01/06/10 | RAN_48 | RP-100589 | 265 | - | Clarification for TDD when multiplexing ACK/NACK with SR of ACK/NACK with CQI/PMI or RI | 9.1.0 | 9.2.0 |
| 01/06/10 | RAN_48 | RP-100590 | 268 | 1 | Clarification of PRS EPRE | 9.1.0 | 9.2.0 |